3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205720

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluations and conclusions of KI#1

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: It is proposed to have evaluations and conclusions of KI#1.

1. Introduction/Discussion

There are sixteen solutions for KI#1. The details are shown in Table 1-1

Table 1-1

| Solutions | Title | NWDAF Selection | Alarm Detection | Model Selection | Model Training |
| --- | --- | --- | --- | --- | --- |
| 1 | Solution #1: Improving Correctness using multiple ML models for an analytics report |  |  | X |  |
| 2 | Solution #2: Improving the Correctness of Service Experience Predictions with Contribution Weights |  |  |  | X |
| 3 | Solution #3: Accuracy based NWDAF Analytics Correctness Improvement |  | X |  |  |
| 4 | Solution #4: Determining ML model drift for improving analytics accuracy |  | X |  |  |
| 5 | Solution #5: Enhancements on ML model provision to improve correctness of NWDAF analytics |  | X |  |  |
| 6 | Solution #6: Correctness improvement of NWDAF by determining ML model performance |  | X |  |  |
| 7 | Solution #7: Enhancements to NWDAF analytics services | X |  |  |  |
| 28 | 23.700-81: KI#1, New Sol: Detect and Improve correctness of NWDAF analytics. |  | X |  |  |
| 29 | 23.700-81: KI#1: New Solution for Detection of ML Model Degradation. |  | X |  |  |
| 30 | 23.700-81: KI #1,New Sol, How to improve correctness of NWDAF analytics |  |  | X |  |
| 31 | 23.700-81: KI#1, New Sol: Multiple analytics outputs based NF action decision. | X |  |  |  |
| 32 | 23.700-81: Solution for KI#1: Enhanced ML model provisioning. |  |  | X |  |
| 33 | 23.700-81: KI#1, New Sol: Improving correctness of NWDAF analytics by providing correction information. |  |  |  |  |
| 34 | 23.700-81: KI #1, New Sol: Accuracy report. | X |  |  |  |
| 35 | 23.700-81: KI #1, New Sol: Area monitoring analytics. | X |  |  |  |
| 36 | 23.700-81: KI #1, New Sol: inference input info. |  |  |  | X |

Most of the solutions could be divided into four groups:

- NWDAF Selection

Solution #7, #31, #34 and #35 believe selecting a good NWDAF will improve the correctness. These solutions assume different NWDAFs support different levels of correctness and there will be always a NWDAF which can fulfil the requirements of the consumer. However, this KI is about how to detect that improving the correctness of an Analytics ID is needed/to detect that degradation on an ML model has happened, i.e. the degradation of correctness can happen within the NWDAF. So these solutions do not solve the real problem. The degradation could still happen after selecting the NWDAF.

- Alarm Detection

Solution #3, #4, #5, #6, #28 and #29 focus on how to detect that improving the correctness of an Analytics ID is needed. The details are shown in Table 1-2.

| Solutions | Title | Who decides correctness improvement is needed | Who will provide input |
| --- | --- | --- | --- |
| 3 | Solution #3: Accuracy based NWDAF Analytics Correctness Improvement | AnLF | MTLF |
| 4 | Solution #4: Determining ML model drift for improving analytics accuracy | MTLF | Consumer NF |
| 5 | Solution #5: Enhancements on ML model provision to improve correctness of NWDAF analytics | MTLF | AnLF MTLF |
| 6 | Solution #6: Correctness improvement of NWDAF by determining ML model performance | AnLF | Consumer NF AnLF |
| 28 | 23.700-81: KI#1, New Sol: Detect and Improve correctness of NWDAF analytics. | AnLF | Consumer NF (Enforcement NF) AnLF |
| 29 | 23.700-81: KI#1: New Solution for Detection of ML Model Degradation. | AnLF | AnLF |

Most solutions propose the Consumer NF to provide the feedback on the enforcement of the Analytic ID. And NWDAF (AnLF or MTLF) will decide whether the improvement is needed. Considering only NWDAF will interact with MTLF currently, let MTLF make the decision will have bigger impact on the current architecture.

Besides, these solutions also propose NWDAF could make the decision by itself in some cases (e.g. based on the interaction between AnLF and MTLF)

- Model Selection

Solutions #1, #30 and #32 focus on how to select the best model, mainly after the improvement. This does not solve the KI directly but can be discussed after the conclusion of Alarm Detection.

- Model Training

Solution #2 and #36 focus on special aspects to train the model. This group can be evaluated independently as they are purely about the improvement itself.

- Others

Solution #33 does not belong to any of the above groups. It proposes a method to notify the consumer about the status of correctness. This does not solve the KI directly but can be discussed after the conclusion of Alarm Detection.

Based on the above discussion. It is proposed to adopt the solutions in Group 2) in normative phase.  Solutions in Group 1) shall be abandoned. Other Groups could be further discussed.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * * All new

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.1	Evaluation on Key Issue #1: How to improve correctness of NWDAF analytics

There are sixteen solutions for KI#1. The details are shown in Table 1-1

Table 1-1

| Solutions | Title | NWDAF Selection | Alarm Detection | Model Selection | Model Training |
| --- | --- | --- | --- | --- | --- |
| 1 | Improving Correctness using multiple ML models for an analytics report |  |  | X |  |
| 2 | Improving the Correctness of Service Experience Predictions with Contribution Weights |  |  |  | X |
| 3 | Accuracy based NWDAF Analytics Correctness Improvement |  | X |  |  |
| 4 | Determining ML model drift for improving analytics accuracy |  | X |  |  |
| 5 | Enhancements on ML model provision to improve correctness of NWDAF analytics |  | X |  |  |
| 6 | Correctness improvement of NWDAF by determining ML model performance |  | X |  |  |
| 7 | Enhancements to NWDAF analytics services | X |  |  |  |
| 28 | Detect and Improve correctness of NWDAF analytics. |  | X |  |  |
| 29 | New Solution for Detection of ML Model Degradation. |  | X |  |  |
| 30 | How to improve correctness of NWDAF analytics |  |  | X |  |
| 31 | Multiple analytics outputs based NF action decision. | X |  |  |  |
| 32 | Enhanced ML model provisioning. |  |  | X |  |
| 33 | Improving correctness of NWDAF analytics by providing correction information. |  |  |  |  |
| 34 | Accuracy report. | X |  |  |  |
| 35 | Area monitoring analytics. | X |  |  |  |
| 36 | inference input info. |  |  |  | X |

Most of the solutions could be divided into four groups:

- NWDAF Selection

Solution #7, #31, #34 and #35 believe selecting a good NWDAF will improve the correctness. These solutions assume different NWDAFs support different levels of correctness and there will be always a NWDAF which can fulfil the requirements of the consumer. However, this KI is about how to detect that improving the correctness of an Analytics ID is needed/to detect that degradation on an ML model has happened, i.e. the degradation of correctness can happen within the NWDAF. So these solutions do not solve the real problem. The degradation could still happen after selecting the NWDAF.

- Alarm Detection

Solution #3, #4, #5, #6, #28 and #29 focus on how to detect that improving the correctness of an Analytics ID is needed. The details are shown in Table 1-2.

| Solutions | Title | Who decides correctness improvement is needed | Who will provide input |
| --- | --- | --- | --- |
| 3 | Accuracy based NWDAF Analytics Correctness Improvement | AnLF | MTLF |
| 4 | Determining ML model drift for improving analytics accuracy | MTLF | Consumer NF |
| 5 | Enhancements on ML model provision to improve correctness of NWDAF analytics | MTLF/AnLF | AnLF MTLF |
| 6 | Correctness improvement of NWDAF by determining ML model performance | AnLF | Consumer NF AnLF |
| 28 | Detect and Improve correctness of NWDAF analytics. | AnLF | Consumer NF (Enforcement NF) AnLF |
| 29 | New Solution for Detection of ML Model Degradation. | AnLF | AnLF |

Most solutions propose the Consumer NF to provide the feedback on the enforcement of the Analytic ID. And NWDAF (AnLF or MTLF) will decide whether the improvement is needed. Considering only NWDAF will interact with MTLF currently, let MTLF make the decision will have bigger impact on the current architecture.

Besides, these solutions also propose NWDAF could make the decision by itself in some cases (e.g. based on the interaction between AnLF and MTLF)

- Model Selection

Solutions #1, #30 and #32 focus on how to select the best model, mainly after the improvement. This does not solve the KI directly but can be discussed after the conclusion of Alarm Detection.

- Model Training

Solution #2 and #36 focus on special aspects to train the model. This group can be evaluated independently as they are purely about the improvement itself.

- Others

Solution #33 does not belong to any of the above groups. It proposes a method to notify the consumer about the status of correctness. This does not solve the KI directly but can be discussed after the conclusion of Alarm Detection.

Editor's note: Solution #1, #2, #30, #32, #33 and #36 will be evaluated after the conclusion of Alarm Detection.

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Interim Conclusion on Key Issue #1: How to improve correctness of NWDAF analytics

Solution #7, #31, #34 and #35 shall not be considered during normative phase.

It is agreed to adopt the following principle of Solution #3, #4, #5, #6, #28 and #29 in normative phase.

- The Consumer NF to provide the feedback on the enforcement of the Analytic ID to help the NWDAF to decide whether the improvement of correctness is needed.

- NWDAF could also make the decision based on the action of AnLF or MTLF.

# * * * * End of changes * * * *