3GPP TSG-WG SA2 Meeting #152E 	S2-2205792

# Electronic Meeting, August 17–26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3517 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification that NEF uses time sync services provided by TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description of NEF service operations are misleading as it may be understood that NEF is a NF that creates/updates/deletes etc. time synchronization configuration and activates/deactivates services. NEF provides time synchronization service exposure as described in clause 5.27.1.8 of TS 23.501, however, the configurations are created/updated/deleted etc. by using services that are provided by TSCTSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the NEF service descriptions that NEF provides the time synchronization service exposure, but the time synchronization services as such are provided by TSCTSF. Additionally, the relevant Tables with required and optional parameters are moved from NEF clauses to TSCTSF clauses where these parameters are the Required/Optional inputs that AF (directly or via NEF) uses to invoke a specific service operation. Furthermore, instead of duplicating all Inputs/Outputs in NEF service description, we provide references to a corresponding service operation in the TSCTSF clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading specification that may result in descrepancy between Stage 2 and Stage 3 leading to implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.25, 5.2.6.28, 5.2.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 5.2.6.25	Nnef_TimeSynchronization service

##### 5.2.6.25.1	General

Service description: This service provides:

-	authorization of NF Service Consumer requests;

-	time synchronization service exposure as described in clause 5.27.1.8 of TS 23.501 [2] when the NF Service Consumer requests to create or update time synchronization configuration as well as to activate and deactivate the time synchronization service, for which the NEF uses service operations provided by TSCTSF as described in clause 5.2.27.

##### 5.2.6.25.2	Nnef_TimeSynchronization_ConfigCreate operation

Service operation name: Nnef_TimeSynchronization_ConfigCreate

Description: the consumer requests to create a time synchronization configuration and activate the time synchronization service with the configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.2).

Inputs, Required: as specified in clause 5.2.27.2.2.

Inputs, Optional: as specified in clause 5.2.27.2.2.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.2.2.

Outputs, Optional: as specified in clause 5.2.27.2.2.

##### 5.2.6.25.3	Nnef_TimeSynchronization_ConfigUpdate operation

Service operation name: Nnef_TimeSynchronization_ConfigUpdate

Description: the consumer requests to update the time synchronization configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.3).

Inputs, Required: as specified in clause 5.2.27.2.3.

Inputs, Optional: as specified in clause 5.2.27.2.3.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.2.3.

Outputs, Optional: as specified in clause 5.2.27.2.3.

##### 5.2.6.25.4	Nnef_TimeSynchronization_ConfigDelete operation

Service operation name: Nnef_TimeSynchronization_ConfigDelete

Description: the consumer requests to delete the time synchronization configuration and deactivate the corresponding time synchronization service, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.4).

Inputs, Required: as specified in clause 5.2.27.2.4.

Inputs, Optional: as specified in clause 5.2.27.2.4.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.2.4.

Outputs, Optional: as specified in clause 5.2.27.2.4.

##### 5.2.6.25.5	Nnef_TimeSynchronization_ConfigUpdateNotify operation

Service operation name: Nnef_TimeSynchronization_ConfigUpdateNotify

Description: Forward the notification for the time synchronization configuration. When the NEF receives a notification of a change corresponding to a time synchronization configuration from the TSCTSF, it forwards the notification by invoking a Nnef_TimeSynchronization_ConfigUpdateNotify service operation to the NF consumer(s) that has subscribed for the event. The event parameters are described in Table 5.2.27.2.5-1.

Known NF Service Consumers: AF.

Inputs, Required: as specified in clause 5.2.27.5.2.

Inputs, Optional: as specified in clause 5.2.27.5.2.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.25.6	Nnef_TimeSynchronization_CapsSubscribe operation

Service operation name: Nnef_TimeSynchronization_CapsSubscribe

Description: The AF request the subscription to receive notifications about time synchronization capabilities for a list of UE(s) or a group of UEs or any UEs using DNN/S-NSSAI combination, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.6).

Event Filters are used to specify the conditions to match for notifying the event. If there are no conditions to match then the Event Filter is not provided. The Event Filters supported by the service are described in Table 5.2.27.2.6-1.

Inputs, Required: as specified in clause 5.2.27.2.6 .

Inputs, Optional: as specified in clause 5.2.27.2.6.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.2.6.

Outputs, Optional: as specified in clause 5.2.27.2.6.

##### 5.2.6.25.7	Nnef_TimeSynchronization_CapsUnsubscribe operation

Service operation name: Nnef_TimeSynchronization_CapsUnsubscribe

Description: The AF requests to unsubscribe from receiving notifications about time synchronization capabilities for a list of UE(s) or a group of UEs or any UE using a DNN/S-NSSAI combination, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.7).

Inputs, Required: as specified in clause 5.2.27.2.7.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.2.7.

Outputs, Optional: as specified in clause 5.2.27.2.7.

##### 5.2.6.25.8	Nnef_TimeSynchronization_CapsNotify operation

Service operation name: Nnef_TimeSynchronization_CapsNotify

Description: Forward the notification for the time synchronization configuration.

When the NEF receives a notification of a change corresponding to a Subscription from the TSCTSF, it forwards the notification by invoking a Nnef_TimeSynchronization_CapsNotify service operation to the NF consumer(s) that has subscribed for the event. The event parameters are described in Table 5.2.27.2.8-1.

Known NF Service Consumers: AF.

Inputs, Required: as specified in clause 5.2.27.2.8.

Inputs, Optional: as specified in clause 5.2.27.2.8.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.25.9	Void

##### 5.2.6.25.10	Void

##### 5.2.6.25.11	Void

##### 5.2.6.25.12	Void

>>>> Next Change <<<<

#### 5.2.6.28	Nnef_ASTI service

##### 5.2.6.28.1	General

Service description: This service provides:

-	authorization of NF Service Consumer requests;

-	time synchronization service exposure as described in clause 5.27.1.8 of TS 23.501 [2] when a NF Service Consumer requests to control the 5G access stratum time distribution configuration, for which the NEF uses service operations provided by TSCTSF as described in clause 5.2.27.

##### 5.2.6.28.2	Nnef_ASTI_Create operation

Service operation name: Nnef_ASTI_Create

Description: the consumer requests to activate the 5G access stratum time distribution, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.2).

Inputs, Required: as specified in clause 5.2.27.4.2.

Inputs, Optional: as specified in clause 5.2.27.4.2.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.4.2.

Outputs, Optional: as specified in clause 5.2.27.4.2.

##### 5.2.6.28.3	Nnef_ASTI_Update operation

Service operation name: Nnef_ASTI_Update

Description: the consumer requests to update the 5G access stratum time distribution configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.3).

Inputs, Required: as specified in clause 5.2.27.4.3.

Inputs, Optional: as specified in clause 5.2.27.4.3.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.4.3.

Outputs, Optional: as specified in clause 5.2.27.4.3.

##### 5.2.6.28.4	Nnef_ASTI_Delete operation

Service operation name: Nnef_ASTI_Delete

Description: the consumer requests to delete the 5G access stratum time distribution configuration and deactivate the corresponding 5G access stratum time distribution service, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.4).

Inputs, Required: as specified in clause 5.2.27.4.4.

Inputs, Optional: as specified in clause 5.2.27.4.4.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.4.4.

Outputs, Optional: as specified in clause 5.2.27.4.4.

##### 5.2.6.28.5	Nnef_ASTI_Get operation

Service operation name: Nnef_ASTI_Get

Description: the consumer makes a query about the status of the access stratum time distribution, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.5).

Inputs, Required: as specified in clause 5.2.27.4.5.

Inputs, Optional: as specified in clause 5.2.27.4.5.

Outputs, Required: Operation execution result indication and, in case of successful operation, any outputs as specified in clause 5.2.27.4.5.

Outputs, Optional: as specified in clause 5.2.27.4.5.

>>>> Next Change <<<<

#### 5.2.27.2	Ntsctsf_TimeSynchronization service

##### 5.2.27.2.1	General

Service description: This service provides:

-	Request authorization of NF Service Consumer requests.

-	NF Service Consumer request to create update and delete time synchronization configuration and to activate and deactivate the time synchronization service as described in clause 5.27.1.8 of TS 23.501 [2].

##### 5.2.27.2.2	Ntsctsf_TimeSynchronization_ConfigCreate operation

Service operation name: Ntsctsf_TimeSynchronization_ConfigCreate

Description: Authorize the request, create a time synchronization configuration and activate the time synchronization service with the configuration.

Inputs, Required: Reference to time synchronization capability set (i.e. the Subscription Correlation ID as in the response to Ntsctsf_TimeSynchronization_CapsSubscribe request), user plane node ID, mandatory service parameters as described in Table 4.15.9.3-1, Notification Target Address.

Inputs, Optional: Optional service parameters as described in Table 4.15.9.3-1.

Outputs, Required: Operation execution result indication, in successful operation the PTP instance reference.

Outputs, Optional: None.

##### 5.2.27.2.3	Ntsctsf_TimeSynchronization_ConfigUpdate operation

Service operation name: Ntsctsf_TimeSynchronization_ConfigUpdate

Description: Authorize the request and forward the request to update the time synchronization configuration.

Inputs, Required: PTP instance reference.

Inputs, Optional: List of UE identities (SUPIs) to be added to the time synchronization configuration. List of UE identities (SUPIs) to be removed from the time synchronization configuration. (g)PTP grandmaster enabled, grandmaster priority, Time Domain, Temporary Validity Condition as described in Table 4.15.9.3-1.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.27.2.4	Ntsctsf_ TimeSynchronization_ConfigDelete operation

Service operation name: Ntsctsf_TimeSynchronization_ConfigDelete

Description: Authorize the request, delete the time synchronization configuration and deactivate the corresponding time synchronization service.

Inputs, Required: PTP instance reference.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.27.2.5	Ntsctsf_ TimeSynchronization_ConfigUpdateNotify operation

Service operation name: Ntsctsf_ TimeSynchronization_ConfigUpdateNotify

Description: Forward the notification for the time synchronization configuration. When the TSCTSF detects a change corresponding to a time synchronization configuration, it invokes Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation to the NF consumer(s) which has subscribed for the event. The following table describes the parameters in the event.

Table 5.2.27.2.5-1: Time Synchronization configuration event parameters

| Event filter | Description |
| --- | --- |
| For each PTP port in the PTP instance |  |
| Either UE identity (for a DS-TT port), or "N6 interface" indication | Identifies the UE/DS-TT which the parameters below apply. "N6 interface" indicates that the parameters below apply to the N6 interface. If the "PTP port" needs to be identified, this field refers to the UE identity (GPSI or SUPI). If the N6 termination needs to be identified, then this field indicates "N6 interface" flag, instead of SUPI or GPSI. |
| PTP port state | Active or Inactive. Active = PTP port state is in Leader, Follower or Passive state. Inactive = PTP port is in any other state. If any of the PTP ports in NW-TT are in Leader, Follower, or Passive, then "active" is reported for N6 interface, otherwise "passive" is reported for N6 interface. |

NOTE:	Leader and Follower terms in this specification are aligned with NOTE 2 in clause 5.27.1.2.2.1 of TS 23.501 [2].

Inputs, Required: PTP instance reference, current state of the time synchronization configuration (i.e. whether the time synchronization configuration is active or not for the PTP ports of the PTP instance).

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.27.2.6	Ntsctsf_TimeSynchronization_CapsSubscribe operation

Service operation name: Ntsctsf_TimeSynchronization_CapsSubscribe

Description: The AF subscribes to receive notification about time synchronization capabilities for a list of UE(s) or a group of UEs or any UE using DNN/S-NSSAI combination.

Event Filters are used to specify the conditions to match for notifying the event. If there are no conditions to match then the Event Filter is not provided. The following table describes the Event Filters supported by the service:

Table 5.2.27.2.6-1: Time Synchronization capability event filters

| Event filter | Description |
| --- | --- |
| List of UE identities or External Group Identifier or Internal Group Identifier | Only the included UE identities (GPSI or SUPI) or Groups of UEs identified by an External Group Identifier/Internal Group Identifier are considered for the notification. |
| PTP instance types | Supported PTP instance types as described in clause 5.27.1.4 of TS 23.501 [2]. |
| Transport protocols | Supported transport protocols for PTP as described in clause 5.27.1.4 of TS 23.501 [2]. |
| Supported PTP Profiles | Identifies the PTP profiles supported by 5GS for the reported UE. |

Inputs, Required: Either a combination of (DNN, S-NSSAI) or an AF-Service-Identifier, and Notification Target Address.

Inputs, Optional: Event Filter(s) as described in Table 5.2.27.2.6-1, Report Type (can be either one-time reporting, periodic reporting or event based reporting).

Outputs, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time.

Outputs, Optional: None.

##### 5.2.27.2.7	Ntsctsf_TimeSynchronization_CapsUnsubscribe operation

Service operation name: Ntsctsf_TimeSynchronization_CapsUnsubscribe

Description: The AF unsubscribes to receive notification about time synchronization for a UE or a group of UEs or any UE using a DNN/S-NSSAI combination.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.27.2.8	Ntsctsf_TimeSynchronization_CapsNotify operation

Service operation name: Ntsctsf_TimeSynchronization_CapsNotify

Description: Forward the notification for the time synchronization configuration.

When the TSCTSF detects an event corresponding to a Subscription, it invokes Ntsctsf_TimeSynchronization_CapsNotify service operation to the NF consumer(s), e.g., a NEF or an AF within the operator’s domain, that has subscribed for the event. The following table describes the parameters in the event.

Table 5.2.27.2.8-1: Time Synchronization capability event parameters

| Time Synchronization event parameter | Description |
| --- | --- |
| List of User-Plane Node IDs | Identifies the applicable NW-TT (NOTE). |
| For each User-Plane Node ID |  |
| (g)PTP grandmaster capable | Indicates separately whether User-Plane Node ID supports acting as a gPTP and/or PTP grandmaster. |
| 5G Access Stratum Time source | Indicates the 5G clock quality supported (i.e. the source of time used by the 5GS). Allowed values: Atomic clock, GNSS, terrestrial radio, serial time code, PTP, NTP, hand set, internal oscillator, other. |
| List of UEs associated with the User-Plane Node ID | A list of UEs associated to each user-plane Node ID. |
| For each UE |  |
| UE identity (GPSI or SUPI) | Identifies the UE to which the reported parameters below apply. |
| PTP instance types | Supported PTP instance types as described in clause 5.27.1.4 in TS 23.501 [2]. |
| Transport protocols | Supported transport protocols for PTP as described in clause 5.27.1.4 in TS 23.501 [2]. |
| Supported PTP Profiles | Identifies the PTP profiles supported by 5GS for the reported UE. |
| NOTE: This is needed to limit the PTP instance into a single NW-TT. In this way the AF can know e.g. UE1/UE2/UE3 are served by NW-TT1, UE4/UE5/UE6 are served by NW-TT2. The AF can control the PTP instances per NW-TT. |  |

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: Time synchronization capabilities as described in Table 5.2.27.2.8-1.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.27.2.9	Void

##### 5.2.27.2.10	Void

##### 5.2.27.2.11	Void

##### 5.2.27.2.12	Void

>>>> End of Changes <<<<