SA WG2 Meeting #S2-152E	S2-2205801

17 - 26 Aug, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #4, Sol #24: update to resolve the editor's note

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to remove the editor's notes in solution #24 .

# Discussion

One editor's note in solution #24 is regarding authorization:

Editor's note:	It is FFS whether and how home network to authorize the UE's request to obtain localized service data, in case home network is different than the current serving network.

Home network can have UE's subscription data to indicate whether UE is allowed to use localized service, similar as the authorization for V2X policy ( see TS 23.287 clause 6.2.2).

Another editor's note is regarding what are the service operation used between AMF and PCF to provision the new UE policy:

Editor's note:	Details of service operations to use and content is FFS.

There are two options, one is during the establishment of the UE policy association, and the other is based on PCF subscribes with AMF to be notified of UE policy container when there is already UE policy association established when UE sends the request.

# Proposal

Add the following update to TR 23.700-08.

*** BEGIN CHANGES ***

## 6.24	Solution #24: Localized service data provisioning via UDR

### 6.24.1	Introduction

This solution addresses requirements listed in Key Issue #4. It proposes to use network exposure for provisioning network with information related to localized service and use UE policy for delivering the information to UE.

This solution covers the function described in solution #7 step H4.

### 6.24.2	Functional Description

There are many characteristics to describe a localized service, e.g.:

-	the name and identifier of the localized service;

-	time validity;

-	location validity;

-	a service provider group identifier for end users who will be offered with localized service;

-	information related to hosting network(s), such as network identity, cost and quality of the localized service, S-NSSAI/DNN to be used for accessing the localized service, valid credential types, etc.

It is assumed that UE is usually registered in a serving network prior to registering to hosting network for localized service. Localized service provider can have business relationship with multiple hosting network operators, so that there could be more than one hosting network providing access to the localized service. After the interactions between localized service provider and the hosting network operators (see solution #7 step H1 and H2), localized service provider has gathered all the characteristics of a localized service and is able to provision the data to the UE's serving network (e.g. HPLMN, VPLMN).

When the serving network is provisioned with data related localized service, UE can obtain the data via UE policy and use the data to perform selection of hosting network and localized service.

NOTE:	How does the UE make use of the localized service data is not discussed in this solution.

### 6.24.3	Procedures

#### 6.24.3.1	Provisioning of non-UE specific localized service data to networks

Figure 6.24.3.1-1: Provisioning of non-UE specific localized service data to networks

The above figure reuses the signalling flow defined in clause 4.15.6.7 of TS 23.502 [4], with the following considerations:

-	AF is an external party to the network, which has business agreement with the localized service provider. The AF represents the localized service provider who has gathered all the characteristics of a localized service as described in clause 6.24.2.

-	The name/identifier of the localized service can be used as service description in the AF request to NEF.

-	Time/location validity and information related to hosting network(s) are the localized service specific information which will be provisioned to serving network and delivered to UE.

-	Nnef_ServiceParameter_Create service operation is not-UE specific and therefore does not include any specific UE Identity or External Group Identifier.

-	Optionally, a service provider group identifier can be allocated by the AF for identifying a determined group of end users who will be offered with localized service, and thus be provided with the localized service data.

-	AF can invoke this procedure with multiple networks that have business agreement with the localized service provider.

-	The localized service data is stored in UDR as "Application Data".

#### 6.24.3.2	Delivering localized service data to the UE via UE policy

Figure 6.24.3.2-1: Delivering localized service data via UE policy

The procedure to deliver the localized service data towards UE is described as below:

0.	UE has registered in a network, and optionally UE policy association is established and PCF has subscribed to be notified of the reception of the UE Policy container (see TS 23.502 [4] clause 4.2.4.3 step 0a).

During the registration procedure, AMF fetches the subscription data from UDM and verifies that UE is allowed to use localized services. Then, AMF indicates to the UE there is support of provisioning localized service data via successful registration.

1.	End user is motivated by service provider to seek for localized service, for example scan a QR code, or login a web page, etc.

2.	Based on the knowledge that the current serving network supports localized service data provisioning, the UE requests such data from the serving network reusing the procedure defined in clause 6.2.4 of TS 23.287 [13].

The UE's request includes information for the network to determine what localized service information to provide to the UE e.g. service identifier associated with the localized service, and/or a "filter" e.g. free text name of localized service, location, time, service provider group identifier.

NOTE:	The information included in the UE request message is obtained via mechanisms outside of 3GPP scope.

3a.	In case there is no UE policy association established, AMF sends Npcf_UEPolicyControl_Create request to establishes UE policy association with PCF, including the UE policy container.

3b.	In case there is UE policy association has been established and PCF has subscribed to be notified of the reception of the UE policy container, AMF sends the Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

4.	PCF queries UDR for the data stored for localized service, based on the information provided by the UE (e.g. localized service identifier, a "filter", etc).

NOTE:	In case the current serving network is different than the home network, in addition to localized service data received from the PCF in the home network, the PCF in the serving network can also provide the UE with localized service data based on information stored in the serving network UDR.

5.	PCF receives the localized service data from the UDR. The data is provisioned and stored in UDR as described in clause 6.24.3.1.

PCF can use service provider group identifier to verify if UE belongs to the correct group of receiving the localized service.

6a.	If step 3a is executed, then PCF accepts the UE policy association establishment, and then step 3-5 in clause 4.2.4.3 of TS 23.502 [4] are used to deliver the UE policy to UE.

6b.	If step 3b is executed, then PCF provisions the UE with the localized service data as part of new UE Policies for localized services, re-using procedure defined in clause 4.2.4.3 of TS 23.502 [4].

### 6.24.4	Impacts on services, entities, and interfaces

AF/NEF/UDR:

-	Support provisioning and storing of localized service data.

UE/PCF:

Support UE triggered localized service data delivery via UE policy delivery procedure.

AMF:

-	Indicate to the UE the network support provisioning of localized service data via NAS.

UDM:

-	Indicate to AMF that UE is allowed to use localized services via subscription data.

*** END CHANGES ***