TSG SA WG2 Meeting #152E	S2-2205806

17 - 26 August 2022, Electronic meeting

# Source:	Ericsson, …

# Title:	New WID: Enhanced support of Non-Public Networks Phase 2

# Document for:	Approval

Agenda Item:	10.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title:	Enhanced support of Non-Public Networks Phase 2

## Acronym:	eNPN_Ph2

## Unique identifier: 	To be allocated

Potential target Release:	Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940075 | Study on enhanced support of Non-Public Networks Phase 2 | Antecedent Stage 2 study item |
| 920031 | 5G Networks Providing Access to Localized Services | Stage 1 work item |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

Non-Public Network was introduced in Rel-16 with basic functions, and further enhanced in Rel-17 to enable wider cooperation between different networks/different entities to support use cases for NPN to provide access for UE that has no native credentials/subscriptions beforehand, as well as IMS and VIAPA related use cases.

FS_eNPN_Ph2 study (Rel-18) has concluded 2 key issues that are proposed, see TR 23.700-08, to be progressed to the normative phase:

-	Enabling support for idle and connected mode mobility between SNPNs without new network selection

-	Support of Non-3GPP access for SNPN

In addition, FS_eNPN_Ph2 study includes additional key issues that are not yet concluded.

## 4	Objective

The aim of this work is to specify the following enhancements:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

a.	UE and AMF support of equivalent SNPN list in NAS.

b.	NG-RAN and AMF support of equivalent SNPNs in NGAP.

c.	UE/NG-RAN/AMF take equivalent SNPN list into consideration, for supporting relevant functions, e.g. idle/connected mode mobility.

NOTE 1:	The above includes/excludes support for equivalent SNPNs, with different SNPN IDs, within an RA.

Editor’s note:	Whether to include or exclude support for equivalent SNPNs, with different SNPN IDs, within an RA is expected to be resolved at SA2#152e.

2.	Support for non-3GPP access for SNPN.

a.	Access to SNPN services via Untrusted non-3GPP access network, including support for Emergency services.

b.	Access to SNPN services via Trusted non-3GPP access network, including support for Credentials Holder, Emergency services and UE onboarding.

c.	N3IWF and TNGF being able to include the “selected NID” in the [NGAP] INITIAL UE MESSAGE, which is up to RAN3 to define.

d.	Access to SNPN services via wireline access network by defining a new GCI including a “NID”.

e.	The NSWO procedure be extended to support UE authentication using SNPN credentials.

NOTE 2:	Additional enhancements can be considered dependent on further progress of FS_eNPN_Ph2 and progress in SA3.

## TU estimates and dependencies

It is expected that the work will require 1.25 TU (0.5 for objective 1 and 0.75 TU for objective 2).

## 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Support for equivalent SNPNs Support for non-3GPP access for SNPN | SA#99 Mar 2023 |  |
| 23.502 | Support for non-3GPP access for SNPN | SA#99 Mar 2023 |  |

## 6	Work item Rapporteur(s)

Hedman, Peter, Ericsson, (peter . hedman @ ericsson . com)

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

CableLabs

Charter

China Mobile

China Unicom

Cisco

Convida Wireless

DISH Network

Ericsson

ETRI

Futurewei

KPN

LG Electronics

Intel

InterDigital

Lenovo

MATRIXX Software

MediaTek Inc

Microsoft

MITRE

Motorola Mobility

NEC

NICT

Nokia

Nokia Shanghai Bell

OPPO

Philips

Qualcomm

Samsung

Sennheiser

Siemens

SHARP

Spreadtrum Communications

T-Mobile USA

vivo

ZTE