3GPP TSG-SA WG2 Meeting #152E	S2-2205835

Elbonia, August 17 - 26, 2022

# Title:	[Draft] Reply to LS on PCC aspects related to 5MBS

# Response to:	(S2-2205398/C3-222578) Questions on PCC aspects related to 5MBS

# Release:	Rel-17

# Work Item:	5MBS

Source:	[Ericsson, will be] SA2

To:	CT3

Cc:	-

Contact Person:

Name:	Judy Gan Juying

Tel. Number:

E-mail Address:	Judy.Gan.Juying@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: 23.247 CR0125

1. Overall Description:

SA2 thanks CT3 for the LS “Questions on PCC aspects related to 5MBS”. SA2 discussed the issues and agreed on the attached CR. SA2 would also like to provide answers to the question as follows:

CT3 Question 1: What information is provided by the AF to the NEF? In particular what do the "service requirements" provided by the AF to the NEF consist of and what are the relationship and restrictions between them? Is it possible to receive several media components (per media type)? How should an application be identified (i.e. more details are needed on the Application Identifier, the AF Communication Service Identifier, the emergency indicator Application service provider defined today in TS 23.247).

[SA2 Answer 1] 
“Service requirement” is now replaced by “MBS Service Information”, which consists of QoS requirement and packet filter.

The AF may provide QoS requirement in the format of 3GPP QoS Information (e.g., 5QI, ARP, MBR and GBR), or in the format of SDP or Media-Component (as specified in TS 29.214/TS29.514).

Application Identifier may be included when AF provides MBS Service Information in the format of Media Component.

AF Communication Service Identifier and Emergency indicator are not applicable for MBS Session.

Please see the update in the attached CR.

CT3 Question 2: Can SA2 elaborate on the content of MBS Session "PCC rules", the supported Policy Control Request Triggers, other relevant policy information derived and provided by the PCF and possible information the NF service consumer can provide to the PCF?

[SA2 Answer 2]

"PCC rules" include information:

PCC rule identifier

Flow-Description(s)

QoS Information (e.g. 5QI, ARP, MBR, GBR)

PCRT may be used.

No other policy information is expected to be provided by the PCF.

Possible information the NF service consumer can provide to the PCF is the MBS Service Information.

Please see the update in the attached CR.

CT3 Question 3: Is an Npcf_MBSPolicyControl_Update service operation needed to convey MBS policy trigger information? If it is the case, can SA2 clarify where in the procedure this interaction applies depending on the related trigger?

[SA2 Answer 3] Yes, please see the update in the attached CR.

CT3 Question 4: Regarding the procedure defined in clause 7.1.1.3 of TS 23.247, CT3 would like to understand the need and motivation behind some interactions, i.e.:

-	For both alternatives, can SA2 indicate what information can be retrieved in step 15 and what is the difference with the information provided in step 18 (for alternative A) / step 24 (for alternative B)? Would it be possible to query the UDR only once?

-	For alternative B, step 20 in clause 7.1.1.3-1, can SA2 clarify what status is expected to be reported since the MBS session is not yet created?

[SA2 Answer 4]

Please see the update in the attached CR.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153E	10-14 October 2022 	Elbonia

TSG-SA2 Meeting #154	14-20 November 2022 	Canada