3GPP SA WG2 Meeting #152E	S2-2205935

e-meeting, 17-26 August 2022

Source:	Nokia, Nokia Shanghai Bell, Qualcomm

Title:	Conclusions for KI#6

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes conclusions and way forward for KI#6 in TR 23.700-25

# Discussion

This paper proposes conclusions and way forward for KI#6 in TR 23.700-25

# Proposal

*** Start of changes ***

# 8	Conclusions

## 8.x 	Key issue #6: Adapting downstream scheduling based on RAN feedback for low latency communication

The following bullet points summarize the principles for the way forward:

- Proactive feedback requires that 5GS and the AF receive time information from the same master clock. Since this assumption cannot hold in all deployment, both pro-active and reactive feedback mode shall be supported.

- The AF provides adaptation capability information of the application to 5GS.

- The proactive feedback consists of BAT- and periodicity preferences.

  - Editor’s note: The exact format of the feedback and if the BAW and multiple periodicity values are provided will be determined in a future meeting.

- The reactive feedback consists of the time offset to current observed BAT.

- Existing signaling is enhanced to support adjustment capability- and RAN feedback provision between the AF and RAN.

  - AF request procedure for adaptation capability provision from AF to 5GS.

  - PDU session modification procedure for adaptation capability provision to RAN and feedback providing from the RAN.

  - Notification control procedure for reactive feedback provision.

- The SMF performs clock translation between the external clock and 5GS clock in case the AF is in a different time domain than 5GS (not needed for reactive feedback).

*** End of changes ***