3GPP SA WG2 Meeting #152E	S2-2205944

e-meeting, 17-26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#3, Sol#18: Update of Solution #18

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes an update to solution #18 for the key issue #3 “Support for controlling 5G time synchronization service based on subscription” to the TR 23.700-25.

# Discussion

## Update to Solution #18

Solution #18 for KI#3 of FS_5TRS_URLLC was agreed in SA2#151E. The open point for this solution and KI#3 in general is what information shall be available in UE subscription. See the related editor’s note below for details

Editor’s note: It is FFS what additional subscription information is needed for timing resiliency (e.g., UE’s KPI for time synchronization, such as clock accuracy, clock drift, jitter, periodicity, start time and/or stop time), and how these subscription information can influence TSCTSF's reports to AF. For example, TSCTSF reports the impacted UEs based on not only the time synchronization status degradation, but also the subscription information such as UE's KPI.

Clock accuracy

For ASTI based time synchronization, the accuracy of time synchronization is defined by the Uu Time Synchronization Error Budget. As already proposed by the solution, the UE subscription may optionally include the Uu Time Synchronization Error Budget.

For (g)PTP based time synchronization, the solution proposes the UE subscription includes a reference to a PTP configuration that the TSCTSF uses to determine preconfigured DNN/S-NNSAI of UE’s PDU session and PTP instance configuration. The PTP instance configuration includes clockQuality attributes which are clockClass, clockAccuracy and offsetScaledLogVariance. If the UE subscription includes a reference to a PTP configuration defining PTP instance configuration the TSCTSF then no explicit clock accuracy parameter needs to be part of UE subscription as the TSCTSF may optionally derive the accuracy requirement from the PTP configuration. Still, for PTP operation in the 5GS, the 5GS clock shall be available in DS-TT for timestamping. The synchronization to the 5GS clock is achieved by access stratum time distribution and thus it should be possible to include the Uu Time Synchronization Error Budget the UE subscription for PTP based time synchronization.

Proposal 1: For both the ASTI based time synchronization and the (g)PTP based time synchronization, the Uu Time Synchronization Error Budget defines time synchronization accuracy requirement to 5GS clock at the Uu interface and it should be possible to include this parameter in the UE subscription. For (g)PTP based time synchronization, it is sufficient if the UE subscription holds only a reference that can be used by the TSCTSF to identify a subscribed PTP instance configuration which includes clock attributes defining clock accuracy (i.e. clockClass, clockAccuracy and offsetScaledLogVariance).

Clock drift

The clock drift between 5G clock and the external GM clock, which is reported by UPF/NW-TT, is used by the SMF to adjust information in TSCAI. As such the clock drift is not related to an individual UE subscription but rather to 5G clock’s performance and it should not be the part of UE subscription.

Proposal 2: The clock drift between 5G clock and the external GM clock shall not be a part of UE subscription because it is not related to individual UE but to 5G system clock’s performance.

Periodicity

The periodicity of provisioning reference time information (RTI) (e.g., SIB9) could be part of the UE subscription. It could be expected that RAN nodes will provide RTI with at least the periodicity included in the subscription, which could be beneficial in some mobility scenarios to ensure the continuity of the service. If the periodicity of RTI is not included in the subscription, then it is up to the RAN implementation to determine the RTI periodicity from the required accuracy (i.e., Uu time synchronization error budget). Another example in which the periodicity could be beneficial is the case that the 5GS would align provisioning of RTI and PTP messages.

Proposal 3: The UE subscription may optionally include the periodicity of provisioning reference time information for ASTI and PTP based time synchronization services.

Start/stop time

Both the ASTI and (g)PTP based time synchronization service operations in Rel-17 allow an AF to specify the temporal validity condition that describes the time period when the service should be active. The inclusion of start and top times in the UE subscription could allow for similar operation without AF request. Particularly for ASTI, the UE has the possibility to indicate its preference regarding the reception of RTI in UE Assistance Information (UAI) which could be used to control the time period when the access stratum time distribution should be active even without an AF request. However, if the UE subscription includes start and stop times then it should be clarified that UE’s preference in UAI shall be considered by gNB only during the time period allowed by the UE subscription.

For (g)PTP based synchronization, the active time of the service could be defined by the existence of PDU session connecting the UE to the PTP domain, i.e., (g)PTP based synchronization is always provided when a PDU session for DNN/S-NSSAI is established and there is a PTP configuration preconfigured at the TSCTSF. However, this may lead to ambiguous scenarios if there is a need for connectivity without (g)PTP based service (e.g., when production machines connected to the network behind a UE are in maintenance). In these scenarios, if there is no start and stop times included in the subscription, then the (g)PTP based synchronization service will have to be provided anyway to the UE or the UE will have to establish a PDU session to different DNN/S-NSSAI for which there is no PTP configuration in the TSCTSF.

Proposal 3: The UE subscription may optionally include start and stop times that define the time a time synchronization service (ASTI or (g)PTP based) should be active. If start and stop times are included in the subscription, a RAN node considers UE’s preference for receiving time information only in the active time of the service defined by the start and stop times. Similarly, the TSCTSF configures (g)PTP service only during the active time defined by start and stop times included in the subscription.

Divergence from UTC

Divergence from UTC is gNB’s clock performance attribute and not specific to an individual UE and as such it shall not be part of UE subscription. Furthermore, it is not clear how a RAN node would use this information, e.g., adjust uncertainty value individually for each UE.

Proposal 4: Divergence from UTC shall not be included in the UE subscription.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes ***

## 6.18	Solution #18: Subscription based control of time synchronization service

### 6.18.1	Introduction

This solution is proposed to solve Key Issue #3: Support for controlling 5G time synchronization service based on subscription. In this Key Issue, the time synchronization service scenarios already supported in 5G Release-17 and Release-16 based on access stratum or (g)PTP time distribution methods are addressed.

### 6.18.2	Functional Description

The solution is based on the following principles:

-	UE subscription data types stored in the UDM are extended to include two new data structures related to time synchronization service:

- 	The “Access and Mobility Subscription data” is extended with the following field:

-	Access Stratum Time Synchronization Service Authorization:

-	Indication whether the UE is authorized to receive RTI reception via RRC dedicated signaling Note this field applies to the gNB using dedicated RRC signaling to disseminate access stratum time information to the UE.

-	(Optionally) Uu time synchronization error budget

-	(Optionally) Start and stop times defining active times of Access Stratum Time Synchronization Service for the UE.

-	(Optionally) Maximum periodicity with which reference time information should be provisioned to the UE.

-	The AMF retrieves “Access Stratum Time Synchronization Service Authorization” in the UDM when the UE registers with the 5GS. If “Access Stratum Time Synchronization Service Authorization” is available, the AMF configures the access stratum time distribution information at the NG-RAN accordingly using Rel-17 NGAP signaling.

- 	A new subscription data “Time Synchronization Subscription data” with the following fields:

-	AF request Authorization:

-	Indicates whether the UE is authorized for an AF-requested time synchronization services (either (g)PTP based or ASTI based).

-	The “Time Synchronization Service Authorization”:

- 	One or more Subscribed time synchronization service ID(s): Provides reference to an authorized PTP configuration pre-configured at the TSCTSF (i.e., DNN/S-NSSAI, PTP instance configuration within the 5GS, including e.g. PTP profile, PTP domain, etc).

-	(Optionally) Uu time synchronization error budget

-	(Optionally) Start and stop times defining active times of Time Synchronization Service for the UE.

-	(Optionally) Maximum periodicity with which reference time information should be provisioned to the UE.

-	For (g)PTP time distribution method, the TSCTSF accesses Time Synchronization Subscription data in the UDM and based on the subscription data, the TSCTSF may enable/disable time synchronization configuration for the UE with or without an AF request for the service. The trigger for the TSCTSF to retrieve this subscription data from the UDM is the notification from the PCF that a UE has established a PDU Session for a specific DNN/S-NSSAI (based on the PCF using Npcf_PolicyAuthorization_Notify service operation).

-	For access stratum time distribution method with AF request, the TSCTSF retrieves Time Synchronization Subscription data available at the UDM and checks if the AF is allowed to request access stratum time synchronization as a service or not.

-	The configuration of access stratum time distribution for a UE via subscription information or AF request is assumed to mutually exclusive meaning that the AF is not allowed to request access stratum time distribution if the UE is authorized to receive RTI according to the “Access and Mobility Subscription data”.

-	Access Stratum Time Synchronization Service Authorization and Time Synchronization Subscription Data is stored at the UDR. The UDM retrieves this subscription data from the UDR for other NFs to access it.

-	For (g)PTP time distribution method, the TSCTSF may modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TT and UMIC to the UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

-	For access stratum time distribution method, if AMF receives Access Stratum Time Synchronization Service Authorization from UDM, then AMF provides 5G access stratum time distribution indication and Uu time synchronization error budget (if part of the Access Stratum Time Synchronization Service Authorization received from the UDM) to NG-RAN. If Uu time synchronization error budget is not part of the Access Stratum Time Synchronization Service Authorization received from the UDM then AMF may send to NG-RAN a Uu time synchronization error budget based on a pre-configured default value.

### 6.18.3	Procedures

#### 6.18.3.1	Procedure for subscription based control of access stratum time synchronization service without AF request

An overall procedure for subscription based control of access stratum time synchronization service without AF request for time synchronization is illustrated in Figure 6.18.3.1-1.

Figure 6.18.3.1-1: Subscription based control of access stratum time synchronization service

1.	The UE performs the registration procedure with the 5GS.

2.	While the registration procedure is configured, the AMF retrieves the UE subscription data stored at the UDM.

3. If Access Stratum Time Synchronization Service Authorization is available at the UDM, the UDM provides it to the AMF.

4.  If the AMF receives Access Stratum Time Synchronization Service Authorization from the UDM in step 3, the AMF may subscribe to notifications for updates at the UDM.

5. When the AMF is setting up or modifying the UE Context in the NG-RAN and AMF has received Access Stratum Time Synchronization Service Authorization from the UDM, then AMF provides access stratum attributes to NG-RAN via N2 message (5G access stratum time distribution indication and  Uu time synchronization error budget if part of the Access Stratum Time Synchronization Service Authorization received from the UDM). If Uu time synchronization error budget is not part of the Access Stratum Time Synchronization Service Authorization received from the UDM then AMF may send to NG-RAN a Uu time synchronization error budget based on a default pre-configured default value.

The registration procedure continues as described in TS 23.502[3].

#### 6.18.3.2	Procedure for subscription based control of access stratum time synchronization service with AF request

An overall procedure for subscription based control of access stratum time synchronization service with asynchronous reception of an AF request for ASTI service is illustrated in Figure 6.18.3.2-1.

Figure 6.18.3.2-1: Subscription based control of access stratum time synchronization service with AF request

1.	The AF request to influence the 5G access stratum time distribution.

2.	The TSCSTF retrieves Time Synchronization Subscription Data available at the UDM:

3.	The TSCTSF determines if the AF is allowed to request ASTI service. If authorized, the TSCTSF proceeds with ASTI service configuration. Otherwise, if the AF is not authorized, steps 4-7 are skipped.

4. 	The TSCTSF sends to the PCF for the UE its request to update the AM policy of the UE (identified by SUPI) containing the 5G access stratum time distribution indication (enable, disable) and optionally the calculated Uu time synchronization error budget (if available).

5.	The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE as described in TS 23.502[3] clause 4.16.2.2 to provide AMF the 5G access stratum time distribution parameters.

6.	The AMF sends the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget (if available) to the NG-RAN via N2 signalling.

7.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthorization operation.

8. The TSCTSF responds the AF with the Ntsctsf_ASTI_Create service operation response.

#### 6.18.3.3	Procedure for subscription based control of (g)PTP time synchronization service without AF request

An overall procedure for subscription based control of (g)PTP time synchronization service without AF request is illustrated in Figure 6.18.3.3-1.

Figure 6.18.3.3-1: Subscription based control of (g)PTP time synchronization service without AF request

1.	The UE performs the UE-requested PDU Session Establishment for data establishing connectivity.

2.	The PCF determines if the PDU Session is potentially impacted by time synchronization service and invokes Npcf_PolicyAuthorization_Notify service operation to the TSCTSF discovered and selected for time synchronization to indicate there is a UE connected to a specific DNN/S-NSSAI configured for (g)PTP services.

3.	The TSCSTF uses the SUPI to retrieve the Time Synchronization Subscription Data available at the UDM.

4.	If the Time Synchronization Subscription Data contains:

a) 	one or more Subscribed Time Synchronization Service ID(s) that can be mapped to PTP instance configuration(s), the TSCTSF determines if one or more of the PTP instance configurations match with the DNN/S-NSSAI of the given PDU Session. The TSCTSF assumes that the time-synchronization service cannot be controlled by an AF for the given SUPI.

b) 	An indication that an AF-requested (g)PTP time synchronization service is allowed for the given UE and DNN/S-NSSAI, the TSCTSF adds the given SUPI to the list of SUPIs for which the time-synchronization service can be controlled by an AF.

c) 	If TSCTSF receives neither a) nor b), the TSCTSF assumes that the time-synchronization service cannot be controlled by an AF for the given SUPI.  The TSCTSF releases the AF-session with the PCF.

5. 	For each matching PTP instance configuration determined in step 4, if no PTP instance exists for the given PTP instance configuration, the TSCTSF initializes the PTP instance in 5GS as described in clause K.2.2 of TS 23.501 [2]. The TSCTSF configures a PTP port in DS-TT and adds it to the corresponding PTP instance in NW-TT as described in clause K.2.2 of TS 23.501 [2].,

#### 6.18.3.4	Procedure for (g)PTP time synchronization service with AF request

An overall procedure for (g)PTP time synchronization service with AF request. Steps 1 to 4 below are illustrated in figure 6.18.3.3-1.

1.	The UE performs the UE-requested PDU Session Establishment for data establishing connectivity.

2.	Upon PDU Session establishment, the PCF determines if the PDU Session is potentially impacted by time synchronization service and invokes Npcf_PolicyAuthorization_Notify with the TSCTSF as configured in the PCF. The TSCTSF setups an AF-session with the PCF as described in clause 4.15.9.2 in TS 23.502 [x].

3.	The TSCSTF uses the SUPI to retrieve the Time Synchronization Subscription Data available at the UDM.

4.	If the Time Synchronization Subscription Data contains:

a) 	one or more Subscribed Time Synchronization Service ID(s) that can be mapped to PTP instance configuration(s), the TSCTSF determines if one or more of the PTP instance configurations match with the DNN/S-NSSAI of the given PDU Session. In this case, for each such PTP instance configuration, if no PTP instance exists for the given PTP instance configuration, the TSCTSF initializes the PTP instance in 5GS as described in clause K.2.2 of TS 23.501 [2]. The TSCTSF configures a PTP port in DS-TT and adds it to the corresponding PTP instance in NW-TT as described in clause K.2.2 of TS 23.501 [2]. The TSCTSF assumes that the time-synchronization service cannot be controlled by an AF for the given SUPI.

b) 	An indication that an AF-requested (g)PTP time synchronization service is allowed for the given UE and DNN/S-NSSAI, the TSCTSF adds the given SUPI to the list of SUPIs for which the time-synchronization service can be controlled by an AF.

c) 	If TSCTSF receives neither a) nor b), the TSCTSF assumes that the time-synchronization service cannot be controlled by an AF for the given SUPI. The TSCTSF releases the AF-session with the PCF.

5.	Based on received subscription data in step 4, the TSCTSF determines that the UE is authorized for the service, and the TSCTSF proceeds with rest of the procedure as described in clause 4.15.9.2 in TS 23.502 [x].

When the TSCTSF receives Ntsctsf_TimeSynchronization_CapsSubscribe service operation from the AF, the TSCTSF determines the matching AF-session(s) as described in clause 4.15.9.2 in TS 23.502 [x]. If the UE is not authorized for the service for the given DNN/S-NSSAI in the UDM, the TSCTSF does not consider the AF-session as matching. This implies that the time synchronization capability event notification does not list any of the SUPIs that cannot be controlled by an AF.

### 6.18.4	Impacts on services, entities and interfaces

-	UDR:

-	Storage and retrieval of Time Synchronization Subscription Data by the UDM.

-	UDM:

-	Time Synchronization Subscription Data and Access Stratum Time Synchronization Service Authorization fields management.

-	AMF:

-	Receive Access Stratum Time Synchronization Service Authorization from UDM.

-	Provide access stratum time distribution indication and Uu time synchronization error budget to RAN based on Access Stratum Time Synchronization Service Authorization received from UDM

-	TSCTSF:

-	Time synchronization service configuration (i.e., ASTI and/or (g)PTP time distribution) based on UE’s Time Synchronization Subscription Data.

-	Subscription to Time Synchronization Subscription Data at the UDM.

*** End of changes ***