SA WG2 Meeting #152E (e-meeting)	S2-2205961

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes conclusion for Key Issue #1.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	Conclusions for Key Issue #1

It is concluded that the following principles are to be used as basis for normative work:

-	The AMF determines which network slice needs to be re-mapped to other slice.

-	For PDU Session Establishment case,

-	the AMF provide re-mapped slice to the SMF together with the S-NSSAI the UE requested when the AMF sends PDU Session Establishment Request message to the SMF. The re-mapped slice shall be part of Allowed NSSAI of the UE.

-	the SMF notifies the slice re-mapping to the UE by including re-mapped slice in the PDU Session Establishment Accept message.

-	the SMF may include back-off timer for the S-NSSAI the UE requested so that the UE does not request a new PDU Session with the S-NSSAI the UE requested.

-	For existing PDU Session case,

-	the AMF provide re-mapped slice to the SMF by triggering Nsmf_PDUSession_UpdateSMContext. The re-mapped slice shall be part of Allowed NSSAI of the UE.

-	the SMF sends PDU Session Modification Command to the UE and indicates target slice.

-	the UE triggers PDU Session Establishment procedure to establish a PDU Session with the target slice and the DNN of PDU Session the PDU Session Modification Command is received.

-	the SMF may include back-off timer for the S-NSSAI the UE requested so that the UE does not request a new PDU Session with the S-NSSAI the UE requested.

* * * * End of Changes * * * *