SA WG2 Meeting #152E (e-meeting)	S2-2206108

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI#1, Conclusion

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper provides conclusion for KI#1 "URSP in VPLMN".

Discussion

Regarding Key Issue #1: URSP in VPLMN in clause 5.1 of TR 23.700-85v0.3.0, there are 9 solutions.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 | X |  |  |  |
| 6 | X |  |  |  |
| 27 | X |  |  |  |
| 28 | X |  |  |  |
| 29 | X |  |  |  |

This paper provides conclusion for KI#1 based on these solutions.

Proposal

It is proposed to agree the following changes into TR 23.700-85v0.3.0.

* * * * Start of 1st Change * * * *

!!! All New Texts !!!

8.x	Key Issue #1: URSP in VPLMN

For Key Issue #1, the following aspects are concluded:

-	Regarding which PLMN determines the URSP for VPLMN, it is concluded that HPLMN determines the URSP for VPLMN.

-	Regarding how and which information VPLMN provides to HPLMN in order to assist HPLMN in generating URSP for the VPLMN, Solution#5 that proposes to provide Service Parameters from V-AF to H-PCF by V-PCF is selected for the normative work.

* * * * End of Changes * * * *