3GPP TSG-SA2 Meeting #152E (e-meeting)	S2-2206186

Elbonia, August 17 - 26, 2022 	(Revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0110 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDCP SDU Types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 6.1.2.2 of TS 23.304, the supported SDU type for user plane of NR PC5 only lists IP, Ethernet and Unstructured PDCP SDU types. In LS S2-225464/R2-2206597 Reply LS on SDU type used over user plane for NR PC5 reference point, RAN2 agrees to introduce three new code points of PDCP SDU type to support "Ethernet", "Unstructured" and "ARP" data unit type for 5G ProSe. In clause 5.3.1 of TS 23.304, the following description also states the Address Resolution Protocol data unit type is supported for BC and GC mode communication: For broadcast and groupcast mode 5G ProSe Direc Communication, the following data unit types are supported: IPv4, IPv6, Ethernet, Unstructured, and Address Resolution Protocol (see RFC 826 [19]). The CR is proposed to add Address Resolution Protocol SDU type in PC5 User Plane Protocol stack in clause 6.1.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Address Resolution Protocol SDU type in PC5 User Plane Protocol stack in clause 6.1.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Address Resolution Protocol SDU type is missing in PC5 User Plane Protocol stack in clause 6.1.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ******************

6.1.2	User Plane

6.1.2.1	General

The user plane stack consists of protocols for data transmission via:

-	5G ProSe Direct Communication, specified in clause 6.1.2.2;

-	5G ProSe UE-to-Network Relay, specified in clause 6.1.2.3.

6.1.2.2	UE - UE

Figure 6.1.2.2-1 depicts a user plane for NR PC5 reference point, i.e. PC5 User Plane Protocol stack.

Legend:

-	PC5-U: The SDAP/PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [12].

Figure 6.1.2.2-1: User Plane for NR PC5 reference point

IP, Ethernet, Unstructured and Address Resolution Protocol PDCP SDU types are supported. For IP PDCP SDU type, both IPv4 and IPv6 are supported.

NOTE:	Address Resolution Protocol is only supported for broadcast and groupcast mode 5G ProSe Direc Communication.

The packets from ProSe application layer are handled by the ProSe layer before transmitting them to the AS layer, e.g. ProSe layer maps the IP, Ethernet and Unstructured packets to PC5 QoS Flow and marks the corresponding PFI.

****************** END OF CHANGES ***************