3GPP TSG SA WG2 Meeting #152E e-meeting 	S2-2206189

Elbonia, August 17th –26th, 2022	(revision of)

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NSSAA and Simultaneous Use of Network Slices constraints features interaction

Document for:	Discussion and agreement

Agenda Item:	8.4

Work Item / Release:	eNS_ph2 Rel-17

Abstract: This discussion paper provides background on related CRs

# 1. Technical background

CT1 in their LS C1-224073 "on NSSRG restriction on pending NSSAI" have the following question:

"Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?"

It is our view that the UE should not be constrained to not request S-NSSAI incompatible with a pending S-NSSAI as the NSSRG constraint applies only to S-NSSAIs in the Allowed S-NSSAI. Not allowing the UE to request an S-NSSAI incompatible with a pending S-NSSAI could result in applications in the UE to not be able to run as needed. For example, the UE which had requested S-NSSAI X subject to NSSAA previously may want to exploit S-NSSAI Y and is not interested in using S-NSSAI X because different applications started running on the UE and the optimal network slice for the applications is S-NSSAI Y. There is potentially a case the NSSAA step can take quite some time (e.g. due to some unexpected event in the AAA server (e.g. malfunction, congestion, non reachability)) and so if this happens the UE can only request any other S-NSSAI incompatible with this S-NSSAI only after the failure of the NSSAA procedure causes the S-NSSAI to become rejected from pending. this failure can be subject to timeouts that are not small.

We propose that when the NSSAA completes for the S-NSSAI subject to NSSAA (let's call it S-NSSAI X), if the S-NSSAI that was requested while NSSAA was ongoing (let's call it S-NSSAI Y) was included in the Allowed NSSAI and is incompatible with S-NSSAI X, then the AMF should just remove the S-NSSAI X from pending NSSAI with a UE configuration update (if S-NSSAI X is not removed from the pending NSSAI, the UE will not be able to request S-NSSAI X anymore). If the S-NSSAI X was the only S-NSSAI which is pending, the UCU includes an "empty" Pending S-NSSAI so that the UE has no S-NSSAI in the Pending NSSAI.

# 2. conclusion and proposal

It is proposed that the above forms the basis of our response to CT1. CRs in S2-2206152 and S2-2206159 implement this.