SA WG2 Meeting #152E	S2-2206192

# Aug 17th – 26th, 2022; Elbonia

Source: 	Samsung

Title: 	Update Sol#18 and proposal for removing EN of Sol#12

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This solution proposes the additional steps that may be involved in the procedure described in Sol#18.

# 1	Discussion

This update describes how the Serving network (which can be VPLMN) may trigger the SoR procedure when UE moves to the area (TA, cell ID) where localized service is offered.

Also the removal of following 2 ENs in Solution 12 are proposed:

- Editor's note:	How UDM in home network involving in the SoR procedure is FFS.

- Editor's note:	It is FFS how the AMF triggers the home network to update SoR information during mobility registration update.

Possible use cases of the update proposed can be found from the Clause 5.11 of TR 22.844, where a Home network may choose to steer only some of the UEs which are in the service area of a Hosting network, for e.g. because Visited Network is not able to provide sufficient service quality to the UEs of subscribed Home network. So apart from UE moving in an “Area of Interest” UDM may ask for the number of UEs present in the that Area or information related to UE’s experienced QoS at the time of reporting, so that the Home Network can make decisions whether it is required to steer the UE to Hosting network or not.

As per current standards, the AMF trigger SoR procedure only during “Initial Registration” or “Emergency Registration”. This updated solution proposes how H-UDM can be notified of UE entering the “interested area” , so that the SoR procedure can be triggered.

# 2 Proposal

It is proposed to do the following updates in TR 23.700-08 FS_eNPN_Ph2.

* * * First Change * * * *

## 6.12	Solution #12: Discovering services offered by SNPN while camping in a serving network

### 6.12.3	Procedures

#### 6.12.3.1	UE discovery, selection and access for hosting network using Registration procedure

Figure 6.12.3.1-1: UE discovery, selection and access for hosting network

1.	Registration Request (Mobility Update) update triggered due to TAC changes.

2.	Serving Network detects that the UE moved to area (TA, cell id) where localized service is offered.

3.	In TAs where localized services is available, the serving network includes hosting network information as part of Registration procedure. The serving network may also include necessary information regarding hosting network (SNPN ID, GIN, DNN, S-NSSAI) for it to obtain limited connectivity to hosting network.

4.	Alternative to step 3, the hosting network information can be provided by the SMF in a PCO during PDU session establishment or update. The AMF may provide an indication in Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContext to the SMF to provide hosting network related information to the UE.

5.	UE may initiate PDU Session using DNN, S-NSSAI information received step 3 or 4 that the network redirects to the localized services  portal (PCF policies installed using PDR/FAR rules to redirect to localized services portal).

NOTE 1:	UE can use internet PDU Session, reactivate UP and simply use the link provided by the network e.g. in the PCO to reach the hosting network.

6.	Depending on information, subscription and authorization obtained for hosting network, UE may connect to hosting network using NWu connectivity or select hosting network directly

NOTE 2:	If UE should use hosting network as the underlay network later on and the PLMN as overlay, it might be necessary to provide N3IWF address of the PLMN as well.

#### 6.12.3.2	UE discovery, selection and access for hosting network using UE Configuration Update procedure

Figure 6.12.3.2-1: UE discovery, selection and access for hosting network using UE Configuration Update procedure

1.	UE triggers Registration Request (Mobility Update) to TAC change and Registration procedure completes.

2.	Serving Network detects that the UE moved to area (TA, cell id) where localized service is offered.

3.	When localized services are available, the serving network using UE Configuration Update may provide UE with information of available hosting network(s) and other information as well as trigger the UE to perform scanning of hosting network(s) and if needed also provide the UE a trigger for the UE to determine whether to it needs to initiate change to SNPN selection mode.

4.	Depending on information, received in step 3 the UE may select and connect to hosting network directly or using NWu.

NOTE:	If UE should use hosting network as the underlay network later on and the PLMN as overlay, it might be necessary to provide N3IWF address of the PLMN as well.

#### 6.12.3.3	UE discovery, selection and access for hosting network using Steering of Roaming procedure

Figure 6.12.3.3-1: UE discovery, selection and access for hosting network using SoR procedure

1.	Registration Request (Mobility Update) update triggered due to TAC changes.

2.	Serving Network detects that the UE moved to area (TA, cell id) where localized service is offered.

Note: The H-UDM can be triggered for SoR procedure during Mobility Registration update as per the procedure in Sol#18.

3.	In TAs where localized service is available, Steering of Roaming procedure is used to update information of available hosting network(s) and other information to UE.

4.	Depending on the information, subscription and authorization obtained for home network in SoR, UE may select and connect to hosting network directly.

### 6.12.4	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services, entities, and interfaces.

* * * Second Change * * * *

## 6.18	Solution #18: Steering of UE to select hosting network for obtaining localized services

### 6.18.1	Introduction

One of the aspects of the key issue #5 is how the network would determine the need to steer or instruct the UE to hosting network and other aspect is on how actually network would steer the UE toward the hosting network.

The proposed solution focuses on the case when based on location and time, certain localized services are available, which are being provided by the hosting network and using "Steering of Roaming" mechanism to allow service provider (PLMN or 3rd Party service provider) to update the UE with required information on availability of hosting networks along with the services they are providing. This information will assist the UE to make the switch to appropriate hosting network for obtaining localized services.

Steering of Roaming information container would carry the "operator defined hosting network selector list", which will assist the UE to switch to hosting network for obtaining localized services.

### 6.18.2	Functional Description

The proposed solution will be using similar concept and mechanism to "Steering of Roaming" as defined in TS 23.122 [6].

As per the service level agreement between the PLMN operator and hosting network operator/service provider, UE's HPLMN may create a "operator defined hosting network selector list" for the purpose of automatically steering the UE between PLMN and hosting networks.

The "operator defined hosting network selector list" may consist of following information:

-	A prioritized list of hosting network identifiers (SNPN IDs).

-	A validity time window may be associated with the hosting network identifier i.e. SNPN ID in the "operator defined hosting network selector list".

-	A valid list of tracking areas (TAs) may be associated with the hosting network identifier i.e. SNPN ID in the "operator defined hosting network selector list". The total TAs in this list also indicates the service area of the local services that the UE is allowed to access.

Also Home network (H-UDM) may decide to subscribe when a particular UE enters a specific area (Service area of a Hosting Network which may be hosting a particular event). This location information may be coordinated to the Home network during the service agreement of Hosting network and Home network.

### 6.18.3	Procedures

Figure 6.18.3-1 shows the flow of steering of roaming information container with "operator defined hosting network selector list" to assist in UE with selection of hosting network. This procedure can be triggered during or after registration, UE's location, and time window are critical factors in deciding when this procedure is triggered by the UDM/SoR-AF/SP (Service Provider).

Figure 6.18.3-1: Steering the UE with hosting network information

Procedure:

- The SoR-AF/SP to HPLMN UDM: Nudm_ParameterProvision is sent to the HPLMN UDM to trigger the update of the UE with new list of "operator defined hosting network selector list" or to trigger subscribing for Exposure services for interested location area.

- Namf_EventExposure_Subscribe : H-UDM subscribes for notification for UE moving in or out of the subscribed “Area Of Interest”

- UE moves to the area of interest

- UE/NG-RAN does a mobility update/location reporting to the Serving Network AMF.

- Namf_EventExposure_Notify: AMF reports the event specific parameters that were subscribed to the H-UDM.

6a-b. H-UDM may provide the info to the SoR-AF that may take decision to provide UE the Hosting Network information.

7.	The HPLMN UDM to the AMF: The UDM notifies the changes of the user profile to the affected AMF by the means of invoking Nudm_SDM_Notification service operation.

8.	The AMF to the UE: the AMF sends a DL NAS TRANSPORT message to the served UE. The AMF includes in the DL NAS TRANSPORT message the steering of roaming information received from the UDM.

9.	UE: Upon receiving the SoR information, UE shall perform the security check on it.

10.	The UE to AMF: If the UDM has requested an acknowledgement from the UE in the DL NAS TRANSPORT message, the UE sends an UL NAS TRANSPORT message to the serving AMF with an SOR transparent container including the UE acknowledgement.

11.	The AMF to the HPLMN UDM: If the UL NAS TRANSPORT message with an SOR transparent container is received, the AMF uses the Nudm_SDM_Info service operation to provide the received SOR transparent container to the UDM.

12.	The HPLMN UDM to the SOR-AF: HPLMN UDM informs the SoR-AF/Service Provider (SP) about the successful delivery of the SoR information (i.e. "operator defined hosting network selector list").

13.	The UE will use the received steering of roaming information to assist in selection of the hosting network for obtaining localized services. The UE would select an SNPN if available and allowable (e.g. the time and location is within the validity time window and valid list of TA associated with the SNPN) for obtaining localized services, identified by an SNPN identity contained in the "operator defined hosting network selector list" in priority order.

Editor's note:	Switch between SNPN/PLMN access mode for network selection is FFS.

### 6.18.4	Impacts on services, entities, and interfaces

UDM:

-	Support for additional list (i.e. "operator defined hosting network selector list") in steering of roaming information.

SoR-AF/SP:

-	Support for additional list (i.e. "operator defined hosting network selector list") in steering of roaming information.

UE:

-	Shall support configuration and handling of the new list (i.e. "operator defined hosting network selector list").

-	Use the "operator defined hosting network selector list" for selection of hosting network.