SA WG2 Meeting #S2-152E	S2-2206200

17 - 26 August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3685 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on 5G access stratum distribution in mobility and AM policy modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the 150E meeting, SA2 send the LS (S2-2203621) to RAN3 to confirm the RAN imapct on this. The CR#3265 (S2-2204912) modify 23.501 according to RAN3 response in S2-2203665. But the scenario for mobility registration and AM policy modification is pending for further discussion. In the 38.413 clasue 9.2.2.1 (INITIAL CONTEXT SETUP REQUEST) and 9.2.2.7 (UE CONTEXT MODIFICATION REQUEST), the AMF send the Time Synchronisation Assistance Information wihich includes the 5G access stratum time distribution and the Uu time synchronization error budget to NG-RAN. To keep alignment with RAN WG, it proposes to add the mobility registration and AM policy modification case. In the 23.502, service request case has been added, it proposes to keep with 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AMF inform the target RAN the 5G access stratum time distribution and the Uu time synchronization error budget for the UE in the mobility registration,service request and AM policy modification case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenario for mobility registration,service request and AM policy modification is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.27.1.9	Support for control of time distribution methods

The AF may request a time synchronization distribution method and it may include time synchronization error budget in the request. If the AF included a time synchronization error budget in the AF request, the TSCTSF derives the error budget available for the NG-RAN to provide the 5G access stratum time to each targeted UE via the Uu interface (referred to as Uu time synchronization error budget hereafter). The TSCTSF uses the time synchronization error budget provided by AF (directly or via NEF) in order to derive the Uu time synchronization error budget.

To derive the Uu time synchronization error budget for each targeted UE, the TSCTSF takes the following into account:

-	selected time synchronization distribution method (5G access stratum time distribution or (g)PTP based time distribution);

-	whether 5GS operates as a boundary clock and acts as a GM or whether a clock connected to DS-TT/NW-TT acts as GM in case of (g)PTP based time distribution;

-	PTP port states in case of (g)PTP based time distribution;

-	CN and Device parts of the time synchronization error budget (which may be predefined, or calculated by the 5GS by means that are based on implementation).

If the AF has not included a time synchronization error budget, the TSCTSF uses a preconfigured time synchronization error budget to derive the Uu time synchronization error budget. TSCTSF provides a 5G access stratum time distribution indication and the Uu time synchronization error budget to NG-RAN as described in clause 4.15.9.4 of TS 23.502 [3]. Based on this, NG-RAN provides the 5G access stratum time to the UE according to the Uu interface time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN). During Handover, Service Request, mobility registration and AM policy modification procedure, the AMF may provide the 5G access stratum time distribution indication and the Uu time synchronization error budget to NG-RAN as described in clause 4.15.9.4 of TS 23.502 [3], if needed.

NOTE:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [27].

END OF CHANGES