SA WG2 Meeting #S2-152E	S2-2206207

17 - 26 August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0130 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarificaiton on the mobility in RRC inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the 23.247, it describes the connected mobility procedure for multicast MBS session. But one scenario is missing, i.e. RRC inactive mobility. When the Multicast MBS session is deactivated, the UE may be in RRC inactive state. The UE may resume the connectgion in another RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause to describe the mobility in RRC inactive state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenarios for RRC inactive mobility is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.3.X	UE Triggered Connection Resume in RRC Inactive for inactive MBS session

If the MBS session in Inactive state, the UE may be in CM-CONNECTED with RRC Inactive state. The UE may resume the connection in a different NG-RAN node as specified in the TS 23.502 [6] clause 4.8.2 with following additional enhancement:

-	an MBS non-supporting target NG-RAN retrieves the UE context excluding MBS session information from source RAN node.

-	an MBS supporting target NG-RAN node retrieves all the UE context including MBS session information from source RAN node.

-	If the MBS supporting target NG-RAN node does not establish the 5GC Shared MBS traffic delivery for the MBS session, it triggers setup of the resources for the 5GC Shared MBS traffic delivery, see clause 7.2.1.4 for details.

-	The target NG-RAN initiate the path switch as specified in the clause 7.2.3.2.

END OF CHANGES