SA WG2 Meeting #SA2-152E 	S2-2206217

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	Conclusion for KI#1 service area

Document for:	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC

Abstract of the contribution: This document proposes the conclusion for KI#1 service area.

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-74.

# * * * * Start of change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

8.1	Key Issue #1: Enhance group attribute management

Editor's note:	It is FFS the conclusion for KI#1.

Conclusion for KI#1 service area：

1. 5G LADN mechanism and SMF service area mechanism need to be introduced to enforce the service area applicable to the group of UE.

-	UDM gets the 5G VN group service area.

-	For LADN mechanism: in registration procedure, the AMF gets the 5G VN group service area from UDM, and check whether there are overlapping between LADN service area and 5G VN group service area.

-	If there is some overlapping between two areas, the overlapping area (TAs) can be set as the new LADN service area for this UE.

-	For SMF service area mechanism: in PDU session establishment procedure, the SMF is responsible to check whether the UE is located within the 5G VN group service area, and decides whether the PDU session can be established or not.

# * * * *end of change * * * *