3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206316

Elbonia, August 17th – 26th, 2022	(revision of S2-2206316)

Source:	Huawei, HiSilicon

Title:	KI#3: Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes to evaluate the solutions for Third Party specific user identities.

1. Introduction/Discussion

There are 5 solutions proposed for the KI #3. The following is a table listing a number of criteria that can be used to compare and evaluate the solution.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

7	Overall Evaluation

7.x Evaluation for KI#3

| # | Solution #10 | Solution #11 | Solution #12 | Solution #13 | Solution #19 |
| --- | --- | --- | --- | --- | --- |
| Name | Support of Third Party specific User Identities in IMS re-using the Ms reference point and STIR/SHAKEN | Verification of calling party when using third party specific identity | Solution of SHAKEN based third-party specific user identities | third party specific user identities | Light weight solution of third party specific ID using existing mechanisms |
| Subscriber type supported | SIP Phone and PBX(peer mode and single reg mode) | SIP Phone and PBX(single reg mode) | SIP Phone and PBX(peer mode and single reg mode) | SIP Phone and PBX(peer mode and single reg mode) | SIP Phone and PBX(single reg mode) |
| Group level subscription/ Third Party level subscription | No | Yes | No | Yes | No |
| Subscriber level subscription/ Third Party user level subscription | Yes | Yes | Yes | No | Yes |
| Carrier provision data change frequency for employee job change | Op1: Zero Op2a: Zero Op2b: High | High | Zero | Zero | High |
| Service relies on Third Party service | Op1: Yes Op2a: No Op2b: No | No | Yes | Yes | No |
| Cornerstone services | STIR/SHAKEN, name card in SIP signaling | STIR/SHAKEN, name card in SIP signaling | STIR/SHAKEN, name card in SIP signaling | Customized Ring Signal (CRS), name card in RTP | STIR/SHAKEN, name card in SIP signaling |
| SIP enhancement | SIP and JWT | SIP and JWT | Token info carried in SIP Message | Token info carried in SIP Message | SIP and JWT |
| Subscriber experience | Name card info (e.g. name, images link, location info) can be passed to the terminating end. | Name card info (e.g. name, images link, location info) can be passed to the terminating end. | Name card info (e.g. name, images link, location info) can be passed to the terminating end. | Name card info (e.g. name, images link, location info) can be passed to the terminating end. Carrier and enterprise can also co-design caller name card template with diversified H5 element. | Name card info (e.g. name, images link, location info) can be passed to the terminating end. Custom info, e.g. calling intention, location cannot be displayed. |

# * * * * Second change * * * *

9	Conclusions

9.x	conclusions of KI#3

Editor Note: this clause will be updated based on discussion.

# * * * * End of changes * * * *