3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206320

Elbonia, Aug 17th – 26th, 2022	(revision of S2-2204398)

# Source:	Huawei, HiSilicon

# Title:	New WID on Enhanced IMS to 5GC Integration Phase 2

# Document for:	Approval

# Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Enhanced IMS to 5GC Integration Phase 2

Acronym:	TEI18_eIMS5G2

Unique identifier:

Potential target Release:	Release 18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 780029 | Study on Enhanced IMS to 5GC Integration | Phase 1 of study on enhanced IMS to 5GC Integration |
| 911032 | Study on enhanced IMS to 5GC Integration Phase 2 | Phase 2 of study on enhanced IMS to 5GC Integration |

3	Justification

Considering the conclusions of FS_eIMS5G2, there is a need to have a stage 2 work item to develop stage 2 for the requirements developed by CT1, CT3 and CT4 during the study phase.

Three liaisons C1-216839 (S2-2200042), C1-220734 (S2-2200082) and C4-220445 (S2-2200204) were sent to SA2 for further evaluation.

4	Objective

The objectives of this normative work item are to enhance the necessary stage 2 specifications to specify the requirements developed by CT1, CT3 and CT4 during the study phase, which are specified in TR 23.700-10, TR 23.700-11 and TR 23.700-12.

The stage-2 may include the following (non-exhaustive, additional areas can be identified based on progress in SA2) aspects.

-	Distinguish different IMS networks by providing "IMS home network domain name" in the traffic descriptor, this addresses how a UE which is allowed to access multiple IMS networks can decide PDU session attributes (including S-NSSAI) of a PDU session providing access to each of the IMS networks.

-	Routing of IMS traffic via localized IMS media plane functions, including conveying UPF FQDN to IMS nodes and/or conveying UPF service area to IMS nodes via PCF N5 and discover NFs from IMS nodes using UPF service area.

The following additional aspects have to be considered by SA2 for submitted CRs:

- Roaming Impacts.

- Optimization of selected IMS AGW to ensure e2e UPF optimization

The estimated TU for this work item is about 1 TU.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.228 | Routing of IMS traffic via localized IMS media plane functions | SA#99 |  |
| 23.501 | Convey UPF FQDN to IMS nodes Convey UPF service area to IMS nodes via PCF N5 Discover NFs from IMS nodes using UPF service area | SA#99 |  |
| 23.502 | Convey UPF FQDN to IMS nodes Convey UPF service area to IMS nodes via PCF N5 Discover NFs from IMS nodes using UPF service area | SA#99 |  |
| 23.503 | Distinguish different IMS networks by providing "IMS home network domain name" in the traffic descriptor | SA#99 |  |

6	Work item Rapporteur(s)

Mu Li (Huawei); limu17@huawei.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

Stage 3 aspects in CT1, CT3 and CT4.

9	Supporting Individual Members

Supporting IM name

China Mobile

Ericsson

Huawei

HiSilicon

Nokia

Nokia Shanghai Bell

China Telecom

Deutsche Telekom

China Unicom