3GPP TSG-SA WG2 Meeting #152E (e-meeting)	S2-2206391

# August 17 – 26, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 –Supporting PCF to make AM policies decision based on UE’s accumulated usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_DCAMP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Policy decision based on usage monitoring control requirement is a function that allows PCF taking actions related to the accumulated usage of network resource. In Rel-17, the specification only defines usage monitoring for policy control requirement when PCF for UE and PCF for PDU Session are the same PCF instance, and does not elaborate on how PCF for UE could retrieve usage monitoring report related to AM policies when PCF for PDU session and PCF for UE are different PCF instances. More details can be seen in DP S2-2206343. The related CR can be seen in S2-2206403. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding 4.16.X to elaborate the procedure of how PCF for a UE obtain the accumulated usage and make policy decision based on accumulated usage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of dynamically changing AM policies based on accumulated usage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.503 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * All New Text * * * *

4.16.X	Management of access and mobility related policy information depending on accumulated usage

The procedure for management of access and mobility related policy information depending on usage monitoring enables modification of the access and mobility related policy information on accumulated usage.

The content of this clause applies to non-roaming i.e. to cases where the involved entities (e.g. PCF, SMF and UPF) belong to the home PLMN. For this case, the procedure for management of aceess and mobility related policy information in clause 4.16.14.2 can be reused with the following change.

-	In step 3, the PCF for the UE determines that access and mobility related policy information (e.g. RFSP index value) depends on the report of usage monitoring, then subscribes to the BSF to be notified when a PCF for the PDU Session for this SUPI is registered in the BSF by invoking Nbsf_Management_Subscribe (SUPI).

-	In step 6, the PCF for the UE subscribes to notifications of event " reporting accumulated usage for AM policies decision " as defined in clause 6.1.3.18 in TS 23.503 [20], using Npcf_PolicyAuthorization_Subscribe (UE address, EventId, EventFilter set to "list of UE Identifiers") to the PCF for the PDU Session to the DNN, S-NSSAI.

-	In step 7, the PCF installs PCC Rules and the Policy Control Request Trigger to detect "usage monitoring report" in the SMF.

-	In step 8, the SMF detects that the Policy Control Request Trigger is met, then reports the accumulated usage to the PCF serving the PDU Session.

-	In step 9, EventInformation including the Usage Monitoring Control related information.

* * * * End of Changes * * * *