SA WG2 Meeting #152e	S2-2206405

# August 17th – 26th, 2022; Elbonia               		(revision of)

Source: 	China Telecom

Title: 	Update to Solution#6 on FS_5WWC_Ph2

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: Updating solution#6 to resolve the ENs

1	Discussion

To resolve the following ENs in solution 6, this contribution updates the solution by adding the following:

Adding a NOTE to indicate that this solution can solve the differentiated aspect for both static and randomized MAC address cases, but can solve the charging aspect only for static MAC address case.

Updating the authentication part of the registration procedure.

Updating the details of how to provide differentiated QoS to non-3GPP device.

2 Proposal

It is proposed to update TR 23.700-17 on FS_5WWC_Ph2 as follows

*** Start of the change (all new text) ***

## 6.6	Solution 6: New method for non-3GPP device connected behind a 5G-RG

### 6.6.1	Description

This solution addresses Key Issue #1"Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG".

Figure 6.6.1-1 depicts the architecture for devices connected behind a 5G-RG. In this solution, the AF, which supports the functionality of assigning virtual identifier (e.g. virtual SUCI) can preconfigure a list of the virtual identifier to the UDM through NEF. After the 5G-RG registers to the 5GC, a virtual identifier would be assigned to it as the identifier of the device which has been connected to 5G-RG. Then the 5G-RG could use the virtual identifier to initial registration procedure and PDU session establishment procedure via W-AGF on behalf of the device. In this solution, the 5G-RG receives QoS rules, which is associated with the virtual identifier (e.g. virtual SUCI), for the non-3GPP devices and different QoS profiles may be applied per device behind the 5G-RG by 5GC.

NOTE 1:	AF in figure 6.6.1-1 is managed by operator.

NOTE 2:	The interaction between the AF and 5G-RG may be out of 3GPP scope.

NOTE 3:	The virtual identifier corresponds to 5G-RG's subscription information in UDM.

NOTE 4: For the differentiated QoS aspect, this solution can be applied to both static and randomized MAC address cases. But for the charge aspect, this solution can only be applied to static MAC address case.

Figure 6.6.1-1: architecture for non-3GPP devices connected behind a 5G-RG

### 6.6.2	Procedures

Figure 6.6.2-1: Procedure for 5G RG and Device Registration

0.	The AF preconfigures the list of virtual identifier (e.g. virtual SUPI) to UDM through the NEF, so that the virtual identifier can be a legal identifier of the non-3GPP device behind the 5G-RG for which the 5G-RG is doing the authentication procedure on behalf.

1.	The 5G-RG registration procedure, as descibed in clause 7.2.1.1 of TS 23.316 [5].

2.	The UDM informs the AF that the 5G-RG has been successfully registered with 5GC. AF should save a list of SUCI of registered 5G-RGs.

3.	The device connects to the 5G-RG via a layer-2 (L2) connection.

4.	The 5G-RG applies to the AF for a virtual identifier by carrying the SUCI of the 5G RG and the MAC address of the device.

5.	The AF checks if the 5G-RG is in the list of registered 5G-RGs. If it has registered successfully, the AF will store the non-3GPP device information in UDR as Application Data, as currently defined for Nnef_ServiceParameter service in TS 23.502 [3], clause 4.15.6.7. The information of the non-3GPP device includes the mapping information between the virtual identifier (e.g. the virtual SUCI) and the MAC address of the non-3GPP device, the identifier of 5G-RG, etc.

6.	If the 5G-RG is in the list of registered 5G-RGs, the AF responds to the 5G-RG by assigning a virtual identifier (e.g. the virtual SUCI) to the device. The AF saves the timestamp and the mapping information between the virtual identifier (e.g. the virtual SUCI) and the MAC address of the device.

NOTE:	In steps 4 and 6, the AF and 5G-RG communicate directly without going through the 5GC. Step 4 and 6 may be out of the 3GPP scope.

7.	The 5G-RG uses the virtual identifier to initial registration procedure on behalf of the device. Registration peocedure specified in clause 7.2.1.1 of TS 23.316 [5] apply with the following differences:

The 5G-RG initial registration procedure by the transmission of a NAS message containing a registration request. The registration request contains e.g. the virtual identifier (e.g. virtual SUCI). Because the NAS tunnel has already been established in step 1, the 5G-RG can send a NAS message to AMF directly.

The virtual identify (e.g. virtral SUCI) is used in the authentication procedure to authenticate the device behind the 5G-RG.

8.	When 5G-RG establish a PDU Session, the PCF may contact the UDR to subscribe to the notification on the update of Application Data, as per existing procedure for Service specific parameter provisioning in TS 23.502 [3], clause 4.15.6.7. The PCF thus receives the non-3GPP device information from UDR corresponding to the RG’s SUPI and can take the information into account for its policy decision. ,

The PCF provides QoS rules for the non-3GPP device to the SMF. The 5G-RG receives QoS rules for the non-3GPP devices and the 5G-RG maps the traffic of the non-3GPP device to the QoS flow of this PDU session according to the received QoS rule.

### 6.6.3	Impacts on Existing Nodes and Functionality

UDM:

-	Inform AF that a 5G-RG has been successfully registered.

-	Store the mapping between the virtual identifier and the MAC Address of the non-3GPP device.

5G-RG:

-	Apply to the AF for a virtual identifier.

-	Store the mapping information between the virtual identifier and the MAC address of the non-3GPP device.

-	Support performing registration and PDU Session related procedure on behalf of non-3GPP device behind.

-	Send a NAS message containing a registration request to the AMF.

### *** End of the change ***