3GPP SA WG2 Meeting #152E	S2-2206433

E-meeting, 17-26 Aug 2022

Source:	CATT

Title:	KI #2, Sol #7: Update to clarify time synchronization service based temporal validity condition

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper is to update solution #7 to clarify time synchronization service based temporal validity condition.

1. Discussion

In clause 6.7.3, it states:

“If the UE location is out of Requested Coverage Area, the TSCTSF creates a PTP port in a DS-TT and assigns it into the PTP instance but temporarily removes the UE/DS-TT from the PTP instance and indicates the PTP port state as Inactive for the related DS-TT PTP port.”

It implies that even if the UE is always outside the Requested Coverage Area, the TSCTSF has to create a PTP port in a DS-TT, which leads to some unnecessary resources are being consumed.

In Rel-17, it specified a parameter Temporal Validity Condition, which indicates the time period when the time synchronization service for a PTP instance is active. However, in this solution, it is unclear how to support activation/deactivation of time synchronization services when AF request includes Both of Spatial Validity Condition and Temporal Validity Condition parameters.

To support the backwards compatibility with Temporal Validity Condition, the TSCTSF needs to determine whether activate/deactivate time synchronization services based on combining conditions (i.e. Both of Spatial Validity Condition and Temporal Validity Condition). For example, based on whether Spatial Validity Condition and Temporal Validity Condition are satisfied, there could be some possible scenarios:

- If the current time is within Temporal Validity Condition, and the UE location is out of Requested Coverage Area, the TSCTSF determines not to activate time synchronization services;

- If the current time is within Temporal Validity Condition, and the UE location is in Requested Coverage Area, the TSCTSF determines to activate time synchronization services and create a PTP port in a DS-TT.

- If the current time is within Temporal Validity Condition, and the UE moves into Coverage Area, the TSCTSF determines to activate time synchronization services and create a PTP port in a DS-TT.

- If the current time is within Temporal Validity Condition, and the UE moves out of Requested Coverage Area, the TSCTSF determines to temporarily remove the UE/DS-TT from the PTP instance;

- If the current time is NOT within Temporal Validity Condition, and no matter the UE location is IN or OUT Requested Coverage Area, the TSCTSF determines deactivate time synchronization services.

It is proposed to update the solution based on above discussion.

2. Proposal

It is proposed to agree the following texts in clause 6.7 of TR 23.700-25.

>>>> Start of Changes <<<<

6.7	Solution #7: RequestedCoverage area filters for time synchronization service

6.7.1	Introduction

This solution is proposed to solve Key Issue #2: Time synchronization service enhancements. In this Key Issue, the coverage area is taken into account for the configuration of the time synchronization service. Therefore, this solution addresses the time synchronization service scenarios already supported in 5G Release-17 based on access stratum or (g)PTP time distribution methods.

This solution proposes to use a list of Tracking Areas (TAs) identified by Tracking Area Identities (TAI) or a list of Cells to describe a specific geographical area (a so-called Requested Coverage Area) where an AF requests to enable a time synchronization (TS) service. Furthermore, in order to enforce that the requested TS service is enabled for UEs (a specific UE or a group of UEs) only in that Requested Coverage Area, the proposed solution exploits the 5GS functionality (provided by the AMF) of tracking and reporting "UE mobility on Area of Interest" events (clause 5.3.4.4 of TS 23.501 [2]). An Area of Interest (AoI) for each AMF is represented by a list of TA(s) or by a list of Cells, wherein the Area of Interest is identical to the Requested Coverage Area or the Area of Interest is a TA or cell subset of the Requested Coverage Area.

6.7.2	Functional Description

The existing time synchronization exposure procedures (clause 4.15.9 of TS 23.502 [3]) are enhanced.

The following principles are proposed to enable the use of coverage area for time synchronization service operation:

-	An AF may optionally include a Requested Coverage Area within the timing synchronization service request using a spatial validity condition.

-	As a spatial validity condition, the AF may use a geographical area (e.g. a civic address or shapes) or a TA/Cell list (if it is an AF within the operator's domain, it will have TA(s) or Cell(s) configured):

-	If the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into 3GPP identifiers (e.g. TAI(s) or Cell(s)) based on pre-configuration. Later, the TSCTSF discovers the AMF(s) serving the TAs/Cells comprising Area(s) of Interest. The TSCTSF or AMF subscriptions use Area of Interest that may be the same as the Requested Coverage Area or may be a subset of TAs/Cells from the list of TAs/Cells describing the Requested Coverage Area.

-	If the AF is within the operator's domain, it uses a list of TAs or a list of Cells directly (based on pre-configuration) to formulate the spatial validity condition for the Requested Coverage Area.

-	TSCTSF uses Location or UE presence in Area of Interest services at the AMF Event Exposure service (Namf_EventExposure operations) to identify the UE(s) to which the AF request with a Requested Coverage Area applies. The TSCTSF subscribes to this information at the AMF(s), stores it, and determines the UE(s) presence in the Requested Coverage Area using the notification(s) from one or multiple AMFs.

-	Requested Coverage Area will be used at the TSCTSF for triggering time synchronization service activation/modification/deactivation with the following differences:

1)	the TSCTSF discovers the AMF(s) serving in the Tracking Areas (TAs) comprising the Requested Coverage Area;

2)	the TSCTSF subscribes (using the Namf_EventExposure_Subscribe service operation specified in clause 5.2.2.3.2 of TS 23.502 [3]) to the UE mobility on Area of Interest event notification service from the AMF(s) to be notified the UE(s) presence in an Area of Interest for each AMF (e.g. UE location is IN or OUT an Area of Interest);

3)	the TSCTSF determines (based on the notification from the AMF(s)) whether or not the targeted UE(s) are inside the Requested Coverage Area; and

4)	the TSCTSF proceeds with the time synchronization service activation/deactivation only after the TSCTSF has determined which of the targeted UE(s) are in or out of the Requested Coverage Area. The subscription to reports of UE presence in the Area of Interest procedure is described in clause 5.3.4.4 of TS 23.501 [2].

-	For access stratum distribution activation/deactivation, the TSCTSF will enable/disable ReferenceTimeInformation delivery to the UE at the serving NG-RAN node reusing the procedure in clause 4.15.9.4 of TS 23.502 [3]. For (g)PTP distribution activation/deactivation, the TSCTSF will modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TT and UMIC to the UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

6.7.3	Procedures

6.7.3.1	Procedure for AF requested (g)PTP timing synchronization with Requested Coverage Area

An overall procedure for AF providing coverage area  for (g)PTP based time synchronization service is illustrated in Figure 6.7.3.1-1.

Figure 6.7.3.1-1: (g)PTP time distribution configuration with Requested Coverage Area

1.	The AF creates a time synchronization service configuration for a PTP instance by invoking Nnef_TimeSynchronization_ConfigCreate service operation. The request includes the parameters as described in table 4.15.9.3.1 in TS 23.502 [3] and optionally a spatial validity condition (i.e. Requested Coverage Area) in the format of a geographical area (e.g. a civic address or shapes) a list of TA(s), or a list of Cells.

2.	The NEF authorizes the request. If the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into a list of TA(s) or a list of Cells based on pre-configuration. The list of TA(s) or the list of Cells describes the Requested Coverage Area.

The AF within the operator's domain shall use a list of TA(s) or a list of Cells directly to formulate a spatial validity condition describing the Requested Coverage Area.

3.	After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF, with the parameters as received from the AF.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

4.	TSCTSF determines whether the TSCTSF has subscribed for the UE presence for an area that is indicated in the TAs or Cell IDs in the spatial validity condition. If not, the TSCTSF discovers the AMF(s), serving in the TAs or serving the Cells that either comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the list of TAs or the list of Cells, respectively.

5.	The spatial validity condition  for the UE(s) is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF(s) to receive notifications about the UE presence in an area of interest (or UE location) using Namf_EventExposure operation. The subscribed area of interest may be the same as the Requested Coverage Area specified by the spatial validity condition  or may be a subset of the Requested Coverage Area (e.g. a list of TAs, or cell list) based on the latest known UE location.

6-7. The AMF(s) track the UE's location to determine the UE's presence in an Area of Interest as described in clause 4.15.4.2 of TS 23.502 [3]. Further, the AMF(s) notify the TSCTSF about the UE(s) presence (IN, OUT, or UNKNOWN) in the Area of Interest(s).

8. The AMF(s) accept TSCTSF's subscription and provide the first corresponding event report (if available).

9.	According to the UE location of each UE that is targeted by the request and spatial validity condition in step 1, the TSCTSF determines whether to active time synchronization service for this UE:

-	If the UE location is in an Area of Interest (and thus in the Requested Coverage Area), the TSCTSF determines to active time synchronization service and creates a PTP port in a DS-TT and assigns it into the PTP instance.

-	If the UE location is out of Requested Coverage Area, the TSCTSF determines not to active time synchronization service and not to create a PTP port in a DS-TT..

10.	The TSCTSF responds with the Ntsctsf_TimeSynchronization_ConfigCreate response.

11.	The NEF responds with the Nnef_TimeSynchronization_ConfigCreate response.

12.	The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT.

13.	The TSCTSF uses the procedure in clause 4.15.9.4 of TS 23.502 [3] to manage the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

14.	UE moves in/out of the determined area of interest (i.e. Requested Coverage Area).

15.	NG-RAN determines the UE presence in the area of interest has changed and notifies the AMF.

16.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

17.	If the TSCTSF receives the UE location change notification for AoI. The TSCTSF may update to activate or deactivate the time synchronization service for the given UE.

-	If the UE moves out of the Requested Coverage Area, the TSCTSF temporarily removes the UE/DS-TT from the PTP instance and indicates the PTP port state as Inactive for the related DS-TT PTP port.

-	If the UE moves into an Area of Interest, and according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period), the TSCTSF determines to activate time synchronization service and creates a PTP port in a DS-TT and assigns it into the PTP instance.

-	The TSCTSF keeps the requested TS service unchanged (i.e. active/inactive) when the UE presence in the Requested Coverage Area becomes UNKNOWN until the TSCSTF determines that the UE is inside/outside the Requested Coverage Area.

18.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation. The notification indicates the identities of the UEs currently within the area of spatial validity.

19.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation.

6.7.3.2	Procedure for AF requested 5G access stratum timing synchronization with Requested Coverage Area

An overall procedure for AF requesting a specific coverage area for 5G access stratum based time synchronization service (see TS 23.502 [3]) is illustrated in Figure 6.7.3.2-1.

Figure 6.7.3.2-1: 5G access stratum time distribution configuration with Requested Coverage Area

1.	The AF request to influence the 5G access stratum time distribution providing access stratum time distribution parameters to the NEF (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1 of TS 23.502 [3]) and optionally a spatial validity condition (i.e. Requested Coverage Area) in the format of a geographical area (e.g. a civic address or shapes), a list of TA(s), or a list of Cells.

2.	The NEF authorizes the request. If the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into a list of TA(s) or a list of Cells based on pre-configuration. The list of TA(s) or the list of Cells describes the Requested Coverage Area.

The AF within the operator's domain shall use a list of TA(s) or a list of Cells directly to formulate a spatial validity condition describing the Requested Coverage Area.

3.	After successful authorization, the NEF invokes the Ntsctsf_ASTI_Create service operation with the corresponding TSCTSF.

4.	TSCTSF determines whether the TSCTSF has subscribed for the UE presence for an area that is indicated in the TAs or Cell IDs in the spatial validity condition. If not, the TSCTSF discovers the AMF(s), serving in the TAs or serving the Cell(s) that either comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the list of TAs or the list of Cells, respectively.

5.	The spatial validity condition is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF to receive notifications about change of UE location in an area of interest using Namf_EventExposure operation. The subscribed area of interest may be the same as the Requested Coverage Area specified by the spatial validity condition or may be a subset of the Requested Coverage Area (e.g. a list of TAs, a list of Cells) based on the latest known UE location.

6-7.The AMF(s) track the UE's location to determine the UE's presence in an Area of Interest as described in clause 4.15.4.2 of TS 23.502 [3]. Further, the AMF(s) notify the TSCTSF about the UE(s) presence (IN, OUT, or UNKNOWN) in the Area of Interest(s).

8.	The AMF(s) accept TSCTSF's subscription and provide the first corresponding event report (if available).9.	According to the UE location and spatial validity condition, the TSCTSF determines whether to activate time synchronization service:

-	If the UE location is in an Area of Interest (and thus in the Requested Coverage Area), the TSCTSF determines to activate time synchronization service.

-	If the UE location is out of Requested Coverage Area, the TSCTSF stores the UE identity and determines not to activate ASTI time synchronization service for the UE.

10.	The TSCTSF responds the AF with the Ntsctsf_ASTI_Create service operation response.

11.	The NEF informs the AF about the result of the Nnef_ASTI_Create service operation performed in step 1.

12.	The TSCTSF uses the procedures described in clause 4.15.9.4 of TS 23.502 [3] to configure ReferenceTimeInformation delivery to the UE(s) at the serving NG-RAN nodes.

13.	UE moves in/out of the configured Area(s) of Interest.

14.	NG-RAN determines the change of the UE presence in the area of interest has changed and notifies the AMF.

15.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

16.	If the TSCTSF receives the UE location change notification for AoI. The TSCTSF may update to activate or deactivate the time synchronization service.

-	If the UE moves out of the Requested Coverage Area, the TSCTSF determines to deactivate ASTI time synchronization service as described in clause 4.15.9.4 of TS 23.502 [3].

-	If the UE moves into an Area of Interest, and according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period), the TSCTSF determines to activate ASTI time synchronization service as described in clause 4.15.9.4 of TS 23.502 [3].

-	The TSCTSF keeps the requested TS service unchanged (i.e. active/inactive) when the UE presence in the Requested Coverage Area becomes UNKNOWN until the TSCSTF determines that the UE is inside/outside the Requested Coverage Area.

17.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_ASTI_UpdateNotify service operation for the UE identities that are inside or outside of the area in the spatial validity condition.

18.	The NEF notifies the AF with the Nnef_ASTI_UpdateNotify service operation.

6.7.4	Impacts on services, entities and interfaces

TSCTSF:

-	Subscribe for receiving UE's presence in Area of Interest from AMF.

-	Determination of area of interest based on list of TAs or Cells.

-	Support for time synchronization service configuration conditioned to the AF-Requested Coverage Area using a spatial validity condition.

-	Discovers the related AMF(s) using the NRF's service operation Nnrf_NFDiscovery_Request.

-	Notifies the AF/NEF via a new Ntsctsf_ASTI_UpdateNotify service operation for the UE identities that are inside or outside of the area in the spatial validity condition.

-	Determines whether activate/deactivate time synchronization services based on combining conditions (Spatial Validity Condition and Temporal Validity Condition).

AMF:

-	Tracks a UE's location to determine the UE's presence in an Area of Interest using the existing event reporting type, i.e. UE mobility on Area of Interest.

-	Notifies the subscribed TSCTSF about the change of UE's status (IN, OUT, or UNKNOWN) in the Area of Interest using the (existing) Namf_EventExposure_Notify service operation.

AF:

-	Includes the Spatial Validity Condition in AF request for time synchronization service configuration.

>>>> End of Changes <<<<