SA WG2 Meeting #S2-152E	S2-2206437

17 – 26 August, 2022, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	evaluation and interim conclusions for key issue 3

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This paper proposes evaluation and interim conclusions for key issue 3

# 2		Proposal

It is proposed to include the following changes in TR 23.700-81.

*** 1st Change (all new)***

# 7	Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.X Key issue 3

Table 7.X-1: Comparison of solutions for KI#3 RRC_Inactive mode reception

| Sol | Use Cases | Main Principles | Protection of Exposed data | Main Impacts | Open Issues |
| --- | --- | --- | --- | --- | --- |
| 10 | Exposure of both data and analytics from VPLMN to HPLMN and from HPLMN to VPLMN | A new Gateway Exposure Function (GEF) is used as exit and entry point for request for data or analytics from. | GEF is also responsible for data/analytics manipulation before sending to the other PLMN according to the regulatory constraints and operator policies | new Gateway Exposure Function (GEF) - Send / receive inquiries or subscriptions for data or analytics to / from peer PLMN - data/analytics manipulation before sending to the other PLMN according to the regulatory constraints and operator policies | It is ffs if the GEF is a standalone function or collocated with either NEF,DCCF or NWDAF. It is ffs whether exit GEF is required. It Is FFS to which extent the VPLMN can provide meaningful analytics for a new inbound roamer without historical data about the roaming user and with only limited access to input data sources in the HPLMN. |
| 11 | Analytics of HPLMN (Slice load level, NF load, etc.) can be sent to the VPLMN and used by the AMF in VPLMN for Network Slice and H-SMF selection for Home Routed PDU Session establishment. | AMF contacts H-NWDAF via V-NWDAF | ? | V-NWDAF - Interact with H-NWDAF | No considerations how to restrict exposed analytics |
| 17 | Analytics in VPLMN (i.e. Service Experience for an application and/or Network Slice, Network Slice load level, etc.) can be sent to the HPLMN and used by the H-PCF for decision of network slice selection policies within the URSP rule. | The H-PCF requests analytics via H-NWDAF. The H-NWDAF maps slice info to VPLMN and interacts with V-NWDAF | ? | PCF: - request analytics - updates the NSSP (as part of the URSP rules) to the UE for the UE using the PDU Session in the indicated Network Slice(s) to transmit the application in roaming scenarios. H-NWDAF: - requests VPLMN analytics information from the V-NWDAF. V-NWDAF: - provides VPLMN analytics information to the H-NWDAF | No considerations how to restrict exposed analytics FS_UEPO, key issue 1, aims to enable the provisioning of URSP rules in the VPLMN and may thus remove the need for the H-PCF to determine NSSP |
| 37 | No raw data but only analytics exchange allowed between HPLMN and VPLMN (in both directions) to reduce traffic. Example use case end-to-end service experience analytics | H-NWDAF and V-NWDAF act as entry points for related requests. H-NWDAF can derive overall analytics with V-NWDAF analytics as input, e.g. for end-to-end service experience analytics | Seem to assumes that exchanged analytics do not need to be filtered and restricted | H-NWDAF: - subscribe to VPLMN service experience analytics at V-NWDAF; - derive the E2E service experience analytics based on the service experience analytics in the visited network and the service experience analytics in the home network. V-NWDAF: - Provide service experience analytics for VPLMN to H-NWDAF; | Unclear how the user consent is checked and regulatory constraints and operator policies are considered. It Is FFS to which extent the VPLMN can provide meaningful analytics for a new inbound roamer without historical data about the roaming user and with only limited access to input data sources in the HPLMN. It Is FFS whether raw data aggregation can be an alternative to analytics to reduce signaling load |
| 38 | HPLMN may collect data from VPLMN. VPLMN may consume analytics profiles generated by HPLMN | NWDAF serves as entry point for related requests. Analytics profiles are generated for each UE in advance in HPLMN. | NWDAF serves as entry point for related requests and authorizes requests and restricts exposed data and analytical information. | V-NWDAF: - Offer new data retrieval services that enable data retrieval by NFs from another PLMN: - Retrieve analytics profile for UE from H-NWDAF and extract and provide analytics based on it. H-NWDAF - Offer new analytics retrieval services that enable analytics profile retrieval by VPLMN. - Generate an "UE analytics profile" for each UE subscribed at HPLMN and store it in ADRF. NRF: - Restrict access to other event exposure services and analytics services from outside the own PLMN and redirect to NWDAF data retrieval service at analytics retrieval service ADRF - Store "UE analytics profiles" |  |
| 39 | H-NWDAF retrieves analytics or data for roaming users | H-NWDAF retrieves analytics or data for roaming users from V-NWDAF via H-DCCFs and V-DCCF. | V-DCCF checks whether user VPLMN is authorized to request the data or analytics | Consumer in HPLMN (e.g. H- NWDAF): - subscribes to data/analytics information at the H-DCCF H-DCCF: - discover the V-DCCF from NRF; - requests data or analytics from V-DCCF V-DCCF: - checks if the consumer from HPLMN is authorized to obtain the data/analytics from the VPLMN - collects data/ analytics from data provider(s) for the roaming UE(s) - sends the collected data/analytics to the VPLMN DCCF. | It is FFS whether a new DCCF service may be appropriate to trigger the special checking by the VPLMN DCCF. It is FFS how the V-DCCF obtains applicable policies for the peer PLMN. |
| 40 | Exposure of both data and analytics from VPLMN to HPLMN and from HPLMN to VPLMN | NWDAF in HPLMN/VPLMN accesses the NFs or NWDAF in the other PLMN via NEF for data collection or analytics subscription and behaves as an untrusted AF for the other PLMN. The NEF maps S-NSSAI(s) of VPLMN and HPLMN against each other. | The NEF verifies user consent, operator policy, regulatory constraints and/or roaming | Consumer NF: - include indication for roaming UE and the PLMN ID NWDAF: - Support translation of S-NSSAI(s) NEF: - translation of S-NSSAI(s) - check user consent, operator policy, regulatory constraints and/or roaming | It is FFS whether NEF in VPLMN has access to user consent. It is FFS how NEF realizes that checks of operator policy, regulatory constraints and/or roaming agreements are required. It is FFS if NEF can expose the input data for all types of analytics or extensions are required. It Is FFS to which extent the VPLMN can provide meaningful analytics for a new inbound roamer without historical data about the roaming user and with only limited access to input data sources in the HPLMN. |

*** 2nd Change (all new)***

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Key issue 3

### 8.X.1	Interim Conclusions

The following use cases are supported:

-	HPLMN may collect data from VPLMN.

-	VPLMN may consume analytics profiles or analytics generated by HPLMN

-	Analytics of HPLMN (Slice load level, NF load, etc.) can be sent to the VPLMN and used by the AMF in VPLMN for Network Slice and H-SMF selection for Home Routed PDU Session establishment

A central entry point is used to handle incoming request for raw data or analytics. It authorizes the request according to operator policies and user consent and it filters the information exposed in reply to the request