SA WG2 Meeting #152E eMeeting	S2-2206459

Elbonia, Aug. 17 – 26, 2022 	(revision of S2-20xxxxx)

Source:	OPPO, Toyota, Oracle

Title:	TR 23.700-80: FS_AIMLsys KI#1 Evaluations and Proposed Interim Conclusions

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution is to present the evaluation for KI#1 and to propose the interim conclusions.

# Background

The intent of this PCR is to present the evaluation for the set of solutions for KI#1 and to propose an interim conclusions.

# Discussions

There are 6 solutions which have been proposed to address KI#1, i.e. Solution#1, Solution#11, Solution#16, Solution#28, Solution#34 and Solution#37.  Among these solutions, Solution#16 will be merged into Solution#37, whereas Solution#28 is proposed as the overall architecture framework and will only be further considered when all the KIs are getting close to be concluded.  Hence, in reality there are really 4 solutions to be considered at this point for KI#1, i.e. Solutions#1, 11, 34 and 37.

| Evaluation Criteria | Solution#1 | Solution#11 | Solution#34 | Solution#37 |
| --- | --- | --- | --- | --- |
| Suggest to leverage and/or extending the existing NWDAF analytics to assist AF to monitor the UE’s data transmission performance, i.e. impact to NWDAF | - Proposed to extend existing NWDAF analytic mechanisms to monitor DN performance, UE communication & QoS sustainability - The monitoring is done in order to prepare for the upcoming Application AI/ML transmission | - Proposed to leverage the AF Influence for traffic routing feature to support the transmission of the Application AI/ML traffic which also triggers “some” NWDAF analytics to monitoring the UE’s performance to assist the traffic routing | - Proposed to leverage the NWDAF analytic to monitor the existing Application AI/ML data transfer and to report the ongoing performance to the AF | - No |
| Suggest to leverage other mechanism besides NWDAF to obtain UE’s performance, i.e. no NWDAF impact | - Proposed to leverage PC, SMF and UDF etc. to monitor UL/DL & round trip delay, traffic/data volume, session inactivity timer, | - No | - No | - Proposed to extend the existing UPF event exposure with new event to report the UE’s on-going bit rate periodically to NEF. NEF will then aggregate the group of UEs bit rate and compare it against the Group-MBS threshold to ensure the group performance does not exceed the aggregated throughput performance. If exceeded, NEF will notify the AF for this new event. |
| Support aggregated bit rate monitoring to assist AF to conduct the Application AI/ML operation according to the SLA NOTE: Corresponding to Principle#3 which receives general support during the Moderated AP Discussions | - No | - No | - No | - Yes |
| Support aggregated performance monitoring reporting for optimization (e.g. minimizing signalling overhead). | - No | - No | - No | - Yes |
| Support QoS monitoring for AI/ML-based services triggered by AF to assist AF to conduct the Application AI/ML operation according to the SLA. NOTE: Corresponding to Principle#4 which receives general support during the Moderated AP Discussions | - Yes | - Yes (Built-in to the AF Influence to Traffic Routing mechanism) | - Yes | - Partially (focus on bit rate monitoring) |
| Both the network and the AF have the need to monitor usage of network resources for AI/ML training traffic. NOTE: Corresponding to Principle#7 which receives general support during the Moderated AP Discussions | - Yes | - Yes | - Yes | - Yes |
| Assist AF to perform QoS policy modification on the UE after monitoring the UE’s performance NOTE: Not specific to this KI | - Trigger AF session request to update UE’s QoS configuration | - Leverage the imbedded procedure of the AF Influence to modify the QoS policy when needed | - Not specified | - Not specified |
| Applicable to FL “aggregated” performance monitoring | - No | - No | - No | - Yes |
| Proposed mechanism (e.g. gating, QoS policy modification, etc.) to be used by AF or by 5GC to respond to the UE’s performance monitoring NOTE: Not specific to this KI | - No | - No | - No | - No |
| Pros: | - Able to leverage the existing NWDAF analytics to adjust the QoS policy and to assist the AF’s decision (e.g. UE selection) before starting the application AI/ML operation with the proper UE. | - Able to reuse existing AF Influence mechanism to monitoring the on-going Application AI/ML operation and to adjust the QoS policy to adapt to the UE’s changing condition | - Able to leverage the existing NWDAF analytics to assist the AF to continue monitoring the UE, QoS etc, performance during the application AI/ML operation. | - Able to reuse the existing UPF exposure mechanism to monitor the bit rate for the selected group of UEs to support aggregated monitoring request from AF. - The monitoring can be applied before or during the Application AI/ML operation - The mechanism can support FL operation for aggregated bit rate monitoring with minimum signaling overhead between the 5GC and AF |
| Cons: | - Not always efficient as the monitoring is mainly done on per UE basis to report to the AF, i.e. signaling overheads could be high - Not clear if the monitoring is on-going while the Application AI/ML operation was started. This is especially important in case of the FL operation. - No all performance/resource monitoring are necessarily done by the NWDAF | - No assistance to the AF on learning the UE’s performance before starting the Application AI/ML operation. - Not always efficient as the monitoring is mainly done on per UE basis to collect User Experience and/or DN performance from the AF, i.e. signaling overheads could be high - No all performance/resource monitoring are necessarily produced by the NWDAF - There is no on-going feedback from the 5GC to the AF to monitor each UE’s performance for participating in the Application AI/ML operation | - No assistance to the AF on learning the UE’s performance before starting the Application AI/ML operation. - Not always efficient as the monitoring is mainly done on per UE basis to report to the AF, i.e. signaling overheads could be high - No all performance/resource monitoring are necessarily done by the NWDAF | - Limited to the aggregated bit rate monitoring only |
| Propose Interim Conclusions for KI#1: | - Propose to merge Solution#1, Solution#11, Solution#34 which leverage NWDAF analytics to perform some performance/resource monitoring prior to the start of the Application AI/ML operation as follows: - Refer to the procedure in Solution#1 to perform the pre application AI/ML operation monitoring and provide the feedback to the AF - In addition, if any of the NWDAF analytics that were described in Solution#11 and Solution#34 which are useful and applicable to assist the AF’s operation decision to prepare for the start of the application AI/ML operation, such NWDAF analytics should also be included as the pre application AI/ML operation monitoring. - Propose to extend the AF Influence include NWDAF analytics proposals from Solution#11 and Solution#34, if applicable - Extend the AF Influence procedures as described in Solution#11 to include the support of the additional NWDAF analytics (could be with extension) as described in Solution#1 and Solution#34, if applicable - Propose to include the Group-MBR monitoring procedures as described in Solution#37 for pre Application AI/ML data transfer operation and during the Application AI/ML data transfer operation in order to provide the on-going monitoring. NOTE: Further enhancement to optimize the overall system performance for these solutions’ merger may be needed during the normative phase of this study. |  |  |  |

# Proposals

# ***** Start of Changes *****

# 7 	Evaluation

# 7.X	Key Issue#1

The following table summarize the similarity and differences among the 4 solutions to address KI#1

| Evaluation Criteria | Solution#1 | Solution#11 | Solution#34 | Solution#37 |
| --- | --- | --- | --- | --- |
| Suggest to leverage and/or extending the existing NWDAF analytics to assist AF to monitor the UE’s data transmission performance, i.e. impact to NWDAF | - Proposed to extend existing NWDAF analytic mechanisms to monitor DN performance, UE communication & QoS sustainability - The monitoring is done in order to prepare for the upcoming Application AI/ML transmission | - Proposed to leverage the AF Influence for traffic routing feature to support the transmission of the Application AI/ML traffic which also triggers “some” NWDAF analytics to monitoring the UE’s performance to assist the traffic routing | - Proposed to leverage the NWDAF analytic to monitor the existing Application AI/ML data transfer and to report the ongoing performance to the AF | - No |
| Suggest to leverage other mechanism besides NWDAF to obtain UE’s performance, i.e. no NWDAF impact | - Proposed to leverage PC, SMF and UDF etc. to monitor UL/DL & round trip delay, traffic/data volume, session inactivity timer, | - No | - No | - Proposed to extend the existing UPF event exposure with new event to report the UE’s on-going bit rate periodically to NEF. NEF will then aggregate the group of UEs bit rate and compare it against the Group-MBS threshold to ensure the group performance does not exceed the aggregated throughput performance. If exceeded, NEF will notify the AF for this new event. |
| Support aggregated bit rate monitoring to assist AF to conduct the Application AI/ML operation according to the SLA NOTE: Corresponding to Principle#3 which receives general support during the Moderated AP Discussions | - No | - No | - No | - Yes |
| Support aggregated performance monitoring reporting for optimization (e.g. minimizing signalling overhead). | - No | - No | - No | - Yes |
| Support QoS monitoring for AI/ML-based services triggered by AF to assist AF to conduct the Application AI/ML operation according to the SLA. NOTE: Corresponding to Principle#4 which receives general support during the Moderated AP Discussions | - Yes | - Yes (Built-in to the AF Influence to Traffic Routing mechanism) | - Yes | - Partially (focus on bit rate monitoring) |
| Both the network and the AF have the need to monitor usage of network resources for AI/ML training traffic. NOTE: Corresponding to Principle#7 which receives general support during the Moderated AP Discussions | - Yes | - Yes | - Yes | - Yes |
| Assist AF to perform QoS policy modification on the UE after monitoring the UE’s performance NOTE: Not specific to this KI | - Trigger AF session request to update UE’s QoS configuration | - Leverage the imbedded procedure of the AF Influence to modify the QoS policy when needed | - Not specified | - Not specified |
| Applicable to FL “aggregated” performance monitoring | - No | - No | - No | - Yes |
| Proposed mechanism (e.g. gating, QoS policy modification, etc.) to be used by AF or by 5GC to respond to the UE’s performance monitoring NOTE: Not specific to this KI | - No | - No | - No | - No |
| Pros: | - Able to leverage the existing NWDAF analytics to adjust the QoS policy and to assist the AF’s decision (e.g. UE selection) before starting the application AI/ML operation with the proper UE. | - Able to reuse existing AF Influence mechanism to monitoring the on-going Application AI/ML operation and to adjust the QoS policy to adapt to the UE’s changing condition | - Able to leverage the existing NWDAF analytics to assist the AF to continue monitoring the UE, QoS etc, performance during the application AI/ML operation. | - Able to reuse the existing UPF exposure mechanism to monitor the bit rate for the selected group of UEs to support aggregated monitoring request from AF. - The monitoring can be applied before or during the Application AI/ML operation - The mechanism can support FL operation for aggregated bit rate monitoring with minimum signaling overhead between the 5GC and AF |
| Cons: | - Not always efficient as the monitoring is mainly done on per UE basis to report to the AF, i.e. signaling overheads could be high - Not clear if the monitoring is on-going while the Application AI/ML operation was started. This is especially important in case of the FL operation. - No all performance/resource monitoring are necessarily done by the NWDAF | - No assistance to the AF on learning the UE’s performance before starting the Application AI/ML operation. - Not always efficient as the monitoring is mainly done on per UE basis to collect User Experience and/or DN performance from the AF, i.e. signaling overheads could be high - No all performance/resource monitoring are necessarily produced by the NWDAF - There is no on-going feedback from the 5GC to the AF to monitor each UE’s performance for participating in the Application AI/ML operation | - No assistance to the AF on learning the UE’s performance before starting the Application AI/ML operation. - Not always efficient as the monitoring is mainly done on per UE basis to report to the AF, i.e. signaling overheads could be high - No all performance/resource monitoring are necessarily done by the NWDAF | - Limited to the aggregated bit rate monitoring only |

# 8 	Conclusions

## 8.1	Interim Conclusions

### 8.1.X	Key Issue#1

Proposal for the interim conclusions for KI#1:

- Propose to merge Solution#1, Solution#11, Solution#34 which leverage NWDAF analytics to perform some performance/resource monitoring prior to the start of the Application AI/ML operation as follows:

  - Refer to the procedure in Solution#1 to perform the pre application AI/ML operation monitoring and provide the feedback to the AF

  - In addition, if any of the NWDAF analytics that were described in Solution#11 and Solution#34 which are useful and applicable to assist the AF’s operation decision to prepare for the start of the application AI/ML operation, such NWDAF analytics should also be included as the pre application AI/ML operation monitoring.

- Propose to extend the AF Influence include NWDAF analytics from Solution#1 and Solution#34, if applicable

  - Extend the AF Influence procedures as described in Solution#11 to include the support of the additional NWDAF analytics (could be with extension) as described in Solution#1 and Solution#34, if applicable

- Propose to include the Group-MBR monitoring procedures as described in Solution#37 to assist the AF decision prior to the start of the Application AI/ML data transfer operation and during the Application AI/ML data transfer operation in order to provide the on-going monitoring.

NOTE: Further enhancement to optimize the overall system performance for these solutions’ merger may be needed during the normative phase of this study.

***** End of Changes *****