3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206515

Elbonia, 17th – 26th Aug, 2022	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	FS_XRM_KI#9 Sol#61 EN Remove about AMF procedure

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This paper proposes to update solution 61 about AMF procedure.

1. Introduction/Discussion

In solution#61, there is following Editor's notes in clause 6.61.3.1:

Editor’s Note:	The detailed mechanism of UE battery level provision is FFS.

The Solution highlights the enhancement to support trade-off among throughput, latency and reliability and device battery life, with the UE battery based QoS adjustment. And how to get the UE battery status from UE by the AMF or SMF is not address during SA2#151e meeting.

When the AMF get UE battery status from UE, the UE battery status can be provided to the AMF (e.g. using the UE 5GMM Core Network Capability information) during the Initial Registration or Mobility Registration Update procedure within the NAS message. In this case, the RAN receive and forward the UE status information within the NAS message. There is no RAN impact.

It is proposed to add the description about the UE battery provision and remove the Editor’s note in clause 6.61.3.1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60 V0.3.0.

# * * * * First change * * * *

6.61.3.1	AM Policy Association Establishment and Modification

The following figure represents a high-level procedure of the solution that UE battery level change is defined as an access related policy control request trigger and PCF subscribes to AMF on UE battery level change.

Figure 6.61.3-1: A high-level procedure of AM Policy Association Establishment and Modification.

1-4. Step 1-4 according to Step 1-4 of Clause 4.16.1.2 in TS 23.502[3]. The UE battery level change is defined as an access related policy control request trigger. PCF subscribes to AMF on UE battery level change in step 3. The AMF deploys the access and mobility related policy information which includes storing the Policy Control Request Trigger(s) of the AM Policy Association (e.g. UE battery level change).

5. When the UE battery level change Trigger condition is met, the AM Policy Association Modification procedure is initiated by the AMF. The UE status (e.g. UE battery level) can be provided to the AMF (e.g. using the UE 5GMM Core Network Capability information) during the Initial Registration or Mobility Registration Update procedure within the NAS message.

6-9. Step 6-9 follow Step 1-4 of Clause 4.16.2.1.1 in TS 23.502[3]. When UE battery level change event is received from AMF in step 6, PCF determines the policy based on the current UE battery level in step7, e.g. a QoS level corresponding to a low or ultra-low battery level is selected to conduct the service. PCF evaluates policies for the SDF of UE XRM services or UE PDU sessions of XRM services.

Alternatively, the UE battery level can be stored to the UDR by the PCF for later use and skip the follow steps. The UE battery level change can be used with the combination of other events for policy determination, e.g. GFBR of the QoS Flow can no longer be guaranteed.

UE battery level is also can be reported to AF by the PCF, so that AF can take some action (e.g. change codec) to keep an aligned policy.

6.61.4	Impacts on services, entities and interfaces

UDR:

-	The UE battery level can be stored to the UDR by the PCF.

PCF:

-	When UE battery level change event is received, PCF determines the policy based on the current UE battery level (or stores into the UDR), reports to AF for taking action (e.g. change codec) to keep an aligned policy.

AMF:

-	When the PCF subscribed to AMF on UE battery level change, the AMF deploys the policy information which includes storing the UE battery level change trigger, and notifies to PCF when the trigger condition is met. The AMF can get the UE battery level from UE within the NAS message.

SMF:

-	When the PCF subscribed to SMF on UE battery level change, the SMF enforces the subscription, and notifies the PCF for the event when the trigger condition is met.

Editor’s Note:	How the SMF obtains the battery level information is FFS.

UE:

-	The UE provides the battery level to AMF/SMF (via RAN).

# * * * * End of changes * * * *