3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206516

Elbonia, 17th – 26th Aug, 2022	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	FS_XRM_KI#9 Sol#61 EN Remove and SMF procedure Update

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This paper proposes to update solution 61 about SMF procedure.

1. Introduction/Discussion

In solution#61, there are following Editor's notes in clause 6.61.3.2 and 6.61.4:

Editor’s Note:	The detailed mechanism of UE battery level provision is FFS.

Editor’s Note:	How the AMF or SMF obtains the battery level information is FFS.

The Solution highlights the enhancement to support trade-off among throughput, latency and reliability and device battery life, with the UE battery based QoS adjustment. And how to get the UE battery status from UE by the AMF or SMF is not address during SA2#151e meeting.

When the SMF get UE battery from UE, the UE battery status information can be provided to the SMF (e.g. using the PCO information element) during the PDU session establishment or the PDU session modification procedures within the NAS message. In this case, the RAN receive and forward the UE status information within the NAS message. There is no RAN impact.

It is proposed to add the description about the SMF procedures and the impacts NFs, and then remove the Editor’s note in clause 6.61.3.2 and the 6.61.4.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60 V0.3.0.

# * * * * First change * * * *

6.61.3.2	SM Policy Association Establishment and Modification

The following figure represents a high-level procedure of the solution that UE battery level change is defined as a non-access related policy control request trigger and PCF subscribes to SMF on UE battery level change.

Figure 6.61.3-1: A high-level procedure of AM Policy Association Establishment and Modification.

1-4. Step 1-4 according to Clause 4.16.4 in TS 23.502[3]. The UE battery level change is defined as an non-access related policy control request trigger. PCF can subscribe to SMF events, e.g. UE battery level change, associated with the PDU Session in the Npcf_SMPolicyControl_Create response in Step 4.

5. When the UE battery level change Trigger condition is met, the SM Policy Association Modification procedure is initiated by the SMF. The UE status (e.g. UE battery level) can be provided to the SMF (e.g. using the PCO information element) during the PDU session establishment or the PDU session modification procedures within the NAS message.

6-9. Step 6-9 follow the description of Clause 4.16.5.1 in TS 23.502[3]. When UE battery level change event is received from SMF in step 6, PCF determines the policy based on the current UE battery level in step8, e.g. a QoS level corresponding to a low or ultra-low battery level is selected to conduct the service. PCF evaluates policies for the SDF of UE XRM services or UE PDU sessions of XRM services managed by the SMF.

Alternatively, the UE battery level can be stored to the UDR by the PCF for later use and skip the follow steps. The UE battery level change can be used with the combination of other events for policy determination, e.g. GFBR of the QoS Flow can no longer be guaranteed.

UE battery level is also can be reported to AF by the PCF in step 7, so that AF can take some action (e.g. change codec) to keep an aligned policy.

6.61.4	Impacts on services, entities and interfaces

UDR:

-	The UE battery level can be stored to the UDR by the PCF.

PCF:

-	When UE battery level change event is received, PCF determines the policy based on the current UE battery level (or stores into the UDR), reports to AF for taking action (e.g. change codec) to keep an aligned policy.

AMF:

-	When the PCF subscribed to AMF on UE battery level change, the AMF deploys the policy information which includes storing the UE battery level change trigger, and notifies to PCF when the trigger condition is met.

SMF:

-	When the PCF subscribed to SMF on UE battery level change, the SMF enforces the subscription, and notifies the PCF for the event when the trigger condition is met. The SMF can get the UE battery level from UE within the NAS message.

UE:

-	The UE provides the battery level to AMF/SMF (via NAS).

# * * * * End of changes * * * *