3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206562

Elbonia, Aug 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion on KI#3

Document for:	Approval

Agenda Item:	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract: This paper proposes conclusion on support of redundant traffic steering.

1. Introduction/Discussion

Based on the evaluation described in the 7.x, as proposed in the contribution S2-2206561, conclusion on KI#3 is added.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause enlists the conclusions of the study, including agreements for work to be done in the normative phase.

8.x	Conclusions on KI#3: Support of redundant traffic steering

The following is concluded for normative work:

- A new steering mode is to be defined for the redundant traffic steering.

- A PLR threshold value may be provided with the redundancy steering mode as a trigger for the redundant transmission, as defined in solution 3.1. This PLR threshold can be set by PCF based on the QoS parameters received from AF as defined in solution 3.2.

- When PLR threshold is not provided, the traffic will always be transmitted redundantly over both accesses.

# * * * * End of changes * * * *