SA WG2 Meeting #152E (e-meeting)	S2-2206576

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI#1, Evaluation

Document for:	Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2 / Rel-18

Abstract of the contribution: This paper evaluates solutions for KI#1 "Transport C2 communication over PC5 interface".

Discussion

Regarding Key Issue #1 - Transport C2 communication over PC5 interface in clause 5.1 of TR 23.700-58v0.3.1, there are 3 solutions.

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |  |  |  |
| 1 | X |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |

This paper provides evaluations on these solutions.

Proposal

It is proposed to agree the following changes into TR 23.700-58v0.3.1.

* * * * Start of 1st Change * * * *

!!! All New Texts !!!

7.x	Key Issue #1 - Transport C2 communication over PC5 interface

For Key Issue #1, three solutions are proposed:

-	Solution#1 mainly proposes the following:

-	C2 communication between UAV and UAV-C using NR PC5 unicast link.

-	The PC5 unicast link can be established.

-	If Layer 2 ID of paired device is configured (e.g. for the case that the UAV and UAV-C are pre-paired), Direct Discovery procedure is not needed. Otherwise, Direct Discovery procedure is needed to learn Layer 2 ID of paired device.

-	Solution#4 mainly proposes the following:

-	C2 communication between UAV and UAV-C using NR PC5 unicast link.

-	The PC5 unicast link can be established by using UE oriented or service oriented Layer-2 link establishment procedure, without performing Direct Discovery procedure.

-	UAS Service Identifier included in the Direct Communication Request message sent by the UAV may be preconfigured or derived from the UAV's CAA-Level UAV ID.

-	Solution#6 mainly proposes the following:

-	C2 communication path switching from over PC5 path to over network.

-	UAV-C can determine whether to perform switching to C2 connection over network or C2 connection release.

-	UAV-C's UAS layer may provide C2 connection status information such as 'Signal strength/status of C2 connection over PC5', 'Indication whether C2 connection over network is possible' to the application layer.

-	UAV-C's application layer may request switching to C2 connection over network or C2 connection release to the UAS layer, e.g. based on the C2 connection status information.

* * * * End of Changes * * * *