SA WG2 Meeting #S2-152e(e-meeting)	S2-2206613

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #1: Add Solution Evaluation for Key Issue #1

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3

Abstract of the contribution: the pCR is proposed to add the solution evaluation for key issue#1.

1. Discussion

The solutions of key issue#1 include sol#1, sol#2, sol#3, sol#4, sol#5, sol#15 and sol#32. This pCR is proposed to add solution evaluation for key issue#1.

2. Proposal

It is proposed to add solution evaluation for key issue#1.

* First change *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.x	Key Issue #1: Support of Network Slice Service continuity

The solutions of key issue#1 include sol#1, sol#2, sol#3, sol#4, sol#5, sol#15 and sol#32. The evaluation of solutions of key issue#1 is shown in Table 7.x-1.

Table 7.x-1: Evaluation of Solutions for Key Issue #1

| Sol | Sol#1 | Sol#2 | Sol#3 | Sol#4 | Sol#5 | Sol#15 | Sol#32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NF decides slice/ slice instance change | AMF | PCF | NG-RAN | AMF | AMF | SMF, AMF | AMF |
| Information used by the NF to make the decision | Load level information or experience for a Slice or slice instance provided by the NWDAF | PCF local configuration of pairs of slices and AMF request of slice re-mapping. | NG-RAN local configuration of slice re-mapping information | The slice on which the PDU Session is initially required is not available or is overload | The current network slice is not supported or overload in the target CN. | The Network Slice instance is overloaded, e.g. based on OAM | Slice congestion or operator maintenance requirements |
| How to change the slice/slice instance | During PDU session establishment or HO procedure, AMF notifies the additional S-NSSAI to SMF. SMF provides the additional S-NSSAI to RAN. | Re-use the SSC mode 2 or mode 3 mechanisms with enhancement that SMF provides the UE the selected S-NSSAI and a new cause indicating that PDU Session re-established to selected S-NSSAI. | After HO，NG-RAN sends end slice and new slice to AMF. AMF sends the information to SMF which then triggers SSC mode 3. | UE includes compatible S-NSSAI in the PDU Session establishment request. The AMF may select compatible S-NSSAI to establish the PDU session. | AMF triggers PDU Session modification to change the S-NSSAI of the PDU Session to alternative S-NSSAI which is provided by UE during PDU Session establishment procedure. | Re-use the mechanism of SSC mode#2 or mode#3. | 1. AMF provides source and target S-NSSAIs to UE via UCU or to SMF and then to UE via SM container. 2. UE establishes PDU session with target S-NSSAI and releases PDU session with source S-NSSAI. |
| Applicable scenario | Scenarios 1b, 1c and 2d | It is an FFS in the solution. | Inter RA mobility | Scenario 1b | Scenario 2d | Both no mobility and inter RA mobility scenario | Both no mobility and inter RA mobility scenario |
| Impacted NF(s) | AMF, SMF, UPF, NG-RAN (TBD) | AMF, PCF, SMF, NRF | NG-RAN, AMF, SMF | AMF | AMF, SMF, NG-RAN | AMF, SMF | AMF, SMF（impacted only in NW initiated approach） |
| UE impacted | No | Yes | Yes | Yes | Yes | No | Yes |
| Cons | The S-NSSAI of PDU Session is mismatched between UE and network. | The pair of S-NSSAIs is pre-configured in the PCF. It is not clear how to determine the pair dynamically based on slice load. | Slice mapping information is pre-configured in the NG-RAN. It is not clear how to determine the mapping information dynamically based on slice load. | The UE is impacted to store the slice mapping information. So the signalling cost is high to update/delete the mapping information. Furthermore, it is not clear how PCF determines the mapping information. | Same as Cons for sol#4. Furthermore, AMF may select another SMF during the PDU session modification procedure. It is not clear how it works and how to guarantee slice service continuity. | Only network slice instance change is supported. | SSC mode of PDU Session can be overrided by UE. |

* End of change *