3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206627

# Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0112 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 5G ProSe Policy Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the description in clause 4.3.1 of TS 23.304, UE indicates 5G ProSe Policy Provisioning Request in UE Policy Container for Registration Request or UE triggered 5G ProSe Policy provisioning, which may request one or multiple types of policies/parameters as listed below: - Policy/parameters for 5G ProSe Direct Discovery; - Policy/parameters for 5G ProSe Direct Communication; - Policy/parameters for 5G ProSe Layer-2 Remote UE; - Policy/parameters for 5G ProSe Layer-3 Remote UE; - Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay; - Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay. …. And also there is the related description in clause 5.3.2.2 of TS 24.554, In order to initiate the UE-requested ProSeP policy provisioning procedure, the UE shall create a UE POLICY PROVISIONING REQUEST message (see example in figure 5.3.2.2.1). The UE shall include the Requested UE policies IE indicating whether the UE policies for 5G ProSe direct discovery, the UE policies for 5G ProSe direct communications, the UE policies for 5G ProSe UE-to-network relay or any combination of them are requested; …. However, in clause 6.2.2, it wrongly states that 5G ProSe capability is also included in the 5G ProSe Policy Provisioning Request. Actually, the UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message in clause 6.6.2 of TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the description on 5G ProSe capability included in the 5G ProSe Policy Provisioning Request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe capability is included in the 5G ProSe Policy Provisioning Request |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [5], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [5] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [5] apply with the following additions:

-	If the UE indicates 5G ProSe Capability in the Registration Request message and if the UE is authorized to use 5G ProSe service based on subscription data, the AMF selects the PCF which supports 5G ProSe Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for 5G ProSe Policy/Parameter delivery.

-	If the UE supports one of 5G ProSe capabilities and it does not have valid 5G ProSe policy/parameters, the UE includes the UE Policy Container with indicating the 5G ProSe Policy Provisioning Request during registration procedure. The UE may request one or multiple types of policies/parameters as defined in clause 4.3.1 in the 5G ProSe Policy Provisioning Request and PCF may determine the ProSe Policy/Parameters for the UE based on the 5G ProSe Policy Provisioning Request.

-	If the UE indicates the 5G ProSe Policy Provisioning Request in the UE Policy Container, the PCF determines whether to provision 5G ProSe Policy/parameters to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [9], and the PCF provides the 5G ProSe Policy/parameters (see clause 5.1.2.1, clause 5.1.3.1 and clause 5.1.4.1) to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [5].

The PCF may update the 5G ProSe Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [5].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform 5G ProSe services. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [5].

-	When there is a change of service specific parameter (including path selection policy) as described in clause 6.2.5 (performing the procedure in clause 4.15.6.7 of TS 23.502 [5]).

-	When the timer associated with some Policy/parameter expires.

-	When the UE determines that the ProSe Policy/parameter(s) is invalid and performs UE triggered Policy Provisioning procedure to the PCF.

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF as specified in clause 6.2.4 when the UE determines the 5G ProSe Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

When the UE disables a ProSe capability, the PCF may stop updating the corresponding ProSe Policy/parameter(s) and when the UE enables a ProSe capability the PCF may need to provide or update the corresponding ProSe Policy/parameter(s).

When a 5G ProSe Layer-3 Remote UE is accessing to 5GC via a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

# * * * * End of changes * * * *