3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206633

Elbonia, August 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	KI#5 Sol#27 Update: Adding Support of multipath transmission via Layer-3 UE to Network Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This update adds procedures to enable multipath transmission using a Layer 3 UE to Network Relay without the need of deploying N3IWF or ATSSS. The solution improves the reliability and is transparent for the AS.

# 1. Introduction/Discussion

This contribution updates solution 27 to support two-path transmission via a Layer 3 UE to Network Relay without N3IWF or support in the application function.

Summary of the solution when using Layer 3 UE to Network Relay:

- The solution considers the general case where the Remote UE and U2N relay are using different UPFs.

- The UPF used by the first path performs the role of anchor UPF and routes the traffic to and from the application function.

- A forwarding tunnel is established between the anchor UPF and the UPF of another path.

- The Remote UE triggers the establishment of multipath.

- The solution is transparent to the AF

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 6.27	Solution #27: Remote UE traffic handling for Multi-Path with Layer-3 relay without N3IWF

### 6.27.1	Description

This solution is related to the Key Issue #5: Support of multi-path transmission for UE-to-Network Relay, and it mainly focuses on supplementing one direct network communication path with one indirect network communication path established via Layer 3 UE-to-Network Relay without N3IWF.

In this solution, the 5G ProSe Remote UE triggers the establishment of multi-path communication based on one or more of the following criteria:

-	Application provides multi-path requirements;

-	UE ProSe policy, UE ProSe policy contains multi-path usage indication which can be pre-configured or configured by the network;

-	UE implementation, e.g. based on the application layer high QoS requirement;

-	Enhance URSP, RSD includes a new item "Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication".

When the 5G ProSe Remote UE determines to use the multi-path transmission based on the above triggers, the 5G ProSe Remote UE still follows the traffic handling for 5G ProSe UE-to-Network Relay support as described in clause 6.5.4 of TS 23.304 [3].

When to decide to use multi-path transmission with including the 5G ProSe Layer-3 relay without N3IWF path, the 5G ProSe Remote UE should have same understanding on the PDU session parameters usage on the direct network path and indirect network path, details can be seen in clause 6.27.2.1.

For the application with capability to integrate the traffic from multiple paths, the Remote UE can directly use direct and indirect path for multi-path transmission, the integration is up to application layer.

For the application without capability to integrate the traffic from multiple paths, the procedure in clause 6.27.2.2 is applied, in the procedure, the 5G ProSe Remote UE provides the necessary information to the 5GC for the direct path and indirect path PDU sessions to be correlated. In case the two PDU sessions terminate at different UPFs, the 5GC creates a forwarding tunnel between the UPFs.

The UPF serving the originally established path performs the role of anchor UPF and routes the traffic to and from the application server while the UPF terminating the additional path receives the downlink data for the Remote UE from anchor UPF and sends the uplink data from the Remote UE to the anchor UPF. With reference to Figure 6.27.1-1, UPF-1 acts as anchor when the direct path was established first, UPF-2 acts as anchor when the indirect path via the Layer-3 UE-to-Network Relay path was established first.

UE

/

Remote UE

NG-RAN

Layer-3

UE-to Network

Relay

UPF-1

NG-RAN

UPF-2

AS

Alt1: UPF-1 is anchor

Alt2: UPF-2 is anchor

UE

/

Remote UE

NG-RAN

Layer-3

UE-to Network

Relay

UPF-1

NG-RAN

UPF-2

AS

Alt1: UPF-1 is anchor

Alt2: UPF-2 is anchor

Figure 6.27.1-1: Multi-path transmission using a direct path and a path via Layer-3 UE-to-Network Relay without N3IWF. The UPF of the first path established acts as anchor path.

### 6.27.2	Procedure for Remote UE traffic handling for Multi-Path with Layer-3 relay without N3IWF

#### 6.27.2.1	Establishment of multipath transmission using Layer-3 UE-to-Network Relay

Figure 6.27.2.1-1: Multi-path transmission with using 5G ProSe UE-to-Network Relay without N3IWF

As described in clause 6.5.4 of TS 23.304 [3], the application traffic on the 5G ProSe Remote UE is managed by URSP rules, the 5G ProSe Remote UE establishes indirect or direct path as the first path based on whether the selected RSD contains the component of "5G ProSe Layer-3 UE-to-Network Relay Offload indication". The 5G ProSe Remote UE triggers the establishment the second path when multipath transmission is desired, for example, the first path cannot satisfy the QoS requirement of the application traffic. When establishing the second path, the Remote UE determines the parameters of the second path using the following procedure:

-	If the selected RSD contains the "5G ProSe Layer-3 UE-to-Network Relay Offload indication", then the indirect network path is 5G ProSe Layer-3 UE-to-Network Relay, then to determine the direct network path:

-	The 5G ProSe Remote UE determines the PDU session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) based on the UE ProSe policy as described in clause 5.1.4.1 of TS 23.304 [3].

-	The 5G ProSe Remote UE takes the above determined PDU session parameters as the PDU session parameters of the direct network path, and to evaluate the existing PDU sessions, or establish a new PDU session.

-	If the selected RSD does not contain the "5G ProSe Layer-3 UE-to-Network Relay Offload indication" and it determines to use direct network path as the first path, then to determine the indirect network path:

-	If there is the configurations for 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and the 5G ProSe Layer-3 UE-to-Network Relay without N3IWF is selected as the indirect path, the 5G ProSe Layer-3 Remote UE takes UE ProSe policy as described in clause in clause 5.1.4.1 of TS 23.304 [3] to determine the RSC and corresponding PDU session parameters for the indirect network path setup.

Another option is that the indirect path and direct path are always established for transmitting the application traffic based on URSP rules. The 5G ProSe Remote UE establishes the indirect path and direct path based on the selected RSD containing the new component of "Multi-Path 5G ProSe Layer-3 UE-to-Network Relay Offload indication". Either the indirect path or direct path can be used as the first path based on UE implementation. The 5G ProSe Remote UE can trigger the establishment of the indirect path and direct path consecutively in either order. The PDU session parameters together with "Multi-Path 5G ProSe Layer-3 UE-to-Network Relay Offload indication" in the selected RSD are used as the PDU session parameters for the direct path, and the RSC used for the indirect path can be decided by ProSe policy as per Rel-17.

#### 6.27.2.2	Procedure

Figure 6.27.2.2-1 illustrates the procedure for establishing multipath transmission via a Layer 3 UE-to-Network relay by correlating direct path and indirect path. Steps 1-5 illustrate the scenario1where an indirect path via a Layer-3 UE-to-Network Relay is added to an existing direct path; steps 6-10 illustrate the scenario2 where a direct path is added to an established indirect path via a Layer-3 UE-to-Network Relay. The other steps are common to both scenarios.

When establishing the second path, the Remote UE determines the PDU session parameters of the second path as specified in clause 6.27.2.1.

In this procedure, we always assume that AMF-1, SMF-1 and UPF-1 serve the PDU session of the Remote UE using the direct path, and AMF-2, SMF-2 and UPF-2 serve the PDU session of the Layer 3 UE-to-Network Relay in the indirect path. Which UPF performs the role of anchor UPF depends on which PDU session was first established, i.e. for scenario 1 the UPF-1 performs the role of anchor UPF and for scenario 2 the UPF-2 performs the role of anchor UPF.

Layer-3

UE-to-NW Relay

Remote

UE

AMF-1

AMF2

SMF-1

UPF-2

UPF-1

SMF-2

11. UL NAS transport (L3 relay multipath, relay UE 5G-GUTI, PDU session ID, QFI, Remote User ID, Anchor path, PDU Session Modification Request)

12. AMF-1 gets the address of SMF-2 that serves relay UE’s PDU session

13. Nsmf_PDUSession_UpdateSMContext Request (L3 relay multipath, relay UE 5G-GUTI, PDU session ID, QFI, Remote User ID, address of SMF-2, PDU Session Modification Request)

14a. N4 Session Modification Request

14b. N4 Session Modification Response (CN Tunnel Info-1)

15. Nsmf_PDUSession_UpdateSMContext Request (L3 relay multipath, relay UE PDU session ID, QFI, Remote User ID, CN Tunnel Info-1)

16a. N4 Session Modification Request (CN Tunnel Info-1)

16b. N4 Session Modification Response (CN Tunnel Info-2)

17. Nsmf_PDUSession_UpdateSMContext Response (CN Tunnel Info-2)

18a. N4 Session Modification Request (CN Tunnel Info-2)

18b. N4 Session Modification Response

19. Other procedures for PDU Session Modification defined in TS23.502

Data

20b.Data

20b.Data

20a.Data

3. L2 link Est/mod request (L3 relay multipath)

5. L2 link Est/mod Accept (relay UE 5G GUTI, PDU session ID, QFI)

PCF

2. Layer3 U2N Discovery Procedure

7. L2 link Est/mod request (L3 relay multipath)

9. L2 link Est/mod Accept (relay UE 5G GUTI, PDU session ID, QFI)

6. Layer3 U2N Discovery Procedure

Data

Scenario 1

Scenario 2

Scenario 1

21b.Data

21a.Data

21b.Data

Scenario 2

0. Remote UE and relay UE are authorized to perform multipath with L3 U2N relay

1. Remote UE establishes PDU session-1 and QoS flow-1 on direct path

4. Relay UE may establish/modify PDU session -2 and QoS flow-2 for relaying

8. Relay UE may establish/modify PDU session -2 and QoS flow-2 for relaying

10. Remote UE establishes PDU session-1 and QoS flow-1 on direct path

Figure 6.27.2.2-1: Establishment of multipath transmission using Layer-3 UE-to-Network Relay

0.	The Remote UE is authorized by network to use Layer-3 multipath transmission with Layer-3 UE-to-Network Relay. The Layer-3 UE-to-Network Relay is authorized by network to provide indirect path for Remote UE performing Layer-3 multipath transmission with a Layer-3 UE-to-Network Relay.

Steps 1-5 illustrate scenario 1 where an indirect path via a Layer-3 UE-to-Network Relay is added to an existing direct path. In scenario 1, the remote UE exchanges data with an Application Server via UPF-1.

Steps 6-10 illustrate scenario 2 where a direct path is added to an established indirect path via a Layer-3 UE-to-Network Relay. In scenario 2, the remote UE exchanges data with an Application Server via UPF-2.

In step 3 and step 10, the Remote UE determines the PDU session parameters of the second path as specified in 6.27.2.1.

1.	The Remote UE establishes PDU session-1 and QoS flow-1 as defined in TS 23.502 [8]. This PDU session is served by UPF-1.

2.	The Remote UE performs Layer-3 UE-to-Network Relay discovery procedure as defined in TS 23.304 [3].

3. 	The Remote UE request Layer-3 UE-to-Network to establish or modify one-to-one communication as defined in TS 23.304 [3] with following additions:

The Remote UE provides "L3 relay multipath" that indicates to the Layer-3 UE-to-Network Relay that the Remote UE intends to use the indirect path via the Layer-3 UE-to-Network Relay in a multipath configuration.

The Remote UE also indicates Layer-3 UE-to-Network Relay PC5 QoS flow Identity on which traffic data is subject to multipath transmission.

The Remote UE may also indicate PC5 QoS rule if only some traffic on a PC5 QoS flow is subject to multipath transmission.

4.	The Layer-3 UE-to-Network Relay may establish a new PDU session to relay the traffic of the Remote UE or modify an existing PDU session.

5.	The Layer-3 UE-to-Network Relay sends response to the Remote UE as described in TS 23.304 [3] with following additions:

The Layer-3 UE-to-Network Relay sends its 5G-GUTI, the PDU session ID and QFI that it uses to relay the Remote UE traffic for multipath transmission. This information will be used for correlating the direct and indirect communication paths.

The Layer-3 UE-to-Network Relay may also send QoS rule if the Remote UE indicates PC5 QoS rule in step 3.

Steps 6-9 are the same as steps 2-5 and step 10 is the same as step 1.

11.	The Remote UE initiates the PDU Session Modification procedure transmitting a NAS message (N1 SM container (PDU Session Modification Request)) to AMF-1 as defined in TS 23.502 [8] with the additional parameters in NAS: "L3 relay multipath", Layer-3 UE-to-Network Relay 5G-GUTI, Layer-3 UE-to-Network Relay PDU session ID (PDU session ID-2) and QFI (QFI-2) or QoS rule (QoS rule-2) used for the Remote UE traffic, Remote User ID, PDU session ID-1 and QFI(QFI-1) or QoS rule (QoS rule-1) for the data for Layer-3 multipath transmission and anchor path.

The anchor path is used to indicate which path should be configured as anchor path. For scenario 1, the anchor path is direct path and for scenario 2, the anchor path is indirect path.

12.	AMF-1 uses the Layer-3 UE-to-Network Relay 5G-GUTI to determine AMF-2 which serves the Layer-3 UE-to-Network Relay and gets the address of the SMF that serves the PDU session ID-2 (SMF-2).

13.	AMF-1 requests SMF-1 to modify the PDU session using Nsmf_PDUSession_UpdateSMContext Request as defined in TS 23.502 [8]. In this step, AMF-1 forwards the additional parameters received in step 11 to SMF-1 with the addition of the address of SMF-2.

14a. The SMF-1 sends an N4 Session Modification Request to the UPF-1.

If the SMF-1 supports the Layer-3 UE-to-Network Relay multipath transmission, SMF-1 instructs UPF-1 to establish a forwarding tunnel. For scenario 1, SMF-1 also instruct UPF-1 to send a copy of the downlink data for Layer-3 multipath transmission to UPF-2 and for scenario 2, SMF-2 also instructs UPF-1 to send the uplink data received from UPF-2 via forwarding tunnel to the Application Server.

14b. UPF-2 acknowledges the request to establish a tunnel by sending an N4 Session Modification Response.

UPF-1 provides CN uplink Tunnel Info (scenario 1) or CN downlink Tunnel Info (scenario 2) for data forwarding tunnel to SMF-1.

15.	SMF-1 sends Nsmf_PDUSession_UpdateSMContext Request as defined in TS 23.502 [8] to SMF-2 with the following parameters: "L3 relay multipath", Layer-3 UE-to-Network Relay SUPI, Layer-3 UE-to-Network Relay PDU session ID (PDU session ID-2) and QFI (QFI-2) or QoS rule (QoS rule-2) used for the Remote UE traffic, Remote User ID and anchor path.

If the anchor path is direct path, based on the Remote User ID, the SMF-2 retrieves the Remote UE info and generate PDR for UPF-2 to identify traffic data from the Remote UE for which should be sent to UPF-1.

16a. SMF-2 sends an N4 Session Modification Request to the UPF-2.

If SMF-2 supports Layer-3 multipath transmission for the Remote UE, SMF-2 provides UPF-2 the information about the CN uplink/downlink Tunnel info established in step 14.

The SMF-2 also instructs UPF-2 to establish a forwarding channel (CN downlink Tunnel for scenario 1 or CN uplink Tunnel for scenario 2). For scenario 1, SMF-2 also instruct UPF-2 to send a copy of the downlink data for Layer-3 multipath transmission to UPF-1, and for scenario 2, SMF-2 also instruct UPF-2 to send the uplink data received from UPF-1 via forwarding tunnel to the Application Server.

16b. UPF-2 acknowledges by sending an N4 Session Modification Response.

UPF-2 provides CN downlink Tunnel Info (scenario 1) or CN uplink Tunnel Info (scenario 2) as forwarding tunnel to SMF-2.

17.	SMF-2 to SMF-1: Nsmf_PDUSession_UpdateSMContext Response (CN Downlink Tunnel Info).

18a. SMF-1 sends an N4 Session Modification Request to the UPF-1(CN downlink Tunnel Info (scenario 1) or CN uplink Tunnel Info (scenario 2)).

18b. The UPF-1 acknowledges by sending an N4 Session Modification Response.

19.	Other procedures for PDU Session Modification defined as defined in TS 23.502 [8] with following additions:

SMF-1 indicates the Remote UE starting Layer-3 multipath transmission in the PDU session modification command.

20-21.The traffic data applied to Layer-3 multipath transmission are transmitted both on direct path (20a for scenario 1 and 21a for scenario 2) and indirect path (20b for scenario 1 and 21b for scenario 2) based on the Layer-3 multipath transmission strategy (switching/redundancy).

For the data to be transmitted on the first path, the Remote UE generates IP packets based on IP address and port number of the first path

For the data to be transmitted on the second path, the following manipulation of the IP header is enhanced for the uplink and downlink as following:

Uplink

- The Remote UE sets two IP headers, the inner IP header is generated with the IP address and port of the first path, and the outer IP header is generated with the IP address and port of the second path.

- The UPF that serves the first path removes the outer IP header received from the UPF that serves the second path and performs IP layer elimination and sorting for the data from two path.

Downlink

- The Remote UE removes the outer IP header and performs IP layer elimination and sorting for the data from two path.

- The UPF that serves the second path adds an additional IP header for the data received from the UPF that serves the first path based on the IP address and port number assigned for the Layer-3 UE-to-Network Relay that serves the Remote UE (scenario 1) or the IP address and port number assigned to the Remote UE (scenario 2).

### 6.27.3	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

Remote UE(s):

-	Support multi-path transmission.

-	Can determine the PDU session parameters based on another path without URSP evaluation for multi-path transmission with including the 5G ProSe Layer-3 relay without N3IWF path.

-	Support packet replication and elimination for two paths.

Layer-3 UE-to-Network Relay:

-	Provide its PDU session info used to relay traffic for the Remote UE.

PCF:

- Support authorisation for both the Remote UE and Layer-3 UE-to-Network Relay with Layer-3 multipath transmission.

SMF:

- Support enhanced PDR for Layer-3 multipath transmission.

UPF:

- Support IP data handling and forwarding based on PDR for Layer-3 multipath transmission.

# * * * * End of changes * * * *