3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206658

Elbonia, 17th – 26th Aug, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eLCS_Ph3 KI#3 evaluation and conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: This paper proposes evaluation and conclusion for KI#3

1. Introduction/Discussion

This paper proposes evaluation and conclusion for KI#3

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71 V0.3.0.

# * * * * First change * * * *

7.x	Key Issue #3: Local Area Restriction for an LMF and GMLC

There are 4 solutions(solution 6,7,8,9) for KI#3.

Solution 6 and solution 7 proposes that additional information is provided from GMLC to AMF to assist the LMF selection.

-	Solution 6 proposes that the GMLC provides LMF ID associated with the GMLC to AMF. The AMF uses the LMF ID to select an LMF via local configuration or NRF. The LMF ID is local configured in GMLC. This solution has less impact on AMF configuration.

-	Solution 7 proposes that the GMLC provides client identifiers or AF identifiers to AMF. The AMF uses the client identifier or AF identifier to select an LMF via local configuration or NRF. Normally the client identifier or AF identifier are used only in application layer. The usage of these application layer identifiers in 5GC will cause complexity in configuration.

Solution 8 and solution 9 proposes LMF selection based on local configuration in AMF.

-	Solution 8 proposes that the AMF selects the LMF based on the GMLC service area via local configuration or via NRF. The GMLC service area is local configured per GMLC in the AMF. This solution adds more complexity in AMF configuration.

-	Solution 9 proposes that the AMF is configured with a mapping table of UE identity(e.g. MSISDN) and the LMF/GMLC address, or the AMF fetches the local LMF and local GMLC address in LCS subscription from UDM. In this solution, the UE is bound to single LMF/GMLC, which is not flexible. The per UE configuration in AMF also cause system complexity.

# * * * *2nd change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.x	Key Issue #3: Local Area Restriction for an LMF and GMLC

The following aspects are concluded for normative work:

-	The GMLC provides LMF ID to AMF. The AMF uses the LMF ID to select the LMF, as described in solution#6.

-	The AMF is configured with GMLC service area per GMLC. The AMF uses the GMLC service area to select the LMF, as described in solution#8.

# * * * * End of changes * * * *