3GPP SA WG2 Meeting #152e 	S2-2206740

Electronic Meeting, 17 August – 26 August 2022	(revision of S2-2204431)

Source:	China Telecom, China Unicom, ZTE, vivo, CATT, Inspur, Huawei, Spreadtrum Communications, InterDigital

Title:	New WID on Dynamically Changing AM Policies in the 5GC Phase 2 (TEI18 _DCAMP_Ph2)

Document for:	Approval

Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Dynamically Changing AM Policies in the 5GC Phase 2

Acronym:	TEI18_DCAMP_Ph2

Unique identifier:

Potential target Release:	{Rel-18}

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 870005 | Dynamically Changing AM Policies in the 5GC | Rel-17 Work Item |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

3	Justification

The access and mobility policy control introduced in 5G encompasses the management of service area restrictions, the management of the RFSP functionalities, the management of the UE-AMBR, the management of the UE Slice-MBR and the management of the SMF selection by PCF.

Some potential enhancements on AM policy are identified based on the Rel-17 SA2 issues and requirements:

- As conclusion from DCAMP, the PCF serving a UE may receive notification from AF or from PCF(s) serving a PDU session that specific application traffic starts/stops, and then trigger AM policy check and update. More than one application is likely to be notified as “active” at the same time. However, the solution in R17 is only applicable to non-roaming scenario. In the real network, the AF may not aware any roaming UEs or group of roaming UEs is in roaming state or not. To support AF’s requirement can be applied to roaming UEs, e.g. in LBO scenario where PCF for UE and the PCF for PDU session belong to VPLMN, enhancement on the current mechanism of dynamically changing AM policies for AF's request is needed.

- In clause 6.1.2 of TS23.503, it mentions:

The PCF modifies the RFSP Index based on operator policies that take into consideration e.g. accumulated usage, load level information per network slice instance, the indication that high throughput is desired for a specific application traffic or independently of the application in use and other information.

Although the accumulated usage can impact RFSP index, it only considers the scenario where PCF for a UE and PCF for a PDU Session are the same PCF instance. One the other hand, up to R17, it defines how the PCF for PDU session interact with SMF for usage monitoring report for session related policy decisions. However, it does not elaborate on how PCF for UE could retrieve usage monitoring report when PCF for PDU session and PCF for UE are not the same one.

4	Objective

The objective is to enhance the dynamically changing AM policies, as follows:

Objective 1: To define the mechanism of supporting DCAMP in roaming scenario. This objective will focus on LBO roaming scenario.

Objective 2: To define the mechanism of allowing PCF for the UE to make AM policy decision based on UE’s accumulated usage. This objective will cover aspects when the PCF for the UE and the PCF for the PDU Sessions of a UE are different PCFs.

Note: This work will require 0.75 TU.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 TS 23.503 | Update the procedures to show how the AF influences AM policy decisions in LBO roaming scenario. Define the procedure of dynamically changing AM policies based on accumulated usage Define the mechanism of usage monitoring to enable PCF for UE could retrieve usage monitoring report when PCF for PDU session and PCF for UE are not the same one. | TSG#99 | This will be handled as “TEI18”. |

6	Work item Rapporteur(s)

Yubing Liu, liuyb9@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

China Unicom

ZTE

vivo

CATT

Inspur

Huawei

Spreadtrum Communications

InterDigital