3GPP TSG-SA WG2 Meeting #152-e	S2-2206762

# E-Meeting, 17th – 26th Aug 2022

Title:	[draft]LS Out on NSSRG restriction on pending NSSAI

Response to:	S2-2205406/ C1-224073

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks for CT1 LS on the NSSRG restriction on pending NSSAI. SA2 answers the question as follows:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

SA2 Answer: No. The NSSRG information is provided together with the Configured NSSAI. The UE only includes in the Requested NSSAI S-NSSAIs that share a common NSSRG. The UE shall not include in the Requested NSSAI any of the S-NSSAIs from the Pending NSSAI the UE stores.

2. Actions:

To CT1 working group.

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153e		10-14,Oct, 2022	E-Meeting

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting