SA WG2 Meeting #152E	S2-2206786

# Aug 17th – 26th, 2022; Elbonia

Source: 	Samsung

Title: 	New Solution KI#2: Provisioning N3IWF info for Credential Holder scenarios

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This solution proposes to update the SNPN Credential Holder related subscription data with N3IWF info.

# 1	Discussion

Since there were many unresolved Editor’s notes for the Credential Holder scenario support in Untrusted 3GPP access case, we propose a simple solution that can be considered.

All the SNPNs which broadcast a particular GIN may be large in number. We have suggested to include N3IWF information (for few SNPNs) in CH prioritized lists of GIN/SNPNs during provisioning as well as during default configuration for On-boarding purposes.

Rather than including only a single N3IWF per UE, it might be necessary to include multiple N3IWFs because a particular 3GPP access node may not be reachable from a particular N3IWF in case of internal networks.

The next suggestion is How to update these prioritized list.

Because of dynamic relationships between a CH and the individual SNPNs, it is necessary to update the N3IWF list (Note that this was not the issue in NG-RAN/Trusted Non-3GPP access where GIN can be broadcasted by the Access network cells; and the SNPN who no longer has service relationship with a particular CH will stop broadcasting that particular GIN). Currently, SoR procedure can only be done when CH is UDM/AUSF (which is essentially a separate PLMN/SNPN) which can be used to update these lists. Since the current SoR information already include Credential Holder Prioritized lists, no further impacts would be necessary.

# 2 Proposal

It is proposed to consider this solution for KI#2 of TR 23.700-08 FS_eNPN_Ph2.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  | X | X |  |
| 12 |  |  | X | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| 19 |  | X |  |  |  |  |
| 20 |  | X |  |  |  |  |
| 21 |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |
| 23 |  |  |  | X |  |  |
| 24 |  |  |  | X |  |  |
| 25 |  |  |  | X |  |  |
| 26 |  |  |  | X |  |  |
| 27 |  |  |  | X | X |  |
| 28 |  |  |  | X | X |  |
| 29 |  |  |  | X |  |  |
| 30 |  |  |  | X |  |  |
| 31 |  |  |  | X |  |  |
| 32 |  |  |  | X |  |  |
| 33 |  |  |  | X |  |  |
| 34 |  |  |  | X |  |  |
| 35 |  |  |  |  | X |  |
| 36 |  |  |  |  | X |  |
| 37 |  |  |  |  | X |  |
| 38 |  |  |  |  |  | X |
| 39 |  |  |  |  |  | X |
| X |  | X |  |  |  |  |

* * * Second Change * * * *  ALL text is new

## 6.X	Solution #X: Provisioning N3IWF info for credential Holder scenarios

### 6.X.1	Introduction

### This solution proposes to add additional information related to N3IWF info in Credential Holder prioritized list of GINs or Credential Holder prioritized list of SNPNs. UE can use these list during N3IWF selection when connecting to SNPN when UE have credentials for a Credential Holder.

### 6.X.2	Functional Description

The following additions are proposed for UE subscription information as mentioned in clause 5.30.2.3 of TS 23.501.

|  | PLMN ID and NID of the subscribed SNPN |
| --- | --- |
|  | Subscription identifier (SUPI) and credentials for the subscribed SNPN |
|  | Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located |
| Optionally, if the UE supports access to an SNPN using credentials from a Credentials Holder : | User controlled prioritized list of preferred SNPNs; |
|  | Credentials Holder controlled prioritized list of preferred SNPNs, optionally an N3IWF FQDN of that SNPN |
|  | Credentials Holder controlled prioritized list of GINs; optionally N3IWF FQDN(s) of those SNPN(s) which broadcast this GIN. |

Note: The N3IFW FQDN is stored per SNPN or per GIN in the prioritized lists.

Note: How the Credential Holder knows the N3IWF information is out of 3GPP scope.

### For the case when Credential Holder is UDM/AUSF the prioritized list can be updated via SoR procedure as defined in TS 23.122.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

UE impact: doing N3IWF selection based on the information present in CH controlled prioritized lists.