3GPP TSG-SA WG2#152e	S2-2206920

Elbonia, 17-26 August 2022	(was S2-220xxxx)

Source:	Intel

Title:	KI#7 new solution: CAG based MBSR access control

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/ Rel-18

Abstract of the contribution: This paper proposes a new solution for key issue 7.

# 1 Discussion

The solution proposed in this pCR address the following in KI#7:

- 	Whether and how to manage access control for the UE's access to IAB-node;

- 	whether and how to manage the UE's access (e.g. permitted/restricted) to IAB-node for specific geographical location(s), time window(s), or other conditions/criteria;

-	whether and how to improve the control of new UEs.

In this solution, CAG mechanism is reused for MBSR access control.

# 2 Proposal

It is proposed to update TR 23.700-05 as follows.

Start of change

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | x |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |
| 3 |  | x |  |  |  |  |  |
| 4 |  | x | x |  |  |  |  |
| 5 |  | x | x |  |  |  |  |
| 6 |  |  |  | x |  |  |  |
| 7 |  |  |  |  | x |  |  |
| 8 |  |  |  |  | x |  |  |
| 9 |  |  |  |  |  | x |  |
| 10 | x |  |  | x |  |  |  |
| 11 |  | x |  |  |  |  |  |
| 12 |  |  | x |  |  |  |  |
| 13 |  |  | x |  |  |  |  |
| 14 |  |  |  |  | x |  |  |
| 15 |  |  |  |  | x |  |  |
| X |  |  |  |  |  |  | X |

Next change (all new text)

## 6.X	Solution #X: CAG based MBSR access control

### 6.1.1	Introduction

The solution provides methods for control of UE's access to 5GS via a mobile base station relay.

### 6.1.2	Functional Description

The solution is based on the support of existing Closed Access Group (CAG) concept as specified in TS 23.501 [2] with the following high-level description:

-	CAG is used to prevent/to allow UE(s), to access the mobile base station relay.

-	A mobile base station relay acts as CAG cell which broadcasts one or multiple CAG Identifiers as described in 5.30.3 of TS 23.501

NOTE:	Whether there is any enhancement on CAG cell will be determined by RAN group.

-	A UE, that supports CAG as indicated as part of the UE 5GMM Core Network Capability, may be pre-configured or (re)configured with the CAG information, included in the subscription, e.g. Allowed CAG list, as described in 5.30.3 of TS 23.501:

-	Network and cell (re-)selection, and access control is performed as in 5.30.3.4 of TS 23.501.

### 6.1.3	Procedures

All procedures in TS 23.502 related to CAG, e.g. Registration Management procedures in clause 4.2.2 of TS 23.502 [5], UE Configuration Update in clause 4.2.4 of TS 23.502 [5] can be re-used for control of UE's access to 5GS via a mobile base station relay.

### 6.1.4	Impacts on services, entities, and interfaces

AMF:

-	none.

UE:

-	none.

End of change