SA WG2 Meeting #S2-152e(e-meeting)	S2-2207089

17 - 26 August, 2022, Electronic Meeting	(revision of S2-2206616)

Source:	CATT

Title:	KI #6: Add solution evaluation and conclusion for key issue#6

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: this pCR is proposed to add solution evaluation and conclusion for key issue#6.

1. Discussion

For key issue#6, there is one solution, i.e. sol#14, in the TR 23.700-71. This pCR proposes to add solution evaluation and conclusion for key issue#6.

2. Proposal

This pCR proposes to add solution evaluation and conclusion for key issue #6.

* First change *

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions of clause 6.

7.x	Key Issue #6: UE Positioning without UE/User Awareness

There is one solution in TR 23.700-71, i.e. sol#14, related to key issue#6 which aims to support UE not notified by any means during the LCS session, especially when UE is in CM_IDLE or RRC_INACTIVE state.

To support key issue#6, two indications (i.e. UE unaware indication and user unaware indication) are introduced in sol#14. When UE unaware indication is provided by LCS Client/ AF, if target UE is in CM_IDLE or RRC_INACTIVE state, the target UE is not paged. When user unaware indication is provided by LCS Client/ AF, the only exception with the existing positioning procedure is that interaction between network and user, i.e. privacy check requiring such interaction, is skipped. Thus the function of the two indications is different. The impacts to NFs of the two indications are further analysed in Table 7.x-1.

Table 7.x-1: Impacts to NFs of Different Indications in Solution#14

| Impacts | UE unaware | User unaware |
| --- | --- | --- |
| GMLC impacts | Receive UE unaware indication from LCS Client/NEF and send the indication to AMF. | Receive user unaware indication from LCS Client/NEF and send the indication to AMF. |
| NEF impacts | Receive UE unaware indication from AF and send the indication to GMLC. | Receive user unaware indication from AF and send the indication to GMLC. |
| AMF impacts | Receive UE unaware indication from GMLC and behave differently based on UE state: - UE in CM_CONNECTED: send the UE unaware indication to LMF. - UE in CM_IDLE: not to page UE and rejects the location request from GMLC or returns the latest UE location. | Receive user unaware indication from GMLC and skip the privacy check which requires interaction with user is skipped. |
| LMF impacts | Receive UE unaware indication from AMF, select uplink positioning method (e.g. UL E-CID) based on the indication and send the network positioning message including UE unaware indication to RAN node. | No |
| RAN impacts | Receive UE unaware indication from LMF and behave differently based on UE state: - UE in RRC_CONNECTED: RAN behaviour is not impacted. - UE in RRC_INACTIVE: not to page UE and reject the network positioning message. | No |

Based on the analyses in Table 7.x-1, user unaware indication requires the network to page UE when UE is in CM_IDLE or RRC_INACTIVE state during the positioning procedure which is not required by UE unaware indication. Thus the user unaware indication can only support user unawareness, the UE unaware indication can support both UE and user unawareness.

Editor's note:	The evaluation needs update based on conclusion in SA2#152E meeting.

* Second change *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.X	Conclusions for Key Issue #6

For key issue #6, it is concluded that solution#14 (UE unaware indication) is used as the baseline for normative work.

* End of changes *