SA WG2 Meeting #152E e-meeting	S2-2207100

Elbonia, 17 – 26 Aug, 2022	(revision of S2-226218r04)

Source:	China Telecom, Huawei, Ericsson

Title:	23.700-89 Solution Evaluation for Key Issue 1

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract: Solution evaluation for Key Issue #1 for FS_AMP.

- Introduction

This paper illustrates and compares all the solutions proposed for KI#1 for FS_AMP in TR23.700-89 for N26-based interworking scenario.

2.	Proposal

It is proposed to adopt the following evaluation into 23.700-89.

# * * * * ALL NEW TEXT * * * *

7.1.X	Evaluation on Key Issue #1: RFSP Index consistency when UE moves from 5GC to EPC for N26-based interworking scenario

There are 4 solutions proposed for Key Issue #1, targeting to provide RFSP Index consistency when UE moves from 5GC to EPC for N26-based interworking scenario.

Table 7.1-1 compares the principles between the proposed solutions.

Table 7.1.x-1: Evaluation on principles of KI#1 candidate solutions

| Solution # | Need to keep AM Policy Association after UE Moves from 5GC to EPC | Impacts to Existing Nodes and Proposed Enhancements |  |  |  | Use of Validity Condition(s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | AMF | MME | HSS+UDM | PCF for a UE |  |
| 1 | X | X | X | X (AMF be aware of the change of serving MME from HSS/UDM) | - | New timer for AM association maintenance at AMF and equivalent new timer for MME to use RFSP Index from AMF |
| 3 | - | X | X | - | X (PCF for a UE generates a timer along with the authorized RFSP Index) | MME re-evaluates the RFSP Index when the timer expires. |
| 6 | X | X | X | X (maintain the UE's 5G registration state temporarily for some duration and provide the MME address to AMF) | - | HSS+UDM maintain UE's 5G registration state for AM Policy Association maintenance timer and provides it to AMF. During the validity timer, if PCF for a UE updates RFSP index, AMF queries the MME address from HSS+UDM and send the updated RFSP index to MME. |
| 10 | - | X (AMF replaces "subscribed RFSP Index" with "authorized RFSP Index" from PCF in UE context and send to MME.) | X | - | - | When the configured duration is reached, the MME may remove the "5GC Authorized RFSP Index" from its context for the UE and start using the "subscribed RFSP Index" that was received from the HSS. |

All these solutions are trying to answer the following 3 questions for N26-based interworking in KI#1:

Q1: Whether the current interworking procedure supports MME received the RFSP Index in use from 5GC? If no, what enhancements is needed.

For N26-based interworking: (Solution #1, #3, #6 and #10) the current specification already supports the RFSP Index in use transferred to MME in UE context during handover/TAU.

If there is any enhanced information, it should also be transferred via N26.

Q2: If the MME get the RFSP Index in used in handover procedure or idle mode mobility procedure, how and when it resumes to the subscription RFSP Index.

- (Solution #1 and #6) The PCF for a UE maintains the AM Policy Association when UE move from 5G to 4G and updates the authorized RFSP Index according to triggers and conditions defined in current specification. The MME re-selects the RFSP Index in use considering the dynamic input from PCF at the cost of network overhead and extra procedures.

- When MME realizes a UE is moving from 5GC, it starts a timer within which value of the RFSP Index in use will not be changed. When the timer expires, the MME will be free to re-select the RFSP Index in use according to current method. The value of this timer may be set to:

  - (Solution #10) A local value preconfigured in MME, which is same to all UE and not adjustable.

  - (Solution #3) A value selected by PCF when it generates an authorized RFSP Index value indicating the UE to move from 5G to 4G. So the timer is per UE from 5G and not adjustable after the UE in 4G. PCF can decide that timer based on information, e.g., from UDR or NWDAF. If the AMF modifies the authorized RFSP Index from PCF, i.e., the AMF selects a different value of RFSP Index in use, whether the timer value from PCF should still be sent to MME is clarified in S2-2205525 (ongoing discussion).

Considering the trade-off of system impact and flexibility, the and PCF-selected timer seems to solve the problem in a more agreeable way. Preconfigured timer in MME can implementation specific without standardization as such timer will apply to EPS to 5GS mobility scenario not targeted in the KI .

Q3: When UE is under EPC, should MME receive any update of RFSP Index from 5GC and how.

  - (Solution #1 and #6) Yes. The PCF is able to update the RFSP Index In Use or the timer according to, e.g., AF's influence request. And MME is able to change the RFSP Index in use in a timely manner.

  - (Solution #3 and #10) No. The MME is able to re-select the RFSP Index only after the validity time provided by the AMF (decided by PCF) expires.

The evaluation of Q2 also applies to Q3.

# * * * * End of change * * *