SA WG2 Meeting #152E e-meeting	S2-2207101

Elbonia, 17 – 26 Aug, 2022	(revision of S2-2206237r04)

Source:	China Telecom, Huawei, Ericsson, Nokia, Samsung

Title:	23.700-89 Conclusion for Key Issue 1

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract: Conclusion for Key Issue #1 for FS_AMP.

- Introduction

This paper proposes to conclude the principles for KI#1 for FS_AMP in TR23.700-89 for N26-based interworking.

2.	Proposal

It is proposed to adopt the following conclusion into 23.700-89.

# * * * * ALL NEW TEXT * * * *

8.1.X	Conclusion Key Issue #1: RFSP Index consistency when UE moves from 5GC to EPC for N26-based interworking

The following conclusion are agreed:

-	When a UE moves from 5GC to EPC, the MME sets the "RFSP Index in use" value the same as the value in the UE context received from AMF via N26 if the UE context in AMF also contains a validity period.

-	When the validity period expires, the MME re-evaluates the RFSP Index value according to current specification (see clause 4.3.6 of TS 23.401 [5]).

-	The validity period, i.e., "RFSPinUseExpiryTime" is selected by PCF for a UE. When the PCF for a UE provides the Authorized RFSP Index indicating the UE to move to 4G, it may also provide the value of "RFSPinUseExpiryTime". If the AMF selects the RFSP Index in use identical to the authorized RFSP Index, it should store the received "RFSPinUseExpiryTime" in UE context in AMF and further also send the received "RFSPinUseExpiryTime" to MME in UE context along with the "RFSP Index in use" when N26 interface applied.

-	When mobility happens between MMEs before the "RFSPinUseExpiryTime" expires, the remaining value of the "RFSPinUseExpiryTime" should also be sent to the new MME. The new MME handles RFSP Index in the same way as for mobility from the AMF to the MME as described above.

-	When a UE is served by the EPC, there is no use case or requirement to provide updated RFSP Index from 5GC to the EPC.

# * * * * End of change * * *