3GPP SA WG2 Meeting #152E	S2-2207102

Elbonia, August 17 - 26, 2022	(was S2-2205521)

Source:	Ericsson

Title:	KI#1, 4: Update to solution 16 - MRF.

Document for:	Discussion/Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC

Abstract of the contribution: This contribution proposes an update to solution 16 wrt MRF.

# 1.	Discussion

An IMS network supporting IMS DC does not need to update all MRFs to support enhanced capabilities needed for IMS DC. This should be reflected in the architecture.

# 2. Proposal

It is proposed to accept the following changes to TR 23.700-78

FIRST CHANGE

## 6.16	Solution #16: Solution for IMS Media Support for DC Server

### 6.16.1	Description

In this solution, the MRF is enhanced to support the IMS DC feature. To provide the DC Signalling Function (DCSF) with the enhanced MRF Egress information for Bootstrap data as an example, as well as receive the DCSF bootstrapping addressing information, the IMS AS interacts with the DCSF for that purpose.

Figure 6.16.1-1 depicts a high level architecture diagram for the enhanced MRF solution showing the architecture of the IMS for supporting the IMS Data Channel feature, illustrating the interworking between IMS and other entities needed to support DC feature in this option.

In this architecture, the IMS MRF is enhanced to support additional functionality related to DCSF and other data channel media. This functionality is referred to as Data Channel Media Function (DCMF) while maintaining backward compatibility, and reusability. MRFs that are expected to support DC feature shall be enhanced for that purpose.

NOTE:	Not all deployed MRFs needs to be enhanced to support DC feature.

In this architecture,  the '3rd Party WebRTC enabled Application' provides application media content relayed over established WebRTC 1.0 RTCDataChannels (IF10). The '3rd Party WebRTC enabled Application' establishes peer media connectivity using WebRTC data channels.

Figure 6.16.1.1-1includes the following interfaces:

-	IF1 between the enhanced MRF and DCSF for media transport related to bootstrapping. What is transported over IF1 is only relevant to the UE. IF1 is HTTP based.

-	IF10 between the enhanced MRF and 3pty Web RTC Applications. IMS is transparent to the data channel payload media carried on this interface.

-	IF6 between the IMS AS and DCSF for managing DC feature related to IMS data channel including media control. This interface hast to be extensible and support vendor specific information with no impact on interoperability. IF6 is service based.

-

-	DCSF includes multiple functions. The control function for DCSF decides usage of data channel media within the sessions, interacting with both the IMSAS and third-party WebRTC applications. The Bootstrap media function terminates HTTP bootstrap channels established with call sessions, hereby taking the role of a standard web server, potentially acting as a proxy for HTTP resources hosted by external third-party webservers. The DC Application repository includes bootstrap media content fetched by TS 26.114 [6] UE's.

-	MRF (MRFC/MRFP) includes both legacy media resources (e.g. Voice and Video).

- 	Enhanced MRF (includes the data channel media function DCMF resources).

NOTE:	Not all MRFs have to be enhanced to support DCMF.

-	The IMSAS can utilize the Network Repository Function to discover the MRF instance(s). The NRF provides the IP address or the FQDN of MRF instance(s) to the IMSAS. The MRF selection function in the IMSAS selects the MRF instance(s) based on the available MRF instances obtained from NRF or based on the configured MRF information in the IMSAS. The following factors may be considered during the MRF discovery and selection: - UE location information. - Local operator policies. - Availability of candidate MRFs. - UE IP address. - Access Type. - Proximity to location of selected UPF if possible.

IF6 is in scope of the TR.

Nimsas implements services supported by the IMS AS.

Ndcsf implements services supported by the DCSF.

Editors note:	Nimsas, and Ndcsf are FFS.

Note 1: Application Repository can be outside DCSF

Figure 6.16.1-1: IMS DC Enhanced MRF Architectural Option