3GPP TSG-SA WG2 Meeting #152-e	S2-2207511

Electronic meeting, 17 – 26 August 2022						 			(rev of S2-2206852r02)

Source:	Interdigital，China Telecom

Title:	KI #3: Solution Update on Sol#22

Document for:	Approval

Agenda item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution:  This PCR update solution #22.

# 1. Discussion

This PCR updates solution #22 to include how to support make before break type of path switching for both case.

There is a E.N. “Whether other ProSe signalling can be used for PDU session status exchange is FFS”. According to TS 24.554, among PC5 signalling messages, only ProSe direct link keepalive request and response messages are proper for periodic exchange of signalling message. And in Rel-18, there may be other PC5 signalling message defined. Therefore, it is proposed to change the E.N. to NOTE saying that whether other ProSe signalling can be used for PDU session status exchange is to be defined during normative phase.

Besides, when UE1 and UE2 decide to switch path from direct Uu path to direct PC5 path, e.g. UE1 and UE2 are proximity to each other or to offload traffic from the network, the two addresses of PDU sessions in direct Uu communication path can be easily reused for direct PC5 communication path, since the two addresses are valid locally in this PC5 path. It is also valuable to reuse the addresses of PDU sessions in direct Uu communication path in order to minimize the service interruption between the path switching, considering if new addresses are configured in PC5 path then it may have impact on the application since the addresses of the ongoing communication are changed. This pCR proposes to reuse the addresses (IP, Ethernet, Unstructured) of PDU sessions for the PC5 path.

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-33.

* * * First Change * * *

## 6.22	Solution #22: Direct Communication Path switching between PC5 and Uu

### 6.22.1	Description

This solution resolves Key Issue #3 for direct communication path switching between PC5 and Uu.

### 6.22.2	Procedures

#### 6.22.2.1	Direct Communication Path switching from PC5 to Uu

Figure 6.22.2.1-1: Path switching from PC5 to Uu

0.	UE1 and UE2 are provisioned with path switching policies (e.g. enabled/disabled, preferred interface, radio quality thresholds) by using the procedure as defined in clause 6.2 of TS 23.304 [3]. For each ProSe service which is allowed for path switching between PC5 and Uu, the UEs are provided with PC5 QoS parameters which will be allowed for path switching to Uu connection.

1.	UE1 and UE2 establish a PC5 connection. During this procedure, the Path switching parameters (e.g. path switching enabled/disabled) are exchanged.

2.	UE1 informs UE2 about its PDU session status (e.g. available for PDU session establishment) by using the PC5 Keepalive procedure.

3.	UE2 informs UE1 about its PDU session status (e.g. available for PDU session establishment) by using the PC5 Keepalive procedure.

4.	If UE1 determines to switch path from PC5 to Uu e.g. based on PC5 signal level and UE2's PDU session status (i.e. PDU session establishment is available), UE1 sends a PC5 Path Switching Request message to UE2, including the path switching direction (i.e. PC5-to-Uu).

5.	UE2 may establish a PDU session or modify an existing PDU session for switched traffic. In the PDU session establishment request or modification request, UE2 includes a Path Switch indication from PC5 to Uu informing the 5GCN to assign QoS flows for the PC5 QoS flows of ProSe service to be path switched. Based on the Path Switch indication and, requested DNN, the 5GCN may assign QoS flows for the applications allowing path switch.

NOTE. It is assumed that 5GCN has PCC rules for the applications supported over Uu connection and PC5 connection and allowing path switch between Uu and PC5 connection for each UE.

6.	UE2 sends a PC5 Path Switching Response message to UE1, which includes the IP address associated to UE2's PDU session.

7.	UE1 may establish a PDU session or modify an existing PDU session for switched traffic. In the PDU session establishment request or modification request, UE2 includes a Path Switch indication from PC5 to Uu informing the 5GCN to assign QoS flows for the PC5 QoS flows of ProSe service to be path switched. Based on Path Switch indication and, requested DNN, the 5GCN may assign QoS flows for the applications allowing path switch.

8.	UE1 sends a PC5 Path Switching Ack message to UE2, which includes the IP address associated to UE1's PDU session. UE1 may send a PC5 Path Switching Abort message to abort the path switching procedure e.g. if the PDU session establishment/modification failed.

9.	After the step 8, UE1 and UE2 may release the PC5 connection and use Uu path for communication.

Editor's note:	How to support Ethernet traffic and Unstructured traffic is FFS.

#### 6.22.2.2	Direct Communication Path switching from Uu to PC5

Figure 6.22.2.2-1: Path switching from Uu to PC5

0.	UE1 and UE2 are provisioned with path switching policies (e.g. enabled/disabled, preferred interface, radio quality thresholds) by using the procedure as defined in clause 6.2 of TS 23.304 [3]. UE1 and UE2 use Uu path for communication.

1.	UE1 and UE2 establish a PC5 connection. In order to discover each other during PC5 discovery phase, discovery parameters e.g. Application ID, target User Info ID, may be provided by the application or derived in UE1 and UE2 using the information e.g. application ID, User ID of UE1, User ID of UE2, which are used during step 0. During this procedure, the Path switching parameters (e.g. path switching enabled/disabled) are exchanged. During PC5 connection setup, for ProSe service allowed path switching, UE1 and UE2 negotiate QoS within the provisioned PC5 QoS parameters to be allowed for path switching.

For the case that the data unit type for PDU session in step 0 is IP type, UE1 and UE2 decide to reuse the IP addresses of PDU sessions for PC5 communication, e.g. in case the IP address is public address. During Layer-2 link establishment procedure defined in clause 6.4.3.1 in TS 23.304[3], UE sends an indication to reuse the addresses of the PDU sessions, e.g. UE may send PROSE DIRECT LINK SECURITY MODE COMMAND message with an indication, followed by a PROSE DIRECT LINK SECURITY MODE COMPLETE message without IP address configuration.

For the case that the data unit type for PDU session in step 0 is Ethernet or Unstructured, UE1 and UE2 reuse the addresses and data unit type of the PDU sessions used for direct communication for PC5 communication with an indication e.g. in PROSE DIRECT LINK SECURITY MODE COMMAND message.

Editor's note:	It is FFS how to avoid conflict between shared IP address in Uu and ones in other PC5 connection when IP address assigned from the network and the ones used by U2U are in the same ranges.

2.	UE1 sends a PC5 Path Switching Request message, including the path switching direction (i.e. Uu-to-PC5) and theaddress associated to UE1's PDU session.

3.	UE2 sends a PC5 Path Switching Response message, including the address associated to UE2's PDU session.

4.	UE1 and UE2 use PC5 path for communication. The addresses used over Uu may be re-used over the PC5 session, if indicated during the PC5 link establishment.

5.	UE1 and UE2 may release/modify their PDU session.

### 6.22.3	Impacts on services, entities and interfaces

UE:

-	Support path switch procedure.

-  	Reuse the address of PDU session for PC5 path

PCF:

-	Support path switch policy provisioning and handling policy control based on path switch indication.

SMF:

-	Handling PDU session based on path switch indication.

* * * End of Changes * * *