SA WG2 Meeting #S2-152E	S2-2207709

17 – 26 August , 2022, E-meeting                            rev of S2-2206902r02

Source:	Xiaomi, Ericsson

Title:	evaluation for key issue#3

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation for key issue #3.

*** Start of changes ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.3 Evaluation for KI#3 Enabling NPN as hosting network for providing access to localized services

| Solution#7 High level flow for localized service support | The service provider (can be network operators or 3rd party application providers) of the localized service needs to establish localized service agreement with the operator of a hosting network. A service agreement between hosting network operator and UE's home network operator is needed to enable, e.g.: UE to receive and use configuration provided by a 3rd party service provider, etc. This solution proposes to re-use traditional OAM mechanisms and existing SA5 specified mechanisms to handle the configuration aspect of hosting network. |
| --- | --- |
| Solution#12 Discovering services offered by SNPN while camping in a serving network | Current serving network (PLMN or SNPN) may assist UE in discovering hosting network for localized services in specific conditions, such as when serving network determines that UE moves into area where localized services are available. Some level of co-operation between current serving network and hosting network is needed. The service agreement between serving network and hosting network is assumed in place by means outside of 3GPP. |
| Solution#13 Exposure enhancements to support providing access to localized services | Provides a network-exposure based solution to handle interaction related to how hosting network based on a request from localized service provider to fulfil localized service e.g. the QoS requirements. The AF at the localized service provider sends a request to the NEF (and then PCF) of the hosting network, and the PCF updates the parameters of AMF/SMF in hosting network to enforce the network rules for localized services. Editor's note: A complete evaluation of solution #13 for KI#3 is FFS. |
| Solution#22 Hosting network to provide localized service based on default credentials | It is assumed that UE has default credentials that can be used in the hosting network. Hosting network broadcasts a dedicated Hosting networking service indication to show that this NPN can provide hosting networking for localized services. - In case there is some service agreement between hosting network and home network, the UE can be successfully authenticated by the hosting network by using credentials of home network - In case there is no service agreement between hosting network and home network, the hosting network performs authentication by using the UE’s default credentials. The localized services information are pre-configured in AMF of hosting network. NOTE: Further impacts to address key issue#3 aspects are to be evaluated when additional details/clarifications are available. |

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.3 	Key Issue #3: Enabling NPN as hosting network for providing access to localized services

Editor's note:	These are *INTERIM* conclusions for Key issue #3.

When describing the feature description in the normative phase, the definitions for localized service and localized service provider in clause 3 can be used as basis.

There are different types of interactions between hosting network operator and localized service provider. Depending on the types of the interactions, following principles are proposed to address this key issue:

Editor's note:	Principles to use are FFS.

*** End of changes ***