SA WG2 Meeting #152E e-meeting	S2-2205542

Elbonia, 17th – 26th August 2022

Source: 	Ericsson

Title: 	KI#1, Sol#29: Update to remove EN

Spec:	3GPP TR 23.700-81

Document for: 	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract: This contribution removes an EN for solution #29 which is proposed for KI #1.

# Discussion

An Editor’s Note in Solution #29 reads:

Editor's note:	It is FFS whether and how AnLF indicates its support of ML model monitoring during the ML model provisioning procedure

The purpose of the Nnwdaf_MLModelMonitor_Register request is for the NWDAF containing AnLF to indicate to the NWDAF containing MTLF that it has started making use of a previously downloaded model.

If the NWDAF containing AnLF subscribed for receiving notifications of new ML models, the notification of the existence of a new ML model cannot be understood as if the NWDAF containing AnLF automatically downloads and starts using such new ML model. It may happen that there are no consumers of analytics for the given Analytics ID, so the NWDAF containing AnLF decides not downloading the new available ML model until it needs to use the model. Furthermore, when the actual download takes place, the NWDAF containing AnLF invokes the URL received in a notification. Such URL may point to a storage which is different from the NWDAF containing MTLF, therefore, the NWDAF containing MTLF might never get informed of which NWDAF containing AnLF has decided to download the ML model. Hence, once the NWDAF containing AnLF has downloaded the ML Model, may not use it until some event takes place (e.g., a new service consumer requests a prediction for that Analytics ID).

Conclusion: with the existing procedures in R17, the NWDAF containing MTLF is never informed of which NWDAF containing AnLF is using an ML model.

The proposed Nnwdaf_MLModelMonitor_Register service operation solves this issue. The NWDAF containing AnLF learns that the NWDAF containing MTLF supports this service operation through the NWDAF containing MTLF NF Profile downloaded from NRF, which lists the supported service operations.

It is therefore proposed to remove the Editor’s Note.

# Proposed Solution

The following updates are proposed to be adopted to TR 23.700-81 v0.3.0.

********** Start of Changes ***************

#### 6.29.2.4	Procedure for the registration of ML model monitoring

Figure 6.29.2.4-1 illustrates the procedure by which an NWDAF containing AnLF registers with an NWDAF containing MTLF that it is starting to monitor the degradation of an ML model. A new Nnwdaf_MLModelMonitor_Register service operation is used for that purpose.

Figure 6.29.2.4-1: Procedure for ML model monitoring registration

1.	An NWDAF containing AnLF starts monitoring an ML model, due to start using the ML model for producing analytics.

2-3.	The NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Register request to an NWDAF containing MTLF (NWDAF containing AnLF NF ID, unique identifier of the ML model, optionally: subscription endpoint of the ML Monitoring service operation). The NWDAF containing MTFL is now aware of the NF ID of the NWDAF containing AnLF that are monitoring that ML model.

When the NWDAF containing AnLF does no longer monitor the ML model, it sends an Nnwdaf_MLModelMontior_Unregister service operation. The NWDAF containing MTLF deletes the associated data.

********** End of Changes ***************