SA WG2 Meeting #S2-151E	S2-2204627

16 - 20 May, 2022, Electronic meeting

Source:	Vodafone

Title:	Inter-country VoIMS Handover and IMS idle mode mobility related to SA3LI LS in S2-2203662/S3i220244

Agenda item:	4.1

Document for:	Discussion and decision

# 1	Introduction

3GPP SA3Li has sent an LS in S2-2203662/S3i220244 highlighting the issues of providing the lawful interception for the VoIMS and IMS session using S8HR/N9HR home routing when cross-country Inter-PLMN handover or idle mode mobility occurs.

This discussion paper provides the background information, discusses the issue, and raises the questions focusing mainly on SA2 aspects. To facilitate the discussions, a figure showing mobility from the HPLMN to a VPLMN using an S8HR roaming scenario is below.

## 2	Background

At least 5GAA and several EU based networks have been investigating network deployments that enable inter-PLMN handover of both “data communication” and IMS voice calls. Home Routed PDN connections are assumed when roaming (S8HR for EPC and N9HR for 5GC).

An underlying assumption is that inter-PLMN idle mode mobility with IP address continuity is also enabled.

In recent years, several 3GPP Working Groups have discussed the inter-PLMN handover topic and there was general 3GPP agreement that the 3GPP standards enable it, but that its lack of widespread deployment is just a rollout matter, or an operational decision from operators.

However, recent investigations into the 3GPP standards indicate that there could be some LI issues related to the handover of ongoing VoLTE (and VoNR) calls, as described in the incoming LS in S2-2203662/S3i220244.

The issues relate to the potential need for the ‘new’ PLMN (the one that is the ‘target’ of the handover and aka VPLMN throughout this paper) to be able to intercept IMS communications after the handover.

This can be broken down into 2 primary components:

- The capability to intercept the ongoing call in the ‘new’ VPLMN;

- The capability to intercept new calls that are started in the ‘new’ VPLMN (e.g. after S10 based idle mode mobility, or, if 1, above, is not technically possible).

## 3	Discussion

As 3GPP SA3-LI indicated, in order to allow the lawful interception of VoLTE calls using S8HR in the VPLMN, the VPLMN may require IMS confidentiality protection to be turned off.

This applies to all outbound roaming UEs and there is no exchange of “LI target UE” information between HPLMN and VPLMN. Relying on communication between the LI entities in the HPLMN, old VPLMN and new VPLMN is not acceptable.

The VPLMN needs to know the SIP identities of both the calling and called parties e.g. Law Enforcement Agencies want the capability:

- to intercept calls between a non-targeted UE in the VPLMN and a target SIP identity that is not in that VPLMN, and

- to know with whom a targeted UE is communicating

To enable the VPLMN to obtain the SIP identities, the IMS confidentiality protection needs to be removed.

Vodafone believes that CT 1 is best placed to look into the IMS signalling aspects that could enable the IMS encryption to be disabled during an on going call and how the SIP identities (and codec details) can be made visible to the VPLMN.

However there are other aspects that are more related to SA2, for example:

- Any IMS signalling in the VPLMN would be after the HPLMN and/or UE discovered that the UE was in a new PLMN. Typically this would follow the TA Update that is performed after the Handover is completed. Hence there would be at least some 100’s of ms of communication in the new PLMN before interception could start. This information should be provided back to SA3-LI.

- At inter-PLMN handover, the PDN GW would become aware of the new PLMN ID. However, do the SA2 specifications cause the PLMN ID to be automatically made available to the P-CSCF (via the PCRF)?

- If the new VPLMN detects that the HPLMN is not enabling LI to be performed, what actions can the VPLMN take?

- If the IMS standards or the IMS implementations in UEs do not enable LI of ongoing calls at handover, how do we ensure that at least calls started in the new PLMN can be intercepted?

## Proposals:

- This SA 2 meeting should start discussing the technical aspects of this topic.

- SA2 should take up the offer of a Go-to-meeting session for the interested delegates in SA2, CT 1 and SA3-LI.

- Identify some candidate dates for the above GTM session.

- Discuss whether to proceed first on EPC or progress both EPC and 5GC in parallel.

- Discuss how to make progress on this work:
- R17 (or earlier) correction

- R18 TEI

- R18 WID