3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205988r01

Elbonia, Aug 17th – 26th, 2022	(revision of S2-220xxxx)

Source:	OPPO, Xiaomi?

Title:	KI#2, Sol#15: Update to remove ENs

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this contribution addresses several Editor's Notes of Solution #15.

1. Introduction

In SA2#151E three Editor’s Notes were captured in the description of Solution #15. This document addresses two of them as follows.

This solution proposes to address the KI#2 "PIN and PIN Element discovery and selection" on how to discover the PIN/PINE Service, in the case no PEMC/PEGC is available the PINE that is able to access to 5G network.

In this solution, it is assumed that PINE is configured or aware of the PIN ID, and has the capability to connect to 5G network. And no target PINEs information was configured in the PINE.

Editor's note:	How to solve case for non-3GPP device is FFS.

In this solution, it is assumed that the PINE has the capability to access the 5G network. Therefore, in the case no PEMC/PEGC is available, the PINE can request AMF to query the PIN profile stored in UDM in order to discover the PIN/PINE Service.

For the non-3GPP device, there are three cases as follows.

If neither PEMC nor PEGC are available, a non-3GPP device can’t be a member of a PIN. In such case, there’s no need to discover the PIN/PINE Service.

In the case no PEGC is available, the non-3GPP device can query the PEMC directly. In such case, the other solutions for KI#2 currently documented in the TR can be reused.

In the case no PEMC is available, the non-3GPP device can access the 5G network via PEGC for the PIN/PINE Service discovery.

Proposal 1: Remove the EN and enable the non-3GPP device to access the 5G network via PEGC for the PIN/PINE Service discovery in the case no PEMC is available.

Proposal 2: Add details of PINE/PIN discovery in the case either PEMC or PEGC is available.

7. PINE-1 triggers the PIN communication with PINE-2 by using the information feedback from step 6,

8. Non-3GPP connection is established between PEGC and PINE-2.

Editor's note:	How to ensure the PINE-2 reachable in the Query response is FFS.

As described in this solution, upon receiving the PIN Service Info response, PINE-1 triggers the PIN communication with PINE-2. In order to ensure the PINE-2 reachable, the PEMC will associate PINE-2 with a PEGC if PINE-2 is not associated with any PEGC.

Proposal 3: Remove the EN and add details of connection establishment between PINE-1 and PINE-2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

## 6.15	Solution #15: Discovery of PIN/PINE Services via 5GC

### 6.15.1	Description

This solution proposes to address the KI#2 "PIN and PIN Element discovery and selection" on how to discover the PIN/PINE Service.

In this solution, it is assumed that PINE is configured or aware of the PIN ID, and no target PINEs information was configured in the PINE.

In the case no PEMC is available, if the PINE (i.e., UE) has the capability to connect to 5G, it can request AMF to query the PIN profile stored in UDM in order to discover the PIN/PINE Service. Otherwise, the PINE (i.e., non-3GPP device) accesses the 5G network via PEGC for the PIN/PINE Service discovery.

In the case either PEMC or PEGC is available, the PINE can request the target PINE/PIN Service from PEMC directly or via PEGC.

### 6.15.2	Procedures

#### 6.15.2.1 PINE discovery via 5GC

As in Figure 1, PINE-1 is assumed as one UE that can connect to 5G network. And PINE-1 is configured with PIN ID, no target PINE-2 information. PINE-2 can be reachable via PEGC.

PINE-1 wants to find what the PINE service can be provided by the PIN, and selects the specific PINE Service based on the requirement.

Figure 6.15.2-1: PINE Service discovery via 5GC

1.	PINE-1 sends the PIN Services Info Query request to AMF via gNB, including the PIN ID, and optional PINE-1 information (may be used for further authorization with PEMC). If PINE-1 is a non-3GPP device, the PIN Services Info Query request should be sent to PEGC-1 that has been associated with PINE-1, and then be forwarded to 5GC by PEGC-1.

2.	AMF checks whether the PINE-1 is authorized to use PIN Service.

3.	Additional authorization may be needed from PEMC.

Editor's note:	It is FFS for how to perform the authorization from PEMC.

4.	AMF sends the PIN Service Query request to UDM, with PIN ID

5.	UDM feedbacks the PIN Services information to AMF, including the all the PINEs information, e.g., PINE ID, PINE name, PINE capability, PINE valid time, PINE type, List of PEGCs associated with PINE, etc.

6.	AMF sends the PIN Service Info Query response to PINE-1, including PIN Service Information as specified in step 5. If PINE-1 is a non-3GPP device, the PIN Services Info Query response should be sent to PINE-1 via PEGC-1.

7.	Upon receiving the information feedback from step 6, PINE-1 selects PINE-2 as the target PINE. According to the PINE information received at step 6, PINE-1 can determine if any PEGC has been associated with PINE-2. If so, PINE-1 triggers the PIN communication with PINE-2 via 5GC by using the information feedback from step 6, and step 8-11 can be skipped. Otherwise, PINE-1 sends the PIN Service Configuration request to PEMC, including PINE-2 ID.

8.	Upon receiving the PIN Service Configuration request with PINE-2 ID, PEMC may associate PINE-2 with a PEGC (e.g., PEGC-2), and then PINE-2 is able to connect to 5GC via PEGC. If PEGC-2 is chosen as the associated PEGC of PINE-2, PEMC sends the Connection Setup request including PEGC-2 ID to PINE-2.

9.	Non-3GPP connection is established between PEGC-2 and PINE-2.

10. PINE-2 responds to the PEMC to acknowledge that the Non-3GPP connection has been established.

11. PEMC responds to the PINE-1 with the PIN Service Configuration information, e.g., PEGC-2 ID, etc.

#### 6.15.2.2 Local PINE discovery

Figure 6.15.2.2-1 local PINE discovery

Assuming that there is already connection between PINE and PEGC, or PINE and PEMC

1, PINE sends PINE service Query request to PEMC directly, or via PEGC

2, PEMC sends the PINE Service Query response to PINE directly or via PEGC, including PINE list information, after successful authentication performed by PEMC

3, PINE selects the target PINE2, and initiates the PIN communication between PINE and PINE2 directly, or via PEGC, or via 5GC.

#### 6.15.2.3 PIN discovery

Figure 6.15.2.3-1 PIN discovery

0, PEGC and PEMC are pre-configured with PIN Discovery information

1, PEGC and/or PEMC broadcast the PIN Discovery information, includes PIN ID, PIN name.

2, PINE sends the Join PIN request directly to PEMC or via PEGC, including device ID, capability, valid time, etc.

3a, PEMC triggers to update the PIN Profile in 5GC, e.g., UDM

3b, PEMC updates the PIN profile in PEMC, and sends Join PIN response to PINE directly, or via PEGC.

### 6.15.3	Impacts on Existing Nodes and Functionality

Network impacts:

-	UDM maintains the PIN profile.

-	UDM supports query and feedback the PIN service information based on the request from AMF.

-	AMF supports PIN Service Info Query request/response.

# * * * * End of changes * * * *