SA WG2 Meeting #152e	S2-2205488r03

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, Lenovo

Title: 	Conclusions for KI#3: Policies for finer granular sets of UEs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_eEDGE_Ph2 / Rel-18

Abstract of the contribution: Conclusions for KI#3: Policies for finer granular sets of UEs

- Discussion

Void.

2 Proposal

It is proposed to update TR 23700-48 as follows

FIRST CHANGE all text is new

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.3	Evaluation for KI#3: Policies for finer granular sets of UEs

Following solutions address KI 3

| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |
| --- | --- | --- | --- | --- |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |

Rules to control which PDU sessions of which users are subject to Edge computing are carried via Nnef_TrafficInfluence service and target the PCF.

TS 23.503 [13] Table 6.2-2: “PDU Session policy control subscription information” already defines following subscription data in UDR (information already defined for PCF usage).

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |

Furthermore, there can be the application decided information used as Targeting UE information e.g. the application client category for the application clients, the application decided service level for the application. As an example, for the UEs with the same Subscriber categories defined by the MNO, there can be different service levels provided by the application (e.g. client 1 and client 2 of appx has different service level/application subscriber categories for the application, gold UE1 is used by client 1(application gold client) and gold UE2 is used by client 2(application silver client) to access the application, different service policy can be applied to client 1 and client 2 although the UEs to be used to access the application have the same subscriber category).

Thus, it looks fairly simple to allow AF/NEF to send (Nnef_TrafficInfluence ) to PCF rules targeting e.g. following sets of users:

- All users having subscribed to some “Allowed services” (e.g. “game boost” service to be associated with edge computing at deep edge), and/or

- All users of the same “Subscriber categories” (e.g. gold / solver / bronze users)

- All users used with the same application decided service information like application client category, or application decided service level. The application decided service information can be abstract value associated with the specific application decided service information, in order that the values are not specified in detail by 3GPP.

Using a group Id to send rules targeting the sets of UE(s) currently identified by 23.503 [13] defined “Allowed services” and “Subscriber categories “  would

- imply updating the UDR records.

NEXT CHANGE all text is new

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.3	Conclusions for KI#3: Policies for finer granular sets of UEs

The 5GC and the AF may (as part of a SLA)agree on a Target Category information that is an abstract value (whose values are not specified by 3GPP) and that may represent a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or (abstract value of) application decided service information as defined above

NOTE:	Any logical combination will thus be handled before runtime. If some special logical combination is needed, a certain external group or the new Target Category information can be assigned and used.