SA WG2 Meeting #152e	S2-2205488r07

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Conclusions for KI#3: Policies for finer granular sets of UEs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_eEDGE_Ph2 / Rel-18

Abstract of the contribution: Conclusions for KI#3: Policies for finer granular sets of UEs

- Discussion

Void.

2 Proposal

It is proposed to update TR 23700-48 as follows

FIRST CHANGE all text is new

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.3	Evaluation for KI#3: Policies for finer granular sets of UEs

Following solutions address KI 3

| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |
| --- | --- | --- | --- | --- |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |

Rules to control which PDU sessions of which users are subject to Edge computing are carried via Nnef_TrafficInfluence service and target the PCF.

TS 23.503 [13] Table 6.2-2: “PDU Session policy control subscription information” already defines following subscription data in UDR (information already defined for PCF usage).

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |

Furthermore, there can be the application decided information used as Targeting UE information e.g. the application client category for the application clients, the application decided service level for the application. As an example, for the UEs with the same Subscriber categories defined by the MNO, there can be different service levels provided by the application (e.g. client 1 and client 2 of appx has different service level/application subscriber categories for the application, gold UE1 is used by client 1(application gold client) and gold UE2 is used by client 2(application silver client) to access the application, different service policy can be applied to client 1 and client 2 although the UEs to be used to access the application have the same subscriber category).

Thus, it looks fairly simple to allow AF/NEF to send (Nnef_TrafficInfluence ) to PCF rules targeting e.g. following sets of users:

- All users having subscribed to some “Allowed services” (e.g. “game boost” service to be associated with edge computing at deep edge), and/or

- All users of the same “Subscriber categories” (e.g. gold / solver / bronze users)

- All users used with the same application decided service information like application client category, or application decided service level. The application decided service information can be abstract value associated with the specific application decided service information, in order that the values are not specified in detail by 3GPP.

Using a group Id to send rules targeting the sets of UE(s) currently identified by 23.503 [13] defined “Allowed services” and “Subscriber categories “  would/

- imply updating the UDR records of possibly 10s or even 100s of Millions of users in big networks in order to add the group Ids corresponding to these sets of users in each user subscription

- Would induce a risk of incoherence between the group information stored in UDR for Access and Mobility Subscription data AND the PDU Session policy control subscription information (defined in TS 23.503 Table 6.2-2) stored in UDR

NEXT CHANGE all text is new

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.3	Conclusions for KI#3: Policies for finer granular sets of UEs

Normative specifications will add following features

- the data stored by NEF in UDR (as part of step 3a of Figure 4.3.6.2-1 of TS 23.502 [9]) and the information sent by UDR to the PCF (as part of step 4 of Figure 4.3.6.2-1of TS 23.502 [9]) may refer to a logical combination of Group Ids and/or "Allowed services" and/or "Subscriber categories" as defined in TS 23.503 [13] table 6.2-2. and/or application decided service information

-	The logical combination may e.g. refer to users members of Group Id = X and of "Subscriber categories" = “Gold”

- Nnef_TrafficInfluence service API may refer to a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or application decided service information. The "Allowed services" and "Subscriber categories" are as defined in TS 23.503 [13] table 6.2-2 (and as defined above for the data stored by NEF in UDR as part of step 3a of Figure 4.3.6.2-1 of TS 23.502 [9]).

- Furthermore, the 5GC and the AF may (as part of a SLA) also agree on a Target Category information that is an abstract value (whose values are not specified by 3GPP) and that may represent a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or application decided service information as defined above

Editor’s Note: it is FFS whether both Target Category and application decided service information are needed or how Target Category information is expressed to cover the case that the UEs are grouped considering on the application decided information e.g. the application client category.

- The NEF may, based on local policies determine a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or application decided service information that is to be associated with an incoming Nnef_TrafficInfluence service request . These local policies may relate e.g. with the identity of the AF having issued the Nnef_TrafficInfluence service request and/or relate with the parameters indicated in the Nnef_TrafficInfluence service request (for example the local policies may take into account following information provided in the AF request: the MTC Provider Information, the S-NSSAI and/or the Target category defined above).

- The SMF, EASDF and UPF are not impacted by normative work on this KI. Only the NEF needs to be aware of Target Category and application decided service information.