3GPP TSG-SA2 Meeting #152E	S2-2205537r01

# E-Meeting, 17th – 26th  August 2022

Source:	Ericsson, CMCC

Title:	KI#4: Evaluation and Conclusion for KI#4

Spec:	3GPP TR 23.700-81

Agenda item:	9.23

Document for:	Approval

Work Item / Release:	FS_eNA_ph3 / Rel-18

1. Introduction

This document evaluates KI#4. The corresponding solutions can be classified into four categories:

1,	ADRF stores types of data other than historical data and analytics (e.g. ML models, analytics context) for network analytics. Solutions are Sol#42 and Sol#43.

2,	DCCF relocation. Solutions are Sol#12 and Sol#44

3,	Dealing with buffer overflow of data producer due to muting notifications. Solutions are Sol#41 and Sol#45

4,	ADRF/NWDAF data management enhancement. Solution is Sol#46.

In this contribution, we provide solution evaluations according to the above categories. Based on the evaluation, we also provide interim conclusions for KI#4:

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-81 v0.3.0.

* * * First Changes * * * *

## 7.4	Key Issue #4: How to Enhance Data collection and Storage

According to the Table 6.0-1, solution#12 and #41 to #46 are proposed for Key Issue 4.

Table 7.4-1 gives the comparison of candidate solutions and the corresponding solutions can be classified into four categories:

Table 7.4-1: Comparison of solutions for Key Issue#4

| Solutions | Aspects addressed for the Key Issue |  |  |  | Impacts on services, entities and interfaces |
| --- | --- | --- | --- | --- | --- |
|  | Category-1: ADRF stores types of data other than historical data and analytics | Category-2: DCCF relocation | Category-3: Dealing with buffer overflow of data producer due to muting notifications | Category-4: ADRF/NWDAF data management enhancement |  |
| 12 |  | √ |  |  | Data consumer, DCCF, MFAF. For DCCF and MFAF relocation, new services defined. |
| 41 |  |  | √ |  | Data producer. New parameters defined for existing services. |
| 42 | √ |  |  |  | ADRF, NWDAF. For ML model storage, new services defined. |
| 43 | √ |  |  |  | ADRF,DCCF, NWDAF. For ML model storage, new services defined. |
| 44 |  | √ |  |  | DCCF. New parameters defined for existing services. |
| 45 |  |  | √ |  | Event Consumer NFs (DCCF, NWDAF), Event Producer NFs (AMF, SMF, etc.). New parameters defined for existing services. |
| 46 |  |  |  | √ | NWDAF, DCCF, MFAF, ADRF. New services defined. |

The solution evaluation for Category-1 is the following:

Models are trained and retrieved from NWDAF containing MTLF. ADRF is suggested as an intermediate storage of models, before NWDAF containing AnLF retrieves them. Nothing prevents an NWDAF containing AnLF to retrieve the models directly from an NWDAF containing MTLF, this service is already available in Rel-17. Adding ADRF as an intermediate storage adds multiple ways to retrieve models, increase number of NFs included and thereby complexity and increase signalling between NFs. Models are viewed as business secrets and shall be protected. Model privacy when storing models in ADRF may be a concern, therefore, SA3 needs to investigate how to securely store the model without leaking privacy information.

In Rel-17, MTLF provides the model storage URL and FQDN information to the AnLF. In other words, the MTLF manages the repository where models are stored, obviously this repository is standardized in R17. Therefore, since the ADRF function is built to maintain a centralized repository for AIML operations across the network, it is logical that ADRF can be reused to store the ML models. In addition, saving the ML model generates the additional signaling, in R17 it is hidden under the implementation, now it has been shown due to the standardization of the procedure.

The solution evaluation for Category-2 is the following:

Solution#12 proposes that the source DCCF initiates the DCCF and MFAF reselection due to UE mobility. Target DCCF (MFAF) gets the data subscription and pending outputs from the source DCCF (MFAF). The benefit of the solution is that the data consumer does not need to resend the data subscription to the target DCCF. However, in order to save that signaling, more signaling is induced by the solution. Furthermore, the solution only works when the data consumer is not changing and the data subscription is for single UE, not for group UE or any UE. Overall, the benefits of the solution is not convincing.

Solution#44 argues that it is good enough to terminate the data subscription to the old DCCF and the data consumer does the DCCF selection and subscribes to the new DCCF due to UE mobility.  Unlike NWDAF relocation, where the target NWDAF may need the same data and ML model from the source NWDAF in order to have consistent analytics performance, nothing is really needed by the target DCCF from the source DCCF. Therefore, Solution#44 proposes a simple and clean solution that the old DCCF terminates the subscription and the data consumer select and subscribe to a new DCCF, which has no impact to the current standards.

The solution evaluation for Category-3 is the following:

Solution#41 proposes that when the buffer of the data producer is about to overflow, the data producer informs the data consumer that it cannot keep the muted notifications and send all the muted notifications to the data consumer.

Solution#45 covers Solution#41 and it also proposes that the data consumer can provide exception instruction to the data provider and the data provider will make the final decision on how to treat the muted notifications when its buffer is overflow.

The solution evaluation for Category-4 is the following:

Solution#46 proposes enhancement for the ADRF/NWDAF data management. In particular, data consumer can provide Storage Handling Information to ADRF/NWDAF. In addition, ADRF/NWDAF is also provisioned with the storage policy of the operator. Note that all the data belongs to the operator, it is enough to only have the storage policy of the operator. There is no clear benefit of Storage Handling Information from individual data consumer. Furthermore, there could happen that different data consumers have different Storage Handling Information to the same data. However, it makes sense that the data consumer can ask to be informed if some data is to be deleted.

Futhermore, Solution#46 proposes that DCCF can maintain the data storage information and inform the data consumer about the data deletions. However, DCCF is not a data storage, it does not make sense that DCCF maintains the data storage information. DCCF can only forward the deletion notification from the ADRF/NWDAF.

* * * End of changes * * * *