3GPP TSG-WG SA2 Meeting #152E e-meeting	S2-2205626r02

E-meeting, 2022‑08‑17 -- 2022‑08‑26	(revision of S2-220xxxx)

(merge of S2-2205626, S2-2205906, S2-2206697)

Source:	Ericsson, InterDigital (?), Apple (?)

Title:	KI #6, Solution evaluation and conclusions

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18]

Abstract: An evaluation and conclusions for KI#6 is proposed.

1. Discussion

This contribution provides a scope clarification and an evaluation of the solutions for KI#6.

During the evaluation a gap has been found based on which a pCR to Solution #47 has been sent in that proposes that an indication to the UE to avoid switching to non-3GPP access could be also sent via PCO. This is needed in the cases when a dynamic update needed for an existing PDU session where URSP rules would be difficult to update.

# * * * * 1st change (all new)* * * *

7.X	Evaluation of KI#6: Avoiding UE to switch away from EC PDU Session

There are 9 solutions provided for this key issue: Solution #41 to #49.

Clause 5.6.2 lists some problematic scenarios that should be covered:

-	non-integrated access, where re-connecting to the 5GS is not possible, e.g. because lack of UE support or an N3IWF could not be discovered or connected to and these Edge Computing enablers can only be reached via the 5GS;

-	re-connecting to the 5GS is possible but results in long UP paths because of e.g. a centralized N3IWF;

-	session breakout scenarios where an UL-CL and L-PSA is used to obtain EC connectivity and switching to an access that is not integrated with 5GS would therefore break the EC connectivity.

In the first case, connecting to non-integrated WIFI access should be avoided based on the information known by the 5GC. This requires an indication from the 5GC to the UE to avoid switching away to non-integrated WIFI access.

In other cases, whether the application may still get access to application server via non-integrated access may depend also on the factors outside of 5GC control, such as the IP latency form the WIFI POP to the EHE, the EHE connectivity configuration or transport protocol used between the UE applications and local application server (e.g., QUIC may provide service continuity during access change, but TCP cannot). It may depend on application functionality (e.g., RTT measurements over the different accesses) whether the application could leverage on using non-integrated WIFI. The application may, however, benefit from an indication whether the 5GC currently applies EC treatment, e.g., this may trigger the application logic to control its traffic over different accesses, e.g., based on latency monitoring.

In the second and third cases above, even (re-)connecting to 5GC via non-3GPP access should be avoided. In these cases, the 5GC has the information to decide whether this is needed or not. Thus, it is required that the 5GS sends indication to the UE to avoid switching to non-3GPP access.

There are different alternatives proposed to send an indication from 5GS to avoid switching away to non-integrated WIFI access:

1.	via existing URSP rules. Solution #41 proposes to apply the existing URSP rules to control non-seamless WIFI offload. As stated in Clause 6.1.2.2.1 of TS 23.503: “If the UE has an URSP rule (except the URSP rule with the "match all" Traffic descriptor) that matches the application as defined in clause 6.6.2.3, the UE shall perform the association of the application to the corresponding PDU Session or to Non-Seamless Offload or ProSe Layer-3 UE-to-Network Relay Offload according to this rule”. That is, if 5GC defines policy rules for non-seamless offload form some traffic in the RSD then the UE should apply these rules for that traffic in the corresponding PDU session. For session break-out, if update of traffic descriptors in URSP rules are required, the resulting rule may not be acted upon by the UE immediately. The ATSSS rules can be configured for more refined control of traffic steering if the conflicting non-3GPP access is integrated. The solution can achieve this outcome without any impacts to the specifications. However, in this solution, the UE is not made aware whether any of the edge enablers are configured for the 3GPP access.

2.	via new attribute in URSP rules. Solution #48 proposes an “edge-anchored” indication in URSP RSD(s). The principle of Solution #48 is that an indication may be added to the URSP Rules to indicate that any PDU Session that is associated with the RSD may be using a PSA UPF that is in a local site. However, this status indication cannot guarantee if the UE indeed will get the treatment of “edge anchored” at the time and location when a PDU session is established. The solution makes no reference whether the URSP can be updated based on the AF guidance at run time. Solution #49 proposes to dynamically indicate “ongoing traffic offload” in the RSD. It is however, not stated what the expected impact of this indication is to the UE. Also, the solution given lacks the description of how the UE PCF know which PCF of the PDU Session will be selected for the PDU session so it is unclear how it would subscribe to changes. This is a common issue for all dynamic USRP-based solutions.

3.	as a new indication via PCO. The principle of Solution #47 is that the SMF is aware of whether the PDU Session uses a PSA UPF in local site and the SMF can send an "edge-anchored" indication to the UE that indicates that the PDU Session uses a PSA UPF that is in a local site and also a "5GC-preference" indication that indicates that the network prefers to keep the traffic in the 5GC. The indications can be per Flow Descriptor(s).

4.	as a new indication coupled to EDC attributes. Solution #44 proposes that the ability for network to control UE’s traffic offloading decision is dependent on UE capability and UE’s subscription. UE may indicate to the SMF its capability to support the EDC functionality and to control application traffic switching via ePCO. If the UE subscription information includes EAS traffic switching information, the SMF indicates to UE that EAS traffic switching control is required. The solution proposes this capability to be linked to the EDC functionality, but EDC which is handling DNS queries from the UE may not have visibility of the actual application traffic and cannot enforce traffic routing rules.

NOTE 1:	Solution #44 is similar to Solution #47; Solution#46 also proposes indication to the UE that the PDU Session is using edge computing functionality of the network, but it is not explicitly stated how this indication is conveyed. The UE decision of traffic switching cannot be enforced as stated in the solution since it may happen in upper layers.

There are two potential alternatives to send the indication to UE whether the 5GC currently applies EC treatment for some traffic

1.	as a new indication via PCO. Solution#47 proposes an "Edge-anchored" indication.

2.	via new attribute in URSP rules. See point 2 above.

Providing indication to the UE to avoid switching to non-3GPP access has been proposed in a single solution. Solution#41 proposes to apply the existing URSP rules for this, i.e., setting the Access Type Preference to "3GPP" for the given EC traffic. Besides, for MA PDU sessions, it proposes to use the ATSSS rules to avoid going to non-3GPP access.

NOTE 2:	Recent update to Solution #47 proposes that this type of indication could be also sent via PCO. This could be needed in the cases when a dynamic update needed for an existing PDU session where URSP rules would be difficult to update.

Other solutions include:

-	Solution#42 proposes a new “WLAN Offload Guidance" indication in the Route Selection Descriptor to indicate to the UE that offloading edge computing traffic matching the Traffic Descriptor for this URSP rule to integrated non-3GPP access is conditional to the evaluation of conditions informed by the network.  The UE may then perform RTT measurements on the non-3GPP path and use the measurements to help decide if integrated non-3GPP access should be used. It is not clear what the benefit of this approach is compared to when the application measures RTT directly to EAS over the different accesses. This solution requires that Application Server also supports the best path selection in the UE by configuring performance measurement parameters to compute RTT over 3GPP and non-3GPP paths.

-	Solution#43 provides two approaches:

1.	SMF rejects PDU session establishment from non-3GPP access if EC traffic handling. This requires multi-PDU session capable UEs. Also, if it is not the same SMF, SMF cannot know if the given traffic is EC or not in all connectivity models.

2.	UE decides PDU Session handover according to NWDAF analytics. This requires UE registration to non-3GPP while still using 3GPP access for its sessions. It also requires real-time analytics. Besides, it is not disclosed how the UE would know which traffic belongs to an EC session. Also, the UE is not aware of whether the network has used any of the edge enablers for the PDU Session over 3GPP access and whether it has to perform additional analytics comparisons before deciding on traffic steering for this PDU Session.

-	Solution#45 proposes an application-based solution to select/bind to a proper network interface for communication. This solution does not require SA2 standardization. Note that it assumes that the application is aware that it uses an EC service, which also calls for an indication to the UE (conveyed by the UE to the Application) that the 5GC currently applies EC treatment. It should also be noted that for operator deployed services, application clients can be designed to use this mechanism.

But, it still exists the unclear issues in KI#6.

For KI#6, it describes the situation that the EC traffic should be kept on 5GS and not switched to other access, for example, the non-integrated WiFi.

Although some of the solutions design the mechanism to indicate UE that the EC traffic should be kept on 5GS, it is still unclear whether and how this KI#6 brings benefits.

Firstly, non-integrated WiFi is not discussed or studied in 3GPP scope.

In 3GPP scope, there is no such definition of non-integrated WiFi. And also, according to the section 4.2.8 of TS 23.501[x], all of the non-3GPP access (including WLAN or WiFi) discussed in 3GPP supports connectivity of UE to 5GS.

The 5G Core Network supports connectivity of UEs via non-3GPP access networks, e.g. WLAN access networks.

So, what KI#6 describes and some of the solutions (for example, Sol#44 and etc) design here is to enforce UE not switch the EC traffic to non-integrated WiFi. But, we don’t have any design or mechanism related to non-3GPP access that not connect to 5GS.

So, all of these solutions are not in the scope of 3GPP, and UE can’t have such design to keep traffic not to non-3GPP, for example, non-integrated WiFi.

Secondly, components in URSP rules are also discussed in 3GPP scope that UE connects to 5GS via non-3GPP Access. URSP rules doesn’t discuss or study the non-integrated WiFi situation.

As indicated in the clause 6.6.2 of TS 23.503[x], it discusses the non-3GPP access or Non-Seamless Offload indication. Both of these RSCs, describes the UE behaviour to access 5GS via non-3GPP, not UE behaviour to access another network, for example, the wireline network. Also, the access type preference indicates that the PDU session should be established on which access type. And this access type is also accessed to 5GS.

-	Access Type Preference: If the UE needs to establish a PDU Session when the rule is applied, this indicates the Access Type (3GPP or non-3GPP or multi-access) on which the PDU Session should be established. The type "Multi-Access" indicates that the PDU Session should be established as a MA PDU Session, using both 3GPP access and non-3GPP access.

For Non-Seamless Offload indication, it also describes the situation of UE connect to WLAN and UE connectivity to 5GS via WLAN.

If the selected Route Selection Descriptor contains a Non-Seamless Offload indication and the UE has established a connection to a WLAN access, the UE routes the traffic matching the Traffic descriptor of the URSP rule via the WLAN access outside of a PDU Session.

So, some of the solutions design to add RSC in URSP rules to keep the EC traffic not to non-5GS network is not in the scope of 3GPP. Because, only the UE behaviour of choosing non-3GPP access to 5GS is in the 3GPP scope. If it designs to avoid UE not switch to non-integrated WiFi network, it is UE implementation.

Thirdly, what the benefits that to keep the EC traffic stay on 5GS is not clear.

In KI#6, that the scenario is when the UE connects to another network, the optimal EAS may change and the route from UE to EAS is not the shortest. So, even when user enter the home or office, the EC traffic can’t be switch to WiFi and still stay on the 5GS.

But, there are some of the situations that enforce the EC traffic to stay on 5GS are not right, and stay on 5GS doesn’t bring any benefits:

-	#1: There is no performance comparation between 5GS and non-integrated WiFi. As an example, at some of the location, for example, at home, at tall building or country, the 5GS coverage may not be good enough to provide better QoS performance for edge service. So, why we still enforce the UE to not select a better access for better service experience? If the 5GS at UE location has worse throughput of UL/DL traffic or high time delay, why not to switch the traffic to better WiFi (certainly, in tall building, the WiFi has better signal power and coverage) ?

-	#2: When UE access to non-integrated WiFi, the MNO may also deploy the EAS in wireline access network. As everybody knows, that the application server can also be deployed at the edge of wireline access network. So, even UE switches the traffic to WiFi, the DNS procedure can also provide an EAS IP address in wireline access network to UE, and accomplishes the EAS relocation. So, there is no need to keep the traffic always on 5GS, because in wireline network there still has the deployment of EAS.

-	#3: Keep the traffic on 5GS has the conflict of user preference. Due to the traffic on 5GS may cost more fee to user, so the users want save the money and choose to use WiFi for substitute. But the solutions in KI#6, they enforce the UE to stay on 5GS with no any user agreement or user choices. If there is no user agreement or permission, and directly to enforce the traffic on 5GS, the user will complain about MNO and terminal companies. At last, the users will change the SIM card to another MNO and change the terminal. Maybe the user at last, will switch the 5GS and not use 5G. This has very negative effects to marketing. Users has the choice to use any access mode.

-	#4: How to deal with the situation of handover from 5GS to EPS is also not clear. Due to the UE may has the situation to handover from 5GS to EPS, if we have the design to keep the traffic on 5GS and not to 4G, it means the EPS handover may failure. So, what the design in all of the solutions didn’t solve the problem that how to deal with the conflict when UE handover to another 3GPP access, for example, 3G or 4G.

So, as indicated above, there still has no many problems unsolved and the benefits unclear.

# * * * * 2nd change (all new)* * * *

8.X	Conclusions for KI#6: Avoiding UE to switch away from EC PDU Session

Editor’s note: It is unclear whether the scenarios introduced in KI#6 and the mechanisms defined in solutions under KI#6 has benefit to 5GC or UE.

Editor’s note: The conclusion part needs further update.

# * * * * End of changes * * * *