3GPP TSG-SA WG2 Meeting #152E (e-meeting)	S2-2205663r04

# Elbonia, 17 – 26 August, 2022													 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3507 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE presence in AoI clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current description for the determination of UE presence in area of interests in Annex D covers the case with location reporting from NG-RAN in CM-CONNECTED mode. However, the current text specifies that use of the RRC_INACATIVE is a pre-condition for AMF to request NG-RAN location reporting which is not the intention. The intention is that AMF may ask NG-RAN for location reporting for UE in CM-CONNECTED mode, including the possible case where NG-RAN applied RRC_INACTIVE state for UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the AMF may request location reporting from NG-RAN for UE in CM-CONNECTED state, including the case when UE is in RRC_INACTIVE and also the case when UE is in RRC_CONNECTED state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect interpretion of the specification and leads to missing functionality (i.e. UE location reporting is only triggered when UE is in RRC_INACTIVE state but not RRC_CONNECTED state). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.1, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

# D.1	Determination of UE presence in Area of Interest by AMF

If the AMF has requested NG-RAN location reporting as specified in clause 4.10 for the Area Of Interest and UE is in CM-CONNECTED state, including RRC_CONNECTED and RRC_INACTIVE state, the AMF determines the UE presence in the Area of Interest as the reported value from the NG-RAN as specified in D.2.

If the AMF has requested N2 Notification as specified in clause 4.8.3, the AMF determines the UE presence in the Area Of Interest as follows, taking N2 Notification from NG-RAN into consideration:

-	IN:

-	if the UE is in CM-CONNECTED with RRC_CONNECTED state and if the last received User Location Information for the UE is inside the Area Of Interest service area ; or

NOTE 1:	The above is valid e.g. under the condition that Area Of Interest border coincides with NG-RAN node service area border.

-	if the UE is in CM-CONNECTED with RRC_CONNECTED or CM-CONNECTED with RRC_INACTIVE state and if the UE is inside a Registration Area which is completely contained within the Area Of Interest.

-	OUT:

-	if the UE is in CM-CONNECTED with RRC_CONNECTED state and if the last received User Location Information for the UE is outside the Area Of Interest ; or

NOTE 2:	The above is valid e.g. under the condition that Area Of Interest border coincides with NG-RAN node service area border.

-	if the UE is in CM-CONNECTED with RRC_CONNECTED or CM-CONNECTED with RRC_INACTIVE state and if UE is inside a Registration Area which does not contain any part of Area Of Interest.

-	UNKNOWN:

if none of above conditions for IN or OUT is met.

Otherwise, AMF determines the UE presence of Area Of Interest as follows:

-	IN:

-	if the UE is in CM-CONNECTED and if the last received User Location Information for the UE is inside the Area Of Interest service area; or

NOTE 3:	The above occurs when the Area Of Interest border is configured to coincide with NG-RAN node service area border or the Area Of Interest includes the complete RAN Notification Area.

-	if the UE, in CM-CONNECTED or CM-IDLE, is inside a Registration Area which is completely contained within the Area Of Interest.

-	OUT:

-	if the UE is in CM-CONNECTED and if the last received User Location Information for the UE is outside the Area Of Interest; or

NOTE 4:	The above occurs when the Area Of Interest border is configured to coincide with NG-RAN node service area border or none of the Area of Interest is within the RAN Notification Area.

-	if UE, In CM-CONNECTED or CM-IDLE, is inside a Registration Area which does not contain any part of Area Of Interest.

-	UNKNOWN:

if none of above conditions for IN or OUT is met.

# D.2	Determination of UE presence in Area of Interest by NG-RAN

If the AMF has requested for the Area of Interest, NG-RAN determines the UE presence of Area Of Interest as follows:

-	IN:

-	if the UE is inside the Area Of Interest and the UE is in RRC Connected state; or

-	if the UE is inside an RNA which is completely contained within the Area Of Interest.

-	OUT:

-	if the UE is outside the Area Of Interest in RRC Connected state; or

-	if UE is inside an RNA which does not contain any part of Area Of Interest.

-	UNKNOWN:

-	if none of above conditions for IN or OUT is met.

****************** END CHANGES ***************