3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205717r06

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion for KI#2

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: It is proposed to have evaluation and conclusion for KI#2.

# 1. Introduction/Discussion

There are 13 solutions for KI#2. This KI relates to:

- Whether and how the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC.

- Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take?

- User privacy needs to be considered when defining solution (e.g. 5GC will know if a user started/stopped "any" application).

The details are shown in Table 1-1.

| Solutions | Title | Packet detection | Authorization by CP | URSP enforcement feedback |
| --- | --- | --- | --- | --- |
| 7 | URSP Compliance Verification |  | X |  |
| 8 | URSP Rule Precedence reporting for awareness of URSP enforcement | X |  |  |
| 9 | Per-PDU session awareness of URSP enforcement | X |  |  |
| 10 | Network based URSP rules enforcement feedback | X |  |  |
| 11 | 5GC awareness of URSP recognition by UE |  |  | X |
| 12 | URSP rule enforcement validation via gating control | X |  |  |
| 13 | URSP enforcement via PDU Session authorization/authentication when UE associating with application |  | X |  |
| 14 | 5GC awareness of URSP rule evaluation and verification of network slice usage |  | X |  |
| 15 | UE URSP enforcement validation by the network |  | X |  |
| 30 | Leveraging NWDAF to determine UEs that enforce URSP rules incorrectly | X |  |  |
| 31 | New URSP Notification Component |  |  | X |
| 32 | Application Detection using Domain Descriptor | X |  |  |

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-85.

# * * * * First change * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.x	Evaluation on Key Issue #2: 5GC awareness of URSP enforcement

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Interim Conclusion on Key Issue #2: 5GC awareness of URSP enforcement

It is agreed to adopt the following interim conclusions for each of the KI aspects:

### 8.X.1 How the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC

-	The network needs to be aware of whether a specific URSP rule(s) is/are enforced by the UE

-	The network needs UE assistance in order to determine whether a specific URSP rule(s) is/are enforced by the UE

Editor’s note: Whether “PCF” refers to SM-PCF, UE-PCF, or both is FFS

Editor’s note: How this is achieved is FFS

Editor’s note: Further conclusions are FFS

### 8.X.2 Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take

### Editor’s note: Further actions the 5GS can take are FFS

### 8.X.3 User privacy considerations

### -  UE assistance should be tied to user consent

Editor’s note: Further privacy considerations are FFS

# * * * * End of changes * * * *