3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205736r03

Elbonia, August 17 – 26, 2022	(revision of S2-2205736)

Source:	Huawei, HiSilicon

Title:	Update of conclusion for KI#2

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

1.	Discussion

In SA2#151e, conclusion for KI#2 has been proposed for KI#2 for time synchronization service enhancements. This document provides further update of the conclusion for KI#2.

2.	Text proposal

It is proposed to capture the following changes vs. TR 23.700-25:

# * * * * First change * * * *

8.x	Key Issue #2: Time synchronization service enhancements

The following bullet points summarize the principles for the way forward:

- To request a Requested Coverage Area for time synchronization services, AFs within the operator’s domain formulates a spatial validity condition using a list of Tracking Area or Cell identities, but AFs outside the operator’s domain use a geographical area (e.g., a civic address or shapes) instead, while NEF transforms this information into a list of Tracking Area or Cell identities.

- The TSCTSF is responsible of activate/deactivate time synchronization services (ASTI or (g)PTP based) considering the spatial validity condition provided by the AF.

-	The TSCTSF discovers the AMF(s) serving the list of TA(s)/Cell(s) that comprise the spatial validity condition using the NRF.

-	The TSCTSF subscribes to UE’s presence in Area of Interest at the discovered AMF(s).

-	The TSCTSF determines (based on notifications from the discovered AMF(s)) whether the targeted UE(s) are inside or outside the AF Requested Coverage Area.

-		The TSCTSF activates time synchronization services for UE(s) that are inside the Requested Coverage Area.

- Conclusions in KI#1 are applied to monitor the 5GS network timing synchronization status, to ensure time synchronization requirements can be supported if the UE is inside or moves inside the Requested Coverage Area.

-	TSCTSF notifies the AF whether the targeted UE(s) are inside or move inside or outside of the Requested Coverage Area.

# * * * * End of changes * * * *